|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- |
| ITU-T RECOMMENDATION SUMMARY |  |  |  |
| Rec. No. : G.763 |  | Title : Digital circuit multiplication equipment using 32 kbit/s ADPCM and digital speech interpolation |  |
| Study Group : XV - Transmission Systems and Equipment |  |  |  |
| Version : Revised |  | Date of adoption : 1990 | Notes : |
| Speech signals occurring on telecommunications links are generally the product of two-way conversations. It is customary for one talker to pause while the other speaks; thus, an active speech signal is present on each direction of the trunk channel for only a fraction of the available time. In addition, even when only one talker is speaking, pauses occur between utterances, so there are times when the circuit is idle. Measurements show that speech is present on each direction of the trunk channel approximately 30 to 40% of the time, averaged over a large number of busy trunks. DCME reduces the transmission capacity needed to handle a multiplicity of telephone trunk channels by exploiting the low average channel activity and by transmitting active speech using ADPCM techniques. The DCME provides a nominal 5 : 1 reduction in the transmission capacity required to carry various mixtures of speech, voice-band data and 64 kbit/s unrestricted channels. An overload strategy consisting of variable bit rate encoding and dynamic load control techniques is utilized to limit speech clipping. The DCME data detector discriminates between voice-band data and speech in order to assign the voice-band data signal to a bearer channel protected against the formation of overload channels which degrade the voice-band data performance. Recommendation G.763 is applicable to the design of digital circuit multiplication equipment intended for, but not limited to, use in an international digital circuit. Freedom is permitted in design details which are not covered in Recommendation G.763. |  |  |  |
| To order the complete text of this Recommendation, please use the Order Form for ITU-T Recommendations. An electronic version of this form is available on ITUDOC (Winword: UPI=ITU-5265; ASCII: UPI=ITU-2488). |  |  |  |