TABLE 2/T.30

| Bit No. | DIS/DTC | DCS |
| --- | --- | --- |
| 1 | Transmitter – T.2 operation |  |
| 2 | Receiver – T.2 operation | Receiver – T.2 operation |
| 3 | T.2 IOC = 176 | T.2 IOC = 176 |
| 4 | Transmitter – T.3 operation |  |
| 5 | Receiver – T.3 operation | Receiver – T.3 operation |
| 6 | Reserved for future T.3 operation features |  |
| 7 | Reserved for future T.3 operation features |  |
| 8 | Reserved for future T.3 operation features |  |
| 9 | Transmitter – T.4 operation |  |
| 10 | Receiver – T.4 operation | Receiver – T.4 operation |
| 11, 12, 13, 14 10, 10, 10, 10 10, 11, 10, 10 11, 10, 10, 10 11, 11, 10, 10 10, 10, 11, 10 10, 11, 11, 10 11, 10, 11, 10 11, 11, 11, 10 10, 10, 10, 11 10, 11, 10, 11 11, 10, 10, 11 11, 11, 10, 11 10, 10, 11, 11 10, 11, 11, 11 11, 10, 11, 11 11, 11, 11, 11 | Data signalling rate V.27 ter fall back mode V.27 ter V.29 V.27 ter and V.29 Not used Reserved Not used V.27 ter, V.29 and V.33 Not used Reserved Not used V.27 ter, V.29, V.33 and V.17 Not used Reserved Not used Reserved | Data signalling rate 12 400 bit/s, V.27 ter 14 800 bit/s, V.27 ter 19 600 bit/s, V.29 17 200 bit/s, V.29 14 400 bit/s, V.33 12 000 bit/s, V.33 Reserved Reserved 14 400 bit/s, V.17 12 000 bit/s, V.17 19 600 bit/s, V.17 17 200 bit/s, V.17 Reserved Reserved Reserved Reserved |
| 15 | Vertical resolution – 7.7 line/mm | Vertical resolution – 7.7 line/mm |
| 16 | Two-dimensional coding capability | Two-dimensional coding |
| 17, 18 (0,0) (0,1) (1,0) (1,1) | Recording width capabilities 1728 picture elements along scan line length of 215 mm ± 1% 1728 picture elements along scan line length of 215 mm ± 1% and 2048 picture elements along scan line length of 255 mm ± 1% and 2432 picture elements along scan line length of 303 mm ± 1% 1728 picture elements along scan line length of 215 mm ± 1% and 2048 picture elements along scan line length of 255 mm ± 1% Invalid (see Note 7) | Recording width 1728 picture elements along scan line length of 215 mm ± 1% 2432 picture elements along scan line length of 303 mm ± 1% 2048 picture elements along scan line length of 255 mm ± 1% Invalid |
| 19, 20 (0,0) (0,1) (1,0) (1,1) | Maximum recording length capability A4 (297 mm) Unlimited A4 (297 mm) and B4 (364 mm) Invalid | Maximum recording length A4 (297 mm) Unlimited B4 (364 mm) Invalid |

TABLE 2/T.30 (Cont.)

| Bit No. | DIS/DTC | DCS |
| --- | --- | --- |
| 21, 22, 23 (0, 0, 0) (0, 0, 1) (0, 1, 0) (1, 0, 0) (0, 1, 1) (1, 1, 0) (1, 0, 1) (1, 1, 1) | Minimum scan line time capability at the receiver 20 ms at 3,85 1/mm: T7,7 = T3,85 40 ms at 3.85 1/mm: T7.7 = T3.85 10 ms at 3.85 1/mm: T7.7 = T3.85 15 ms at 3.85 1/mm: T7.7 = T3.85 10 ms at 3.85 1/mm: T7.7 = 1/2 T3.85 20 ms at 3.85 1/mm: T7.7 = 1/2 T3.85 40 ms at 3.85 1/mm: T7.7 = 1/2 T3.85 00 ms at 3.85 1/mm: T7.7 = T3.85 | Minimum scan line time 20 ms 40 ms 10 ms 45 ms 40 ms |
| 24 | Extend field | Extend field |
| 25 | 2400 bit/s handshaking | 2400 bit/s handshaking |
| 26 | Uncompressed mode | Uncompressed mode |
| 27 | Error correction mode | Error correction mode |
| 28 | Set to “0” | Frame size 0 = 256 octets Frame size 1 = 264 octets |
| 29 | Error limiting mode | Error limiting mode |
| 30 | Reserved for G4 capability on PSTN | Reserved for G4 capability on PSTN |
| 31 | T.6 coding capability | T.6 coding enabled |
| 32 | Extend field | Extend field |
| 33 (0) (1) | Validity of bits 17, 18 Bits 17, 18 are valid Bits 17, 18 are invalid | Recording width Recording width indicated by bits 17, 18 Recording width indicated by this field bit information |
| 34 | Recording width capability: 1216 picture elements along scan line length of 151 mm ± 1% | Middle 1216 elements of 1728 picture elements |
| 35 | Recording width capability: 864 picture elements along scan line length of 107 mm ± 1% | Middle 864 elements of 1728 picture elements |
| 36 | Recording width capability: 1728 picture elements along scan line length of 151 mm ± 1% | Invalid |
| 37 | Recording width capability: 1728 picture elements along scan line length of 107 mm ± 1% | Invalid |
| 38 | Reserved for future recording width capability |  |
| 39 | Reserved for future recording width capability |  |
| 40 | Extend field | Extend field |

Notes to Table 2/T.30

Note 1 – Standard facsimile units conforming to T.2 must have the following capability: Index of cooperation (IOC) = 264.

Note 2 – Standard facsimile units conforming to T.3 must have the following capability: Index of cooperation (IOC) = 264.

Note 3 – Standard facsimile units conforming to T.4 must have the following capability: Paper length = 297 mm.

Note 4 – Where the DIS or DTC frame defines V.27 ter capabilities, the equipment may be assumed to be operable at either 4800 or 2400  bit/s.

Where the DIS or DTC frame defines V.29 capabilities, the equipment may be assumed to be operable at either 9600 or 7200 bit/s per V.29, where it defines V.33 capabilities, the equipment may be assumed to be operable at either 14 400 bit/s or 12 000 bit/s per V.33 and where it defines V.17, the equipment may be assumed to be operable at 14 400 bit/s, 12 000 bit/s, 9600 bit/s or 7200 bit/s per V.17.

Note 5 – T7.7 and T3.85 refer to the scan line times to be utilized when the vertical resolution is 7.7 lines/mm or 3.85 lines/mm, respectively (see bit 15 above). T7.7 = 1/2 T3.85 indicates that in the high resolution mode, the scan line time can be decreased by half.

Note 6 – The standard FIF field for the DIS, DTC and DCS signals is 24 bits long. If the "extended field" bit(s) is a 1, the FIF field shall be extended by an additional eight bits.

Note 7 – Existing equipment may send the invalid (1,1) condition for bits 17 and 18 of their DIS signal. If such signal is received, it should be interpreted as (0,1).

Note 8 – The values of bit No. 28 in the DCS command is valid only when the indication of the T.4 error correction mode is invoked by bit 27.

Note 9 – When bit 33 is set to 1 in DCS, the meaning of bit 15 originally defined to indicate 7.7/mm vertical resolution is modified to mean a higher resolution.

Note 10 – When the recording width is A4 only, the field consisting of bits 33-40 need not be present.

Note 11 – The optional T.4 error correction mode of operation requires 0 ms of the minimum scan line time capability. Bits 21-23 in DIS/DTC signals indicate the minimum scan line time of a receiver regardless of the availability of the error correction mode.

In case of error correction mode, the sender  sends DCS signal with bits 21-23 set to 1.1.1 indicating 0 ms capability.

In case of normal G3 transmission, the sender sends DCS signal with bits 21-23 set to the appropriateness according to the capabilities of the two machines.

Note 12 – T.6 coding scheme capability specified by bit 31 is valid only when bit 27 (error correction mode) is set as a “1” .