The drawings contained in this Recommendation have been done in Autocad.

# 12	Error recovery

This section specifies the error recovery mechanisms for errors which may occur during the communication between a TLM terminal and a TLMAU. The type of recovery will depend upon whether the error occurred during the transmission of control information or a normal document. The recovery mechanism of Recommendation T.62 shall apply. In addition, the qualifications given in the following paragraphs shall apply.

For both directions of transfer, if an error is detected during the transmission of a document containing control information, the transmitted part shall be discarded and the complete TAPDU retransmitted.

If an error is detected during the transmission of a document containing message body, the recovery mechanism depends upon the direction of transfer. In the TLMAU–to–TLM terminal direction, two possibilities for recovery are available, the first one is to apply the rules of Recommendation T.62 using CDC, the second one is the complete retransmission of the TAPDU. In the TLM terminal–to–TLMAU direction, three possible recovery mechanisms are identified, the choice of which is a national matter:

1)	No recovery is provided. The complete TAPDU (control information and message body) must be retransmitted.

2)	The TLMAU shall retain the received part up to the last acknowledged checkpoint. The originating TLM terminal is required to resume the sending of the remainder of the TAPDU by using CDC protocol element as described in Recommendation T.62. The TLMAU shall automatically link the retained part with the resumed part prior to sending to the MTS. If the originating TLM terminal does not resume the sending within a predetermined time or if the TLMAU receives new TAPDU within that time, the TLMAU shall discard the received part of the TAPDU.

3)	The last acknowledged normal document is considered the end of the TAPDU. The TLMAU will send this part to the MTS as a complete message. If the TLM terminal wishes to resume transmission of the interrupted TAPDU, it must first resend the control information. There is no correlation between the original message and the continued message at a receipient TLMAU or IPM–UA.

Note 1 – For failures occurring between document boundaries, the last acknowledged document shall be regarded as the end of the current TAPDU.

Note 2 – A TAPDU, which may be composed of more than one document, cannot be considered complete before at least two documents were received.

Note 3 – If there is a relationship between the sequence of normal documents (for example a multi–part body message) being transferred, the recovery mechanism 2 should not be used.

# 13	Control procedures

## 13.1	Session control procedure

The session control procedures shall be in accordance with Recommendation T.62. However, the qualifications listed in the following subsections shall also apply.

### 13.1.1	Session element of procedure

The values of mandatory parameters used during session establishment shall be as given in Table 7/T.330. The format of the TLMAU terminal identifier shall be that specified in Recommendation F.200.

### 13.1.2	Session rules

Change control (CSCC/RSCCP exchange) may occur inside TAPDU boundaries but should occur outside document boundaries.

## 13.2	Document control procedures

The document control procedures shall be in accordance with Recommendation T.62.

Control information is conveyed in either a control document or a normal document. Where encoded information types of the control information is Teletex, the body of the IP–message is conveyed in a sequence (at least one) of normal documents.

Note – The use of the normal document for conveying control information should be restricted to Teletex access to IPMS.

TABLE 7/T.330

Mandatory parameter values during session establishment

Call Originator

T.62 parameters

TLM terminal

TLMAU

Terminal identifier of calling terminal (in CSS)

TLM TID

TLMAU's TID

Terminal identifier of called terminal (in RSSP)

TLMAU TID

TLM TID

Date and time

Date and time the TLM terminal originated call

Date and time the TLMAU terminal originated call

Service identifier

Telematic

Telematic

Window size

Note 1

Note 1

Session user data

Basic terminal capabilities and interchange format(s) which are available as receiving capabilities of the TLM terminal (Note 1)

Basic terminal capabilities and interchange format(s) which are available as receiving capabilities of the TLMAU (Note 1)

Note 1 – This parameter is mandatory for the Group 4 facsimile service.

Note 2 – TID means terminal identification.

Note 3 – Negotiation of the capabilities of handling control documents encoded in T.61 characters is not needed.

In a multiple–TAPDU session, the TAPDUs are delimited by at least one of the following methods:

1)	implicity, that is, by the first document of the TAPDU being a control document;

2)	explicitly, that is, by the first document of the TAPDU containing a parameter indicating the number of subsequence documents in the TAPDU.

Note – The second method is mandatory for multiple–action–unit session, where the first document of the TAPDU is a normal document.

When a single TAPDU is sent on a session connection, the TAPDU is delimited only by the end of the session.

## 13.3	Log–on procedure

It is conceivable that many subscribers will commonly use a single TLM terminal, some mechanism is required to identify a specific user by intermediate system, i.e. log–on procedure using user name and password. Detailed procedure requires further study.

ANNEX A

(to Recommendation T.330)

xe ""§Formal definition of TLMA abstract service

This Annex is an integral part of this Recommendation.

This Annex, a supplement to § 7, formally defines for reference purposes the abstract service TLMA (telematic agent). It uses the PORT and ABSTRACT–BIND, –UNBIND, –OPERATION, and –ERROR macro of Recommendation X.407.

TLMAAbsService { ccitt–t330 }

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

–– Prologue

EXPORTS

–– Primary port types

miscellanea

IMPORTS

–– Abstract service

origination,   reception,   management

FROM IPMSAbstractService { joint–iso–ccitt

mhs(6)   ipms(1)   modules(0)   abstract–service(3) }

import,   export

FROM MTSAbstractService { joint–iso–ccitt

mhs(6)   mts(3)   modules(0)   mTS–abstract–service(3) }

–– Information objects and their aspects

IPM,   RN,   NRN

FROM IPMSInformationObjects { joint–iso–ccitt

mhs(6)   ipms(1)   modules(0)   information–objects(2) }

–– Functional objects

tlma

FROM IPMSFunctionalObjects { joint–iso–ccitt

mhs(6)   ipms(1)   modules(0)   functional–objects(1) }

–– MTAS aspects

MessageDeliveryEnvelope, ORName, MessageIdentifier, Priority, Time, ReportDeliveryEnvelope,

PerRecipientReportDeliveryFields

FROM MTSAbstractService { joint–iso–ccitt

mhs(6)   mts(3)   modules(0)   mTS–abstract–service(3) }

–– Abstract services macros

OBJECT,   PORT,   ABSTRACT–BIND,   ABSTRACT–ERROR,   ABSTRACT–OPERATION

FROM AbstractServiceNotation { joint–iso–ccitt

mhs(6)   adsdc(2)   modules(0)   notation(1) }

DateandTime ::= Time

–– Object

tlma						OBJECT

PORTS {

origination	[S],

reception	[S],

management	[S],

miscellanea	[S],

import		[C],

export		[C] }

::= id–ot–tlma

–– Ports

miscellanea PORT

SUPPLIER PERFORMS {

ChangeSubscriptionProfile,

DSList,

DSDelete,

DSFetch,

MessageStatus }

::= id–pt–miscellanea

TLMAAbsService (continuation)

–– Miscellanea port specific abstract error problems

SubscriptionProfileProblem ::= ENUMERATED {

not–changed(0) }

DSProblem ::= ENUMERATED {

no–message–in–ds(0),

ds–not–supported(1),

ds–not–subscribed(2),

retrieval–identifier–invalid(3),

parameter–invalid(4) }

MessageStatusProblem ::= ENUMERATED {

query–identifier–invalid(0) }

–– RetrievalIdentifier

RetrievalIdentifier ::= PrintableString

–– CallIdentification

CallIdentification ::= SEQUENCE {

TLMAU TID				[0] TerminalIdentifier,

TLM TID				[1] TerminalIdentifier,

date–and–time			[2] DateAndTime,

document–ref–num		[3] DocumentReferenceNumber,

additional–session–ref–num	[4] AdditionalSessionReferenceNumber OPTIONAL }

TerminalIdentifier ::= PrintableString –– defined in Recommendation F.200 ––

DocumentReferenceNumber ::= NumericString

AdditionalSessionReferenceNumber ::= NumericString

–– Change subscription profile abstract operation component types

DSMode ::= SET {

[0] Mode,

frequency		[1] Frequency OPTIONAL,

time				[2] DateandTime OPTIONAL }

Mode ::= CHOICE {

[0] auto–output,

[1] retrieval }

Frequency ::= NumericString	–– in minutes ––

ErrorRecoveryMode ::= CHOICE {  –– see § 12 in this Recommendation ––

[0] recovery–1,		–– no–recovery ––

[1] recovery–2,		–– full–support–of–T.62–recovery–procedures ––

[2] recovery–3		–– recovery–of–document–basis –– }

MessageDeleteMode ::= CHOICE {

[0] auto–delete,

[1] manual–delete }

–– DS List abstract operation component types

ListReport ::= SET {

retrieval–id			[0] RetrievalIdentifier,

message–type		[1] MessageType,

priority				[2] Priority OPTIONAL,

message–length		[3] MessageLength OPTIONAL,

originator–name	[4] ORName OPTIONAL }

MessageType ::= CHOICE {

[0] ipm,

[1] receipt–notice,

[2] non–receipt–notice,

[3] report }

MessageLength ::= NumericString		–– in octet ––

TLMAAbsService (continuation)

–– DS Fetch abstract operation component types

DeleteAfterOutput ::= CHOICE {

[0] delete,

[1] keep }

MessageReport ::= CHOICE {

ipm–fetch			[0] IPMFetch,

rn–fetch				[1] RNFetch,

nrn–fetch				[2] NRNFetch,

report–fetch			[3] ReportFetch }

IPMFetch ::= SEQUENCE {

envelope		[0] MessageDeliveryEnvelope,

content		[1] IPM }

RNFetch ::= SEQUENCE {

envelope		[0] MessageDeliveryEnvelope,

–– not used intended–recipient–name and other recipient–name of deliver–envelope in RNFetch ––

content		[1] RN } }

NRNFetch ::= SEQUENCE {

[0] number–of–docs	[0] NumberOfAssociatedDocuments OPTIONAL,

[1] SET {

envelope			[1] MessageDeliveryEnvelope,

–– not used intended–recipient–name and other recipient–name of deliver–envelope in NRNFetch ––

content		[2] NRN } }

ReportFetch ::= SEQUENCE {

[0] SET {

number–of–docs	[0] NumberOfAssociatedDocuments OPTIONAL,

call–id			[1] CallIdentification },

[1] SET {

envelope			[0] ReportDeliveryEnvelope,

returned–content	[1] IPM OPTIONAL } } }

NumberOfAssociatedDocuments ::= NumericString

–– Message status abstract service

QueryIdentifier ::= CHOICE {

submission–id	[0] MessageIdentifier,

call–id			[1] CallIdentification }

StatusInfo ::= SET {

[0] Status,

per–recipient–info	[1] PerRecipientReportDeliveryFields OPTIONAL }

Status ::= CHOICE {

[0] in–process,

[1] delivered,

[2] non–delivered }

–– Miscellanea abstract–operations

ChangeSubscriptionProfile ::= ABSTRACT–OPERATION

ARGUMENT SET {

ds–mode		[0] DSMode OPTIONAL,

error–recovery–mode	[1] ErrorRecoveryMode OPTIONAL,

message–delete–mode	[2] MessageDeleteMode OPTIONAL }

RESULT { }

ERRORS {

name–error,

ds–error,

subscription–profile–error }

TLMAAbsService (continuation)

DSList		::= ABSTRACT–OPERATION

ARGUMENT { }

RESULT SET {

list–reports [0] SET OF ListReport OPTIONAL }

ERRORS {

subscription–error,

name–error,

ds–error }

DSDelete	::= ABSTRACT–OPERATION

ARGUMENT SET {

selector 	[0] SET OF RetrievalIdentifier }

RESULT

ERRORS {

subscription–error,

name–error,

ds–error }

DSFetch		::= ABSTRACT–OPERATION

ARGUMENT SET {

selector			[0] SET OF RetrievalIdentifier,

delete–after–output	[1] DeleteAfterOutput OPTIONAL }

RESULT SET {

retrieval–id		[0] RetrievalIdentifier,

message–report		[1] MessageReport }

ERRORS {

subscription–error,

name–error,

ds–error }

MessageStatus ::= ABSTRACT–OPERATION

ARGUMENT SET {

[0] QueryIdentifier }

RESULT SET {

report–time			[0] DateandTime,

reported–message–id		[1] MessageIdentifier,

reported–message–id		[2] SET OF StatusInfo }

ERRORS {

subscription–error,

name–error,

message–status–error }

–– Miscellanea port specific abstract errors

subscription–profile–error    ABSTRACT–ERROR

PARAMETER SET {

problem [0] SubscriptionProfileProblem }

::= 0

ds–error    ABSTRACT–ERROR

problem [0] DSProblem }

::= 1

message–status–error    ABSTRACT–ERROR

PARAMETER SET {

problem [0] MessageStatusProblem }

::= 2

END –– of TLMAAbsService

ANNEX B

(to Recommendation T.330)

Format of TAPDU components

In this Annex the formats of the control information for different examples TAPDUS are shown. The principles of encoding are given in § 11. The formats of elements as defined in § 11 are illustrated by the use of the following four different syntax elements:

1)	The elements number field is represented by a sequence of numeric graphic characters.

2)	The element name field is represented by a text string giving the CCITT language reference name of the field. The actual value shall be a language–dependent representation of that reference name.

3)	Separators are shown as they shall be represented in the actual control information.

4)	Element value fields are shown in square brackets (“[” and “]”). The actual parameter values are described in § 11.

Note – The examples shown below are not exhaustive. Not all types (element value of the TAPDU description in § 10 (ASN.1 description) are included.

B.1	Conventions

Following symbols are used in this Annex:

1)	NL	new line function such as (CR LF or LF CR) or LF.

2)		space character.

3)	[ ]	element value field. Actual values are defined in § 11 of this Recommendation.

4)	[ ]	iteration.

B.2	Example of TAPDU

B.2.1	Send–TAPDU

3.1:		SEND:

62:		QUANTITY–OF–DOCS:[number–of–docs]

13:		PRIORITY:[priority–ind]

19:		FLAGS:

[deferred–delivery–time] `NL`

=[disclose–recipients],[alternate–recipient–allowed]

16:		CONVERSION:[conversion–prohibited]

17:		CONTENT–INFO:[content–return–request]

27:		THIS–IPM:

[this–ipm–id]

20:		FROM:

[originating–user]

21:		AUTHORIZING:

[ [authorizing–user] `NL` ]

22:		TO:

[ [primary–recipient] `NL`

=[user–report–request],[rn–request],[nrn–request],[return–request],[reply–request] `NL` ]

23:		CC:

[ [copy–recipient] `NL`

=[rn–request],[nrn–request],[return–request],[reply–request] `NL` ]

24:		BCC:

[ [blind–copy–recipient] `NL` ]

30:		REPLIED–TO–IPM:

[replied–to–ipm–id]

29:		OBSOLETES:

[ [obsoleted–ipm–id] `NL` ]

28:		RELATED–IPMS:

[ [related–ipm–id] `NL` ]

26:		SUBJECT:

[subject–content]

18:		CONTENT–INDICATOR:

[expiry–time] `NL`

=[importance],[sensitivity]

25:		REPLY:

[reply–time] `NL`

[ [reply–recipient] `NL` ]

31:		BODY–TYPE: [ [body–part], ]

B.2.2	SendAck–TAPDU

3.14:	SEND–ACK:

1:		CORRELATION–INFORMATION:

[call–id]

65:		SUBMISSION–ID:

[submission–msg–id]

33:		SUBMISSION–TIME:

[submission–time]

B.2.3	Exception–TAPDU

3.12:	EXCEPTION:

1:		CORRELATION–INFORMATION:

[call–id]

9:		ERRORS: [error–cause]

B.2.4	Probe–TAPDU

3.2:	PROBE:

B.2.5	ProbeAck–TAPDU

3.15:	PROBE–ACK:

1:		CORRELATION–INFORMATION:

[call–id]

66:		PROBE–ID:

[probe–msg–id]

33:		SUBMISSION–TIME:

[submission–time]

B.2.6	ExplicitRN–TAPDU

3.6:		EXPLICIT–RN:

15:		RECIPIENTS:

[recipient–name]

13:		PRIORITY:[priority–ind]

71:		SUBJECT–IPM:

[subject–ipm–id]

69:		IPN–ORIGINATOR:

[ipn–originating–user]

47:		TIME–OF–RECEIPT:

[receipt–time]

44:		CONVERTED–INFORMATION–TYPES:

[ [eIT], ]

B.2.7	ExplicitRNAck–TAPDU

3.16:	EXPLICIT–RN–ACK:

1:		CORRELATION–INFORMATION:

[call–id]

65:		SUBMISSION–ID:

[submission–msg–id]

33:		SUBMISSION–TIME:

[submission–time]

B.2.8	Cancel–TAPDU

3.13:	CANCEL:

65:		SUBMISSION–ID:

[submission–msg–id]

1:		CORRELATION–INFORMATION:

[call–id]

B.2.9	Deliver–TAPDU

3.3:		DELIVER:

62:		QUANTITY–OF–DOCS:[number–of–docs]

13:		PRIORITY:[priority–ind]

14:		ORIGINATOR:

[originator–name]

39:		THIS–RECIPIENT:

[this–recipient–name]

40:		INTENDED–RECIPIENT:

[intended–recipient–name]

41:		OTHER–RECIPIENTS:

[ [other–recipient–name] `NL` ]

33:		SUBMISSION–TIME:

[submission–time]

35:		DELIVERY–ID:

[delivery–msg–id]

42:		CONVERSION–INDICATION:

[ [eIT], ] [conversion–prohibition]

44:		CONVERTED–INFORMATION–TYPES:

[ [eIT], ]

27:		THIS–IPM:

[this–ipm–id]

20:		FROM:

[originating–user]

21:		AUTHORIZING:

[ [authorizing–user] `NL` ]

22:		TO:

[ [primary–recipient] `NL`

=[rn–request],[nrn–request],[return–request],[reply–request] `NL` ]

23:		CC:

[ [copy–recipient] `NL`

=[rn–request],[nrn–request],[return–request],[reply–request] `NL` ]

24:		BCC:

[ [blind–copy–recipient] `NL`

=[rn–request],[nrn–request],[return–request],[reply–request] `NL` ]

30:		REPLIED–TO–IPM:

[replied–to–ipm–id]

29:		OBSOLETES:

[ [obsoleted–ipm–id] `NL` ]

28:		RELATED–IPMS:

[ [related–ipm–id] `NL` ]

26:		SUBJECT:

[subject–content]

18:		CONTENT–INDICATOR:

[expiry–time] `NL`

=[importance],[sensitivity],[autoforwarded]

25:		REPLY:

[reply–time] `NL`

[ [reply–recipient] `NL` ]

67:		EXTENSIONS:

[ [extension–type] `NL`

[extension–value] `NL` ]

31:		BODY–TYPE:[body–part]

32:		FORWARDED–INFO:

[forwarded–time] `NL`

//The delivery–envelope follows.

B.2.10 	ReceiptStatusNotice–TAPDU

3.5:		RECEIPT–STATUS–NOTICE:

62:		QUANTITY–OF–DOCS:[number–of–docs]

13:		PRIORITY:[priority–ind]

35:		DELIVERY–ID:

[delivery–id]

14:		ORIGINATOR:

[originator–name]

39:		THIS–RECIPIENT:

[this–recipient–name]

33:		SUBMISSION–TIME:

[submission–time]

4:		TIME–OF–DELIVERY:

[delivery–time]

42:		CONVERSION–INDICATION:

[ [eIT], ] [conversion–prohibition]

44:		CONVERTED–INFORMATION–TYPES:

[ [eIT], ]

43:		NOTIFICATION–TYPE:[report–type]

71:		SUBJECT–IPM:

[subject–ipm–id]

69:		IPN–ORIGINATOR:

[ipn–originating–user]

70:		PREFERRED–RECIPIENT:

[preferred–recipient]

//The following three elements appear in case of RN.

47:		TIME–OF–RECEIPT:

[receipt–time]

48:		TYPE–OF–RECEIPT:

[type–of–receipt]

68:		SUPPLEMENTARY–INFORMATION:

[suppl–receipt–info]

//The following three elements appear in case of NRN.

49:		NON–RECEIPT–INFO:

[non–receipt–reason],[discard–reason]

50:		COMMENTS:

[comments]

51:		MESSAGE–RETURNED–INDICATION:

B.2.11	DeliveryStatusNotice–TAPDU

3.4:		DELIVERY–STATUS–NOTICE:

62:		QUANTITY–OF–DOCS:[number–of–docs]

1:		CORRELATION–INFORMATION:

[call–id]

65:		SUBMISSION–ID:

[submission–msg–id]

66:		PROBE–ID:

[probe–msg–id]

[    //repeated for each recipient

3:		REPORTED–RECIPIENT:

[reported–recipient–name]

43:		NOTIFICATION–TYPE:[report–type]

40:		INTENDED–RECIPIENT:

[intended–recipient–name]

44:		CONVERTED–INFORMATION–TYPES:

[ [eIT], ]

//The following three elements appear in case of DN.

4:		TIME–OF–DELIVERY:

[delivery–time]

45:		TYPE–OF–UA:

[type–of–ua]

68:		SUPPLEMENTARY–INFORMATION:

[suppl–info]

//The following two elements appear in case of NDN.

46:		NON–DELIVERY–REASON:

[reason–code],[diagnostic–code]

]

72:		CONTENT–RETURNED–INDICATION:

B.2.12	Register–TAPDU

3.11:	REGISTER:

73:		EXPIRED–DISCARD:[discard–ipm]

74:		OBSOLETED–DISCARD:[discard–ipm]

76:		AUTO–FWD–IPMS:

[auto–fwd–ipms]

77:		AUTO–FWD–RECIPIENTS:

[ [auto–fwd–recipient], ]

78:		AUTO–FWD–HEADING:

[auto–fwd–heading]

79:		AUTO–FWD–COMMENT:

[auto–fwd–comment]

58:		DS–MODE:[ds–mode]

59:		TLMAU–OPERATION:[error–recovery–mode],[auto–acknowledgement]

60:		AUTO–OUTPUT:

[frequency] `NL`

[output–time]

81:		MESSAGE–DELETE–MODE:[message–delete–mode]

//and other components require further study.

B.2.13	RegisterAck–TPDU

3.11:	REGISTER–ACK:

B.2.14	DSQuery–TAPDU

3.7:		DS–QUERY:

B.2.15	DSReport–TAPDU

3.8:		DS–REPORT:

[    //repeated for each message

38:		RETRIEVAL–ID:

[retrieval–id]

52:		MESSAGE–TYPE:[message–type]

14:		ORIGINATOR:

[originator–name]

13:		PRIORITY:[priority–ind]

37:		MESSAGE–LENGTH:[message–length]

]

B.2.16	MessageDelete–TAPDU

3.18:	MESSAGE–DELETE:

82:		MESSAGE–SELECTOR:

[ [retrieval–id] `NL` ]

B.2.17  OutputRequest–TAPDU

3.9:		OUTPUT–REQUEST:

[    //repeated for each retrieval identifier

38:		RETRIEVAL–ID:

[retrieval–id]

80:		DELETE–AFTER–OUTPUT:[delete–after–output]

]

B.2.18  OutputMessage–TAPDU

3.10:	OUTPUT–MESSAGE:

[    //repeated for each message

62:		QUANTITY–OF–DOCS:[number–of–docs]

38:		RETRIEVAL–ID:

[retrieval–id]

52:		MESSAGE–TYPE:[message–type]

4:		TIME–OF–DELIVERY:

[delivery–time]

]

The remaining Components of this TAPDU are identical to the components in the Delivery. DeliveryStatusNotice and ReceiptStatusNotice–TAPDU. The actual components to be used depend upon the Message Type parameter value specified in the Message Type component.

B.2.19	StatusQuery–TAPDU

3.19:	STATUS–QUERY:

14:		ORIGINATOR:

[originator–name]

65:		SUBMISSION–ID:

[submission–msg–id]

1:		CORRELATION–INFORMATION:

[call–id]

B.2.20	StatusReport–TAPDU

3.20:	STATUS–REPORT:

1:		CORRELATION–INFORMATION:

[call–id]

84:		TIME–OF–REPORT:

[report–time]

75:		REPORTED–MESSAGE–ID:

[reported–message–id]

[    //repeated for each message

83:		MESSAGE–STATUS:

[status]

4:		TIME–OF–DELIVERY:

[delivery–time]

45:		TYPE–OF–UA:

[type–of–ua]

46:		NON–DELIVERY–REASON:

[reason–code],[diagnostic–code]

]

ANNEX C

(to Recommendation T.330)

Element ID list

This Annex provides a table which maps the coding of TAPDU and the corresponding element ID. The element ID list Table C–1/T.330 comprises:

1)	The first column contains the element number and element name, as defined in § 11 of this Recommendation.

2)	The second column contains the TAPDUId or ElementId name defined in § 10 of this Recommendation.

3)	The third column contains the type of the TAPDU element, defined in § 10 of this Recommendation.

4)	The fourth column contains the Remarks.

TABLE C–1/T.330

Element ID list

TAPDUId/element ID

Name

Type

Remarks

3.1:SEND:

send

–

3.2:PROBE:

probe

–

3.3:DELIVER:

deliver

–

3.4:DELIVERY–STATUS–NOTICE:

deliveryStatusNotice

–

3.5:RECEIPT–STATUS–NOTICE:

receiptStatusNotice

–

3.6:EXPLICIT–RN:

explicitRN

–

3.7:DS–QUERY:

dsQuery

–

3.8:DS–REPORT:

dsReport

–

3.9:OUTPUT–REQUEST:

outputRequest

–

3.10:OUTPUT–MESSAGE:

outputMessage

–

3.11:REGISTER:

register

–

3.12:EXCEPTION:

exception

–

3.13:CANCEL:

cancel

–

3.14:SEND–ACK:

sendAck

–

3.15:PROBE–ACK:

probeAck

–

3.16:EXPLICIT–RN–ACK:

explicitRNAck

–

3.17:REGISTER–ACK:

registerAck

–

3.18:MESSAGE–DELETE:

messageDelete

–

3.19:STATUS–QUERY:

statusQuery

–

3.20:STATUS–REPORT:

statusReport

–

1:CORRELATION–INFORMATION:

correlationInfo

Primitive

2:

Reserved for CF

3:REPORTED–RECIPIENT:

reportedRecipient

Primitive

4:TIME–OF–DELIVERY:

timeOfDelivery

Primitive

5:

Reserved for CF

6:

Reserved for CF

7:

Reserved for CF

8:

Reserved for CF

9:ERRORS:

errors

Primitive

10:

Reserved for CF

11:

Reserved for CF

12:RECEIVED–TID:

Reserved for CF

13:PRIORITY:

priority

Primitive

14:ORIGINATOR:

originator

Constructor

15:RECIPIENTS:

recipients

Constructor

16:CONVERSION:

conversion

Primitive

17:CONTENT–INFO:

contentInfo

Constructor

18:CONTENT–INDICATOR:

contentIndicator

Constructor

19:FLAG:

per Message Indicator

Constructor

TABLE C–1/T.330 (cont.)

TAPDUId/element ID

Name

Type

Remarks

20:FROM:

from

Primitive

21:AUTHORIZING:

authorizing

Constructor

22:TO:

to

Constructor

23:CC:

cc

Constructor

24:BCC:

bcc

Constructor

25:REPLY:

reply

Constructor

26:SUBJECT:

subject

Primitive

27:THIS–IPM:

thisIPM

Primitive

28:RELATED–IPMS:

relatedIPMS

Constructor

29:OBSOLETES:

obsoletedIPMS

Constructor

30:REPLIED–TO–IPM:

repliedToIPM

Primitive

31:BODY–TYPE:

bodyType

Constructor

32:FORWARDED–INFO:

forwardedInfo

Constructeur

For further study

33:SUBMISSION–TIME:

submissionTime

Primitive

34:LATEST–DELIVERY:

date and time

Primitive

35:DELIVERY–ID:

deliveryId

Primitive

36:RETURN–ADDRESS:

return address

Primitive

37:MESSAGE–LENGTH:

messageLength

Primitive

38:RETRIEVAL–ID:

retrievalId

Primitive

39:THIS–RECIPIENT:

thisRecipient

Primitive

40:INTENDED–RECIPIENT:

intendedRecipient

Primitive

41:OTHER–RECIPIENTS:

otherRecipients

Constructor

42:CONVERSION–INDICATION:

conversionIndication

Constructor

43:NOTIFICATION–TYPE:

notificationType

Primitive

44:CONVERTED–INFORMATION–TYPES:

convertedInfoTypes

Primitive

45:TYPE–OF–UA:

typeOfUA

Primitive

46:NON–DELIVERY–REASON:

nonDeliveryReason

Primitive

47:TIME–OF–RECEIPT:

timeOfReceipt

Primitive

48:TYPE–OF–RECEIPT:

typeOfReceipt

Primitive

49:NON–RECEIPT–INFO:

nonReceiptInfo

Primitive

50:COMMENT:

comment

Primitive

51:MESSAGE–RETURNED–INDICATION:

messageReturnedInd

–

52:MESSAGE–TYPE:

messageType

Primitive

53:LANGUAGE:

languageInd

Primitive

54:REDIRECTED–FROM:

redirected from

Constructor

TABLE C–1/T.330 (end)

TAPDUId/element ID

Name

Type

Remarks

55:

–

–

Not used

56:

–

–

Not used

57:

–

–

Not used

58:DS–MODE:

dsMode

Primitive

59:TLMAU–OPERATION:

tLMAUOperation

Constructor

60:AUTO–OUTPUT:

autoOutput

Constructor

61:

–

–

Not used

62:QUANTITY–OF–DOCS:

quantityOfDocs

Primitive

63:

Not assigned

64:

Not assigned

65:SUBMISSION–ID:

submissionId

Primitive

66:PROBE–ID:

probeId

Primitive

67:MSG–INCOMPLETE:

68:SUPPLEMENTARY–INFORMATION:

supplReceiptInfo

SupplInfo

Primitive

69:IPN–ORIGINATOR:

IPNOriginator

Primitive

70:PREFERRED–RECIPIENT:

preferredRecipient

Primitive

71:SUBJECT–IPM:

subjectIPM

Primitive

72:CONTENT–RETURNED–INDICATION:

contentReturned

–

73:EXPIRED–DISCARD:

expiredDiscard

Primitive

74:OBSOLETED–DISCARD:

obsoletedDiscard

Primitive

75:REPORTED–MESSAGE–ID:

reportedMessageId

Primitive

76:AUTO–FWD–IPMS:

autoFWDIPMs

Primitive

77:AUTO–FWD–RECIPIENTS:

autoFWDRecipient

Primitive

78:AUTO–FWD–HEADING:

autoFWDHeading

Constructor

For further study

79:AUTO–FWD–COMMENT:

autoFWDComment

Primitive

80:DELETE–AFTER–OUTPUT:

deleteAfterOutput

Primitive

81:MESSAGE–DELETE–MODE:

messageDeleteMode

Primitive

82:MESSAGE–SELECTOR:

messageSelector

Constructor

83:MESSAGE–STATUS:

messageStatus

Primitive

84:TIME–OF–REPORT:

timeOfReport

Primitive

Conventions:

1) Primitive: element contains only one component

2) Constructor: element contains more than one component

3) : space character

ANNEX D

(to Recommendation T.330)

Element of service for TTX/IPM service intercommunication

This PTXAU provides only the services listed in Table D–1/T.330 to telematic users. These services may be implemented using only the mhs–doc–xfer abstract operations: message send, message delivery, receipt status notice and delivery status notice.

When a non–registered user attempts to access a service not within the PTTXAU set of services, then a subscription error will be returned.

TABLE D–1/T.330

Element of service

Reference Rec. F.400 Annex B

F.400 Elements of service

Message submission from TTX to PTTXAU

Message delivery to TTX from PTTXAU

Information generated by PTTXAU

B.5

Authorizing users indication

X

B.6

Auto–forwarded indication

X

B.8

Blind copy recipient indication

X

B.9

Body part encryption indication

X

B.12

Content type indication

X

X

B.13

Conversion prohibition

X

X

B.15

Converted indication

X

B.18

Cross–referencing indication

X

B.21

Delivery notification

X

NA

X

B.22

Delivery time stamp indication

X

X

B.25

Disclosure of other recipients

X

B.26

DL expansion history indication

X

X

B.29

Expiry date indication

X

B.31

Forwarded IP–message indication

X

B.32

Grade of delivery selection

X

X

B.34

Implicit conversion

X

B.35

Importance indication

X

B.37

IP–message identification

X

B.38

Language indication

X

B.39

Latest delivery designation

X

B.41

Message indentification

X

B.45

Multi–destination delivery

X

NA

X

B.46

Multi–part body

X

B.47

Non–delivery notification

NA

B.48

Non–receipt notification request indication

X

NA

B.52

Obsoleting indication

X

B.54

Original encoded information types indication

X

X

B.55

Originator indication

X

B.56

Originator requested alternate recipient

X

X

B.62

Primary and copy recipients indication

X

B.72

Reply request indication

X

B.73

Replying IP–message indication

X

B.80

Sensitivity indication

X

B.88

Subject indication

X

X

B.89

Submission time stamp indication

X

NA	No applicable

X	Applicable