|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- |
| ITU-T RECOMMENDATION SUMMARY |  |  |  |
| Rec. No. : T.563 |  | Title : Terminal characteristics for group 4 facsimile apparatus |  |
| Study Group : VIII - Terminals for Telematic Services |  |  |  |
| Version : Revised |  | Date of adoption : 1991 | Notes : |
| Group 4 facsimile apparatus is used mainly on public data networks (PDN) including circuit-switched, packet-switched, and the integrated services digital network (ISDN). The apparatus may be also used on the public switched telephone network (PSTN) where an appropriate modulation process will be utilized. The procedures used with Group 4 facsimile apparatus enable it to transmit and reproduce image coded information essentially without transmission errors. Group 4 facsimile apparatus has the means for reducing the redundant information in facsimile signals prior to transmission. The basic image type of the Group 4 facsimile apparatus is black and white. Other image types, e.g. grey scale image or colour image are for further study. There are three classes of Group 4 facsimile terminals: – Class I – Minimum requirement is a terminal able to send and receive documents containing facsimile encoded information (in accordance with Recommendation T.6, T.503 and T.521). – Class II – Minimum requirement is a terminal able to transmit documents which are facsimile encoded (in accordance with Recommendations T.6, T.503 and T.521). In addition, the terminal must be capable of receiving documents which are facsimile coded (in accordance with Recommendations T.6, T.503 and T.521). Teletex coded (in accordance with the basic coded character repertoire as defined in Recommendations T.60 and T.61), and also mixed-mode documents (in accordance with Recommendation T.561). – Class III – Minimum requirement is a terminal which is capable of generating, transmitting and receiving facsimile coded documents (in accordance with the Recommendations T.6, T.503 and T.521), Teletex coded documents (in accordance with the basic coded character repertoire as defined in Recommendations T.60 and T.61), and mixed-mode documents (in accordance with Recommendation T.561). |  |  |  |
| To order the complete text of this Recommendation, please use the Order Form for ITU-T Recommendations. An electronic version of this form is available on ITUDOC (Winword 2.0: UPI=ITU-5265; ASCII: UPI=ITU-2488). |  |  |  |