|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- |
| ITU-T RECOMMENDATION SUMMARY |  |  |  |
| Rec. No. : V.32 bis |  | Title : A duplex modem operating at data signalling rates of up to 14 400 bit/s for use on the general switched telephone network and on leased point-to-point 2-wire telephone -type circuits |  |
| Study Group : XVII - Data Transmission over the Telephone Network |  |  |  |
| Version : New |  | Date of adoption : 1991 | Notes : |
| Recommendation V.32 bis presents a modem which is intended for use on connections on general switched telephone networks (GSTNs) and on point-to-point 2-wire leased telephone-type circuits. The principal characteristics of the modem are as follows: a) duplex mode of operation on GSTN and point-to-point 2-wire leased circuits; b) channel separation by echo cancellation techniques; c) quadrature amplitude modulation for each channel with synchronous line transmission at 2400 symbols/s; d) the following synchronous data signalling rates shall be implemented in the modem: – 14 400 bit/s trellis coded, – 12 000 bit/s trellis coded, – 9600 bit/s trellis coded, – 7200 bit/s trellis coded, – 4800 bit/s uncoded; e) compatibility with Recommendation V.32 modems at 9600 and 4800 bit/s; f) exchange of rate sequences during start-up to establish the data signalling rate; g) a procedure to change the data signalling rate without retraining. Note 1 – On international GSTN connections that utilize circuits that are in accord with Recommendation G.235 (16-channel terminal equipments), it may be necessary to employ a greater degree of equalization within the modem than would be required for use on most national GSTN connections. Note 2 – The transmit and receive rates in each modem shall be the same. The possibility of asymmetric working remains for further study. |  |  |  |
| To order the complete text of this Recommendation, please use the Order Form for ITU-T Recommendations. An electronic version of this form is available on ITUDOC (Winword 2.0: UPI=ITU-5265; ASCII: UPI=ITU-2488). |  |  |  |