The drawings contained in this recommendation have been done in autocad

Recommendation X.420

xe ""§MESSAGE HANDLING SYSTEMS:
INTERPERSONAL MESSAGING SYSTEM1)

(MalagaÑTorremolinos, 1984; amended at Melbourne, 1988 )

The establishment in various countries of telematic services and computerÑbased storeÑandÑforward message services in association with public data networks creates a need to produce standards to facilitate international message exchange between subscribers to such services.

The CCITT,

considering

(a)	the need for message handling systems;

(b)	the need to transfer and store messages of different types;

(c)	that Recommendation X.200 defines the reference model of open systems interconnection for CCITT applications;

(d)	that Recommendations X.208, X.217, X.218 and X.219 provide the foundation for CCITT applications;

(e)	that the X.500Ñseries Recommendations define directory systems;

(f)	that message handling systems are defined in a series of Recommendations X.400, X.402, X.403, X.407, X.408, X.411, X.413, and X.419;

(g)	that interpersonal messaging is defined in Recommendations X.420 and T.330,

unanimously declares

(1)	that the abstract information objects users exchange in interpersonal messaging are defined in Section 2;

(2)	that the abstract service offered to users in interpersonal messaging is defined in Section 3;

(3)	that how that abstract service is provided is specified in Section 4.

TABLE OF CONTENTS

SECTION 1 Ñ Introduction

0	Introduction

1	Scope

2	References

3	Definitions

4	Abbreviations

5	Conventions

5.1		ASN.1

5.2		Grade

5.3		Terms

SECTION 2 Ñ Abstract information objects

6	Overview

7	Interpersonal messages

7.1		Heading fields component types

7.2		Heading fields

7.3		Body part types

8	Interpersonal notifications

8.1		Common fields

8.2		NonÑreceipt fields

8.3		Receipt fields

SECTION 3 Ñ Abstract service definition

9	Overview

10	Primary object types

10.1		Interpersonal messaging system user

10.2		Interpersonal messaging system

11	Primary port types

11.1		Origination

11.2		Reception

11.3		Management

12	Abstract operations

12.1		Origination abstract operations

12.2		Reception abstract operations

12.3		Management abstract operations

13	Abstract errors

13.1		Subscription error

13.2		Recipient improperly specified

14	Other capabilities

SECTION 4 Ñ Abstract service provision

15	Overview

16	Secondary object types

16.1		Interpersonal messaging system user agent

16.2		Interpersonal messaging system message store

16.3		Telematic agent

16.4		Telex access unit

16.5		Physical delivery access unit

16.6		Message transfer system

17	Secondary port types

17.1		Submission

17.2		Delivery

17.3		Retrieval

17.4		Administration

17.5		Import

17.6		Export

18	User agent operation

18.1		State variables

18.2		Performance of origination operations

18.3		Performance of management operations

18.4		Invocation of reception operations

18.5		Internal procedures

19	Message store operation

19.1		Creation of information objects

19.2		Maintenance of attributes

19.3		Notification of nonÑreceipt

19.4		AutoÑforwarding

20	Message contents

20.1		Content

20.2		Content type

20.3		Content length

20.4		Encoded information types

21	Port realization

22	Conformance

22.1		Originating versus reception

22.2		Statement requirements

22.3		Static requirements

22.4		Dynamic requirements

Annex A	Ñ	Heading extensions

Annex B	Ñ	Extended body part types

Annex C	Ñ	Message store attributes

Annex D	Ñ	Reference definition of object identifiers

Annex E	Ñ	Reference definition of abstract information objects

Annex F	Ñ	Reference definition of functional objects

Annex G	Ñ	Reference definition of abstract service

Annex H	Ñ	Reference definition of heading extensions

Annex I	Ñ	Reference definition of extended body part types

Annex J	Ñ	Reference definition of message store attributes

Annex K	Ñ	Reference definition of upper bounds

Annex L	Ñ	Support of the interpersonal messaging service

Annex M	Ñ	Differences between CCITT Recommendation and ISO Standard

Annex N	Ñ	Summary of changes to 1984 specification

SECTION 1 Ñ INTRODUCTION

# 0	Introduction

This Recommendation is one of a set of Recommendations for message handling. The entire set provides a comprehensive blueprint for a message handling system (MHS) realized by any number of cooperation open systems.

The purpose of an MHS is to enable users to exchange messages on a storeÑandÑforward basis. A message submitted on behalf of one user, the originator, is conveyed by the message transfer system (MTS) and subsequently delivered to the agents of one or more additional users, the recipients. Access units (AUs) link the MTS to communication systems of other kinds (e.g., postal systems). A user is assisted in the preparation, storage, and display of messages by a user agen (UA). Optionally, it is assisted in the storage of messages by a message store (MS). The MTS comprises a number of message transfer agents (MTAs) which collectively perform the storeÑandÑforward message transfer function.

This Recommendation defines the message handling application called interpersonal messaging, specifying in the process the message content type and associated procedures known as P2.

The text of this Recommendation is the subject of joint CCITTÑISO agreement. The corresponding ISO/IEC specification is ISO 10021Ñ7.

# 1	Scope

This Recommendation defines interpersonal messaging, a form of message handling tailored for ordinary interpersonal business or private correspondence.

This Recommendation is one of a series on message handling. Recommendation X.402 constitutes the introduction to the series and identifies the other documents in it.

The architectural basis and foundation for message handling are defined in still other Recommendations. Recommendation X.402 identifies those documents as well.

This Recommendation is structured as follows. Section 1 is this introduction. Section 2 defines the kinds of information objects exchanged in interpersonal messaging. Section 3 defines de associated abstract service. Section 4 specifies how it is provided. Annexes provide important supplemental information.

The requirements for conformance to this Recommendation are given in ¤ 22.

# 2	References

This Recommendation cites Recommendation X.402, many of the documents it cites, and those below.

ISO Standard 639.2			Code for the representation of names of languages.

Recommendation T.4			Standardization of Group 3 facsimile apparatus for document transmission.

Recommendation T.30		Procedures for document facsimile transmission in the general switched telephone network.

Recommendation T.100			International information exchange for interactive videotex.

Recommendation T.101			International interworking for videotex services.

Recommendation T.330			Telematic access to IPMS.

Recommendation X.420 (1984)	Message handling systems: Interpersonal messaging user agent layer.

X.400Ñseries implementor's guide, Version 6, 6 November 1987.

# 3	Definitions

For the purposes of this Recommendation, the definitions of Recommendation X.402 apply.

# 4	Abbreviations

For the purposes of this Recommendation, the abbreviations of Recommendation X.402 apply.

# 5	Conventions

This Recommendation uses the descriptive conventions identified below.

## 5.1	ASN.1

This Recommendation uses for the indicated purposes the following ASN.1Ñbased descriptive conventions:

a)	to define the information objects of interpersonal messaging, and other data types and values of all kinds, ASN.1 itself;

b)	to define the functional objects of interpersonal messaging, the OBJECT and REFINE macros of Recommendation X.407;

c)	to define the abstract service of interpersonal messaging, the PORT and ABSTRACTÑOPERATION and ÑERROR macros of Recommendation X.407;

d)	to define the heading extensions, the HEADINGÑEXTENSION macro of ¤ 7.2.17;

e)	to define extended body part types, the EXTENDEDÑBODYÑPARTÑTYPE macro of ¤ 7.3.12;

f)	to define MS attributes, the ATTRIBUTE macro of Recommendation X.500.

The various uses of the ASN.1 notation are summarized in Table 1/X.420. With the two exceptions evident from the table, whenever ASN.1 is employed, it appears both in the body of the Recommendation to aid the exposition, and again, largely redundantly, in an Annex for reference.

TABLE 1/X.420

Uses of the ASN.1 notation

| Subject matter | Exposition | Reference |
| --- | --- | --- |
| Object identifiers | Ñ | Annex D |
| Abstract information objects | Section 2 | Annex E |
| Functional objects | ¤¤ 10, 11, 16 | Annex F |
| Abstract service | ¤¤ 12Ñ13 | Annex G |
| Heading extensions | Annex A | Annex H |
| Extended body part types | Annex B | Annex I |
| Message store attributes | Annex C | Annex J |
| Upper bounds | Ñ | Annex K |

If differences are found between the ASN.1 used in the exposition and that supplied for reference, a specification error is indicated.

Note that ASN.1 tags are implicit throughout the ASN.1 module the Annex defines; the module is definitive in that respect.

Note 1 Ñ The use of ASN.1 to describe a class or piece of information does not in itself imply that that information is transported between open systems. The fact that the information, by virtue of its description in ASN.1 and of ASN.1's basic encoding rules, has a concrete transfer syntax may be immaterial. Information actually conveyed between systems is designated as such by its inclusion in an application protocol.

Note 2 Ñ The use of the ABSTRACTÑOPERATION and ÑERROR macros, derived from the correspondingly named macros of remote operations, does not imply that the abstract operations and errors are invoked and reported across the boundary between open systems. The fact that the abstract operations and errors, by virtue of their description using these macros and with minimal additional specification, actually could be invoked via ROS is immaterial in the present context.

## 5.2	Grade

This Recommendation uses the concept of grade as developed in Recommendation X.402.

## 5.3	Terms

Throughout this Recommendation, terms are rendered in bold when defined, in italic when referenced prior to their definitions, without emphasis upon all other occasions.

Terms that are proper nouns are capitalized, generic terms are not.

SECTION 2 Ñ ABSTRACT INFORMATION OBJECTS

# 6	Overview

This section abstractly describes the information objects that users exchange in interpersonal messaging. They are of two kinds, interpersonal messages (IPMs) and interpersonal notifications (IPNs). One of the latter acknowledges a user's receipt of one of the former.

InformationObject ::= CHOICE {

ipm	[0] IPM,

ipn	[1] IPN }

This section covers the following topics:

a)	interpersonal messages;

b)	interpersonal notifications.

Note 1 Ñ The use, throughout this section, of words such as ÒoriginatorÓ and ÒrecipientÓ anticipates the fact that IPMs and IPNs are conveyed between users as the contents of messages (see ¤ 20). These words, therefore, refer to the roles users and DLs play in such transmittals.

Note 2 Ñ An IPM may appear (see ¤ 7.3.8) in the body of another IPM which itself is conveyed as the content of a message. The words ÒoriginatorÓ and ÒrecipientÓ shall be understood in the context of an IPM's conveyance as the (entire) content of a message, not as a component of the body of another IPM so conveyed.

Note 3 Ñ An IPM or IPN makes various assertions about its own transmittal (e.g., who originates the message containing it). Furthermore, an IPN makes assertions about the transmittal of the IPM to which it responds. All of these assertions are unverified.

# 7	Interpersonal messages

An interpersonal message (IPM) is a member of the primary class of information object conveyed between users in interpersonal messaging.

IPM ::= SEQUENCE {

heading	Heading,

body		Body }

It has the following components:

a)	heading: A set of heading fields (or fields), each an information item that gives a characteristic of the IPM (e.g., its importance);

b)	body: A sequence of body parts, each an information object that the IPM is intended to convey between users (e.g., a document).

Body ::= SEQUENCE OF BodyPart

The structure of an IPM is depicted in Figure 1/X.420.

This paragraph defines and describes the most prominent heading field component types and the defined heading fields and body part types.

Note Ñ An IPM may be likened to a business memo. In fact, the terms ÒheadingÓ and ÒbodyÓ appeal to that analogy.

Fig. 1/X.420/T0705270-88 = 7 cm

## 7.1	Heading field component types

Information items of several kinds appear throughout the heading. These heading field component types ÑÑIPM identifier, recipient specifier, and O/R descriptorÑÑ are defined and described below.

### 7.1.1	xe ""§IPM identifier

An IPM identifier is an information item that unambiguously and uniquely identifies an IPM, distinguishing it from all other IPMs ever conveyed by any user.

IPMIdentifier ::= [APPLICATION 11] SET {

user					ORAddress OPTIONAL,

userÑrelativeÑidentifier		LocalIPMIdentifier }

An IPM identifier has the following components:

a)	user (O): Identifies the user who originates the IPM. One of the user's O/R addresses. This component's omission is discouraged;

b)	xe ""§userÑrelativeÑidentifier (M): Uniquely and unambiguously identifies the IPM, distinguishing it from all other IPMs that the user who is identified by the user component originates. A printable string of from zero to a prescribed number of characters (see Annex K). A length of zero is discouraged.

LocalIPMIdentifier ::= PrintableString

(SIZE (0. .ubÑlocalÑipmÑidentifier))

Note Ñ The Ò11Ó in IPMIdentifier is the only ASN.1 applicationÑwide tag this Recommendation assigns.

### 7.1.2	xe ""§Recipient specifier

A recipient specifier is an information item that identifies a (preferred) recipient of an IPM and that may make certain requests of him.

RecipientSpecifier ::= SET {

recipient			[0]	ORDescriptor,

notificationÑrequests		[1]	NotificationRequests DEFAULT { },

replyÑrequested		[2]	BOOLEAN DEFAULT FALSE }

A recipient specifier has the following components:

a)	recipient (M): Identifies the preferred recipient in question. An O/R descriptor.

If the notificationÑrequests or replyÑrequested component makes a request of the preferred recipient, the formalÑname component of the O/R descriptor above shall be present.

b)	notificationÑrequests (D no values): May make certain requests of the preferred recipient denoted by the recipient component.

NotificationRequests ::= BIT STRING {

rn				(0),

nrn				(1),

ipmÑreturn		(2) }

This component may assume any of the following values simultaneously, except tha the value rn shall not be selected unless the value nrn is selected:

i)	rn: A receipt notification is requested in the circumstances prescribed in ¤ 8.

ii)	nrn: A nonÑreceipt notification is requested in the circumstances prescribed in ¤ 8.

iii)	ipmÑreturn: It is requested that the IPM be returned in any nonÑreceipt notification.

c)	replyÑrequested (D false): Indicates whether a reply is requested of the preferred recipient denoted by the recipient component. A Boolean.

A reply is one IPM sent in response to another. A user may reply to an IPM even though no reply is requested of him and, indeed, even if he is not among the IPM's preferred recipients. Furthermore, a user of whom a reply is requested may refrain from replying.

### 7.1.3	xe ""§O/R descriptor

An O/R descriptor is an information item that identifies a user or DL.

ORDescriptor ::= SET {

formalÑname			ORName OPTIONAL,

freeÑformÑname	[0]	FreeFormName OPTIONAL,

telephoneÑnumber	[1]	TelephoneNumber OPTIONAL }

An O/R descriptor has the following components:

a)	formalÑname (C): Identifies the user or DL in question. One of its O/R names.

This conditional component shall be present if (but not only if) one or more of the following criteria are satisfied:

i)	The freeÑformÑname component is absent.

ii)	The O/R descriptor appears in the reply recipients heading field.

iii)	The O/R descriptor is the recipient component of a recipient specifier and the conditions stated in item 2 of ¤ 7.1.2 are satisfied.

b)	xe ""§freeÑformÑname (O): Identifies the user or DL in question. A telex string of from zero to a prescribed number of characters (see Annex K), chosen from the graphic subset of the teletex string character set. A length of zero is discouraged.

FreeFormName ::= TeletexString (SIZE (0. .ubÑfreeÑformÑname))

c)	xe ""§telephoneÑnumber (O): Provides the telephone number of the user or DL in question. A printable string of from zero to a prescribed number of characters (see Annex K), chosen from the graphical subset of the printable string character set. A length of zero is discouraged.

TelephoneNumber ::= PrintableString (SIZE (0. .ubÑtelephoneÑnumber))

Note Ñ One or more O/R descriptors may appear in each of the following heading fields: originator, authorizing users, primary recipients, copy recipients, blind copy recipients, and reply recipients. In addition, and O/R descriptor may appear in the following notification fields (see ¤ 8): IPN originator and IPM preferred recipient.

## 7.2	Heading fields

The fields that appear in the heading of an IPM are defined and described below.

Heading ::= SET {

thisÑIPM								ThisIPMField,

originator						[0]		OriginatorField OPTIONAL,

authorizingÑusers				[1]		AuthorizingUsersField OPTIONAL,

primaryÑrecipients			[2]		PrimaryRecipientsField DEFAULT { },

copyÑrecipients				[3]		CopyRecipientsField DEFAULT { },

blindÑcopyÑrecipients			[4]		BlindCopyRecipientsField OPTIONAL,

repliedÑtoÑIPM				[5]		RepliedToIPMField OPTIONAL,

obsoletedÑIPMs				[6]		ObsoletedIPMsField DEFAULT { },

relatedÑIPMs					[7]		RelatedIPMsField DEFAULT { },

subject							[8]		EXPLICIT SubjectField OPTIONAL,

expiryÑtime					[9]		ExpiryTimeField OPTIONAL,

replyÑtime						[10]	ReplyTimeField OPTIONAL,

replyÑrecipients				[11]	ReplyRecipientsField OPTIONAL,

importance						[12]	ImportanceField DEFAULT normal,

sensitivity						[13]	SensitivityField OPTIONAL,

autoÑforwarded				[14]	AutoForwardedField DEFAULT FALSE,

extensions						[15]	ExtensionsField DEFAULT { } }

Some fields have components and thus are composite, rather than indivisible. A field component is called a subÑfield.

### 7.2.1	xe ""§This IPM

The this IPM heading field (M) identifies the IPM. It comprises an IPM identifier.

ThisIPMField ::= IPMIdentifier

### 7.2.2	xe ""§Originator

The originator heading field (O) identifies the IPM's originator. It comprises an O/R descriptor.

OriginatorField ::= ORDescriptor

### 7.2.3	xe ""§Authorizing users

The authorizing users heading field (C) identifies the zero or more users who are the IPM's authorizing users. It comprises a sequence of subÑfields, each an O/R descriptor, one for each such user.

AuthorizingUsersField ::= SEQUENCE OF AuthorizingUsersSubfield

AuthorizingUsersSubfield ::= ORDescriptor

An authorizing user is a user who, either individually or in concert with others, authorizes the origination of an IPM. The word ÒauthorizesÓ above is not precisely defined by this Recommendation; it is given meaning by users.

This conditional field shall be present if, and only if, the authorizing users are other than the IPM's originator alone.

Note Ñ Suppose, for example, that a manager instructs his secretary to originate an IPM on his behalf. In this case, the secretary, the IPM's originator, might consider the manager the authorizing user.

### 7.2.4	xe ""§Primary recipients

The primary recipients heading field (D not subfields i.e., elements) identifies the zero or more users and DLs who are the Òprimary recipientsÓ of the IPM. It also identifies the responses the authorizing users ask of each of those users and of each member of those DLs. It comprises a Sequence of subÑfields, each a recipient specifier, one for each primary recipient.

PrimaryRecipientsField ::= SEQUENCE OF PrimaryRecipientsSubfield

PrimaryRecipientsSubfield ::= RecipientSpecifier

The phrase Òprimary recipientsÓ above is not precisely defined by this Recommendation; it is given meaning by users.

Note Ñ The primary recipients, e.g., might be those users and those DLs whose members are expected to act upon the IPM.

### 7.2.5	xe ""§Copy recipients

The copy recipients heading field (D no subfields i.e., elements) identifies the zero or more users and DLs who are the Òcopy recipientsÓ of the IPM. It also identifies the responses the authorizing users ask of each of those users and of each member of those DLs. It comprises a sequence of subÑfields, each a recipient specifier, one for each copy recipient.

CopyRecipientsField ::= SEQUENCE OF CopyRecipientsSubfield

CopyRecipientsSubfield ::= RecipientSpecifier

The phrase Òcopy recipientsÓ above is not precisely defined by this Recommendation; it is given meaning by users.

Note Ñ The copy recipients, for example, might be those users to whom, and those DLs to whose members the IPM is conveyed for information.

### 7.2.6	xe ""§Blind copy recipients

The blind copy recipients heading field (C) identifies zero or more users and DLs who are intended blind copy ÒrecipientsÓ of the IPM. It also identifies the responses the authorizing users ask of each of those users and of each member of those DLs. It comprises a sequence of subÑfields, each a recipient specifier, one for each blind copy recipient.

BlindCopyRecipientsField ::= SEQUENCE OF BlindCopyRecipientsSubfield

BlindCopyRecipientsSubfield ::= RecipientSpecifier

The phrase Òcopy recipientsÓ above has the same meaning as in ¤ 7.2.5. A blind copy recipient is one whose role as such is disclosed to neither primary nor copy recipients.

In the instance of an IPM intended for a blind copy recipient, this conditional field shall be present and identify that user or DL. Whether it shall also identify the other blind copy recipients is a local matter. In the instance of the IPM intended for a primary or copy recipient, the field shall be absent or identify no users or DLs.

### 7.2.7	xe ""§RepliedÑto IPM

The repliedÑto IPM heading field (C) identifies the IPM to which the present IPM is a reply. It comprises an IPM identifier.

RepliedToIPMField ::= IPMIdentifier

This conditional field shall be present if, and only if, the IPM is a reply.

Note Ñ In the context of forwarding, care should be taken to distinguish between the forwarding IPM and the forwarded IPM. This field should identify whichever of these two IPMs to which the reply responds.

### 7.2.8	xe ""§Obsoleted IPMs

The obsoleted IPMs heading field (D no subfields i.e., elements) identifies zero or more IPMs that the authorizing users of the present IPM consider it to obsolete. It comprises a sequence of subÑfields, each an IPM identifier, one for each IPM.

ObsoletedIPMsField ::= SEQUENCE OF ObsoletedIPMsSubfield

ObsoletedIPMsSubfield ::= IPMIdentifier

Note Ñ In the context of forwarding, care should be taken to distinguish between the forwarding IPM and the forwarded IPM. This field should identify whichever of these two IPMs the present IPM obsoletes.

### 7.2.9	xe ""§Related IPMs

The related IPMs heading field (D no subfields i.e., elements) identifies zero or more IPMs that the authorizing users of the present IPM consider related to it. It comprises a sequence of subÑfields, each an IPM identifier, one for each IPM.

RelatedIPMsField ::= SEQUENCE OF RelatedIPMsSubfield

RelatedIPMsSubfield ::= IPMIdentifier

The word ÒrelatedÓ above is not precisely defined by this Recommendation; it is given meaning by users.

Note 1 Ñ A related IPM, e.g., might be one discussed in the body of the present IPM.

Note 2 Ñ In the context of forwarding, care should be taken to distinguish between the forwarding IPM and the forwarded IPM. This field should identify whichever of these two IPMs is related to the present IPM.

### 7.2.10	xe ""§Subject

The subject heading field (O) identifies the subject of the IPM. It comprises a teletex string of from zero to a prescribed number of characters (see Annex K), chosen from the graphic subset of the teletex string character set. A length of zero is discouraged.

SubjectField ::= TeletexString (SIZE (0. .ubÑsubjectÑfield))

### 7.2.11	xe ""§Expiry time

The expiry time heading field (O) identifies when the authorizing users consider the IPM to lose its validity. It comprises a date and time.

ExpiryTimeField ::= Time

### 7.2.12	xe ""§Reply time

The reply time heading field (O) identifies by when the authorizing users request (but do not demand) that any replies to the present IPM be originated. It comprises a date and time.

ReplyTimeField ::= Time

### 7.2.13	xe ""§Reply recipients

The reply recipients heading field (C) identifies zero or more users and DLs whom the authorizing users request (but do not demand) be among the preferred recipients of any replies to the present IPM. It comprises a sequence of subÑfields, each an O/R descriptor, one for each user or DL.

ReplyRecipientsField ::= SEQUENCE OF ReplyRecipientsSubfield

ReplyRecipientsSubfield ::= ORDescriptor

This conditional field shall be present if, and only if, the desired reply recipients are other than the originator of the present IPM alone.

Note Ñ If this field is present and identifies several users and DLs, the originator may include himself among them. If he elects not to do so, he will not be considered among the desired reply recipients.

### 7.2.14	xe ""§Importance

The importance heading field (D normal) identifies the importance that the authorizing users attach to the IPM. It may assume any one of the following values: low, normal, or high.

ImportanceField ::= ENUMERATED {

low			(0),

normal	(1),

high			(2) }

The values above are not defined by this Recommendation; they are given meaning by users.

### 7.2.15	xe ""§Sensitivity

The sensitivity heading field (C) identifies the sensitivity that the authorizing users attribute to the IPM.

SensitivityField ::= ENUMERATED {

personal			(1),

private			(2),

companyÑconfidential		(3) }

This field may assume any one of the following values:

a)	personal: The IPM is conveyed to its preferred recipients as individuals, rather than in their professional capacities.

b)	private: The IPM should be conveyed to no one other than its preferred recipients.

c)	companyÑconfidential: The IPM contains information that should be handled according to companyÑspecific procedures.

The conditional field shall be present if, and only, if the IPM is sensitive.

### 7.2.16	xe ""§AutoÑforwarded

The autoÑforwarded heading field (D false) indicates whether the IPM is the result of autoÑforwarding. It is a Boolean.

AutoForwardedField ::= BOOLEAN

### 7.2.17	xe ""§Extensions

The extensions heading field (D no extensions, i.e., members) conveys information accommodated by no other heading field. It comprises a Set of zero or more heading extensions (or extensions), each conveying one such information item.

ExtensionsField ::= SET OF HeadingExtension

HeadingExtension ::= SEQUENCE {

type			OBJECT IDENTIFIER,

value		ANY DEFINED BY type DEFAULT NULL NULL }

Each extension has the following components:

a)	type (M): Identifies the semantics and restricts the abstract syntax of the value component. An object identifier.

b)	value (D null): An information item whose abstrct syntax is restricted only by the type component. An Any.

The type components of all the extensions in the extensions field shall differ. Not every defined extension need appear in the field.

All extensions are defined in Annex A. Thus, each extension's type component shall have one of the values given in that Annex. An extension whose type component has another value shall be ignored.

Every extension is defined by means of the following macro.

HEADINGÑEXTENSION MACRO ::=

BEGIN

TYPE		NOTATION ::= ÒVALUEÓ type | empty

VALUE	NOTATION ::= value (VALUE OBJECT IDENTIFIER)

END

An instance of the macro's type notation identifies the data type to which the extension's value component shall be restricted. If no type is identified explicitly, null is implied.

An instance of the macro's value notation identifies the object identifier that shall appear as the extension's type component.

Note Ñ Future versions of this Recommendation may define additional extensions. Furthermore, future versions are likely to add to the heading only by means of this field.

## 7.3	xe ""§Body part types

The types of body parts that may appear in the body of an IPM are defined and described below:

BodyPart ::= CHOICE {

ia5Ñtext						[0]		IA5TextBodyPart,

voice						[2]		VoiceBodyPart,

g3Ñfacsimile				[3]		G3FacsimileBodyPart,

g4Ñclass1					[4]		G4Class1BodyPart,

teletex						[5]		TeletexBodyPart,

videotex					[6]		VideotexBodyPart,

encrypted					[8]		EncryptedBodyPart,

message					[9]		MessageBodyPart,

mixedÑmode				[11]	MixedModeBodyPart,

bilaterallyÑdefined		[14]	BilaterallyDefinedBodyPart,

nationallyÑdefined		[7]		NationallyDefinedBodyPart,

externallyÑdefined		[15]	ExternallyDefinedBodyPart }

Body parts of some of the types defined below have two components, parameters and data. The parameters component (M) comprises a sequence of information items that describe the information object the body part represents and that typically are format and control parameters. The data component (M) is the information object itself.

Note 1 Ñ In Recommendation X.420 (1984), contextÑspecific tags 1 and 10 denote telex and simple formattable document body parts, respectively, which are no longer defined. These tags, therefore, are avoided in BodyPart.

Note 2 Ñ Under some circumstances, an IPM may be subjected to conversion while in transit between users. Such a transmittal event may alter a body part's type.

### 7.3.1	xe ""§IA5 text

An IA5 text body part represents IA5 text. It has parameters and data components.

IA5TextBodyPart ::= SEQUENCE {

parameters		IA5TextParameters,

data				IA5TextData }

IA5TextParameters ::= SET {

repertoire [0] Repertoire DEFAULT ia5 }

IA5TextData ::= IA5String

The parameters component comprises the following parameters:

a)	repertoire (D IA5): Identifies the character set to which the data component is constrained.

Repertoire ::= ENUMERATED {

ita2 (2),

ia5  (5) }

This parameter may assume one of the following values:

i)	ITA2: The data component shall be limited to ITA2 (i.e., telex) character set.

ii)	IA5: The data component may draw upon the full IA5 character set.

The data component is the text, an IA5 string. It may contain lines of any length. Whenever the component is rendered (e.g., displayed to or printed for a user), all (rather than only a part) of the text must be communicated (e.g., lines may be folded but shall not be truncated).

Note Ñ Many terminals have a maximum line length of 80 characters. Therefore, lines that do not exceed that length are most likely to be satisfactorily rendered (e.g., are most likely to avoid being folded).

### 7.3.2	xe ""§Voice

A voice body part represents speech. It has parameter and data components.

VoiceBodyPart ::= SEQUENCE {

parameters		VoiceParameters,

data				VoiceData }

VoiceParameters ::= SET ÑÑ for further study

VoiceData ::= BIT STRING ÑÑ for further study

The parameters of such a body part, and the digitized speech encoding technique that those parameters might identify and parameterize, are for further study.

The data component is the speech, a bit string.

### 7.3.3	xe ""§G3 facsimile

A G3 facsimile body part represents Group 3 facsimile images. It has parameters and data components.

G3FacsimileBodyPart ::= SEQUENCE {

parameters			G3FacsimileParameters,

data					G3FacsimileData }

G3FacsimileParameters ::= SET {

numberÑofÑpages		[0]	INTEGER OPTIONAL,

nonÑbasicÑparameters	[1]	G3FascimileNonBasicParameters OPTIONAL }

G3FacsimileData ::= SEQUENCE OF BIT STRING

The parameters component comprises the following parameters:

a)	numberÑofÑpages (O): Identifies the number of pages of Group 3 facsimile data present in the data component. A nonÑnegative integer.

b)	nonÑbasicÑparameters (C): Identifies the nonÑbasic parameters (NBPs) for Group 3 facsimile that characterize the data component. A G3 NBPs descriptor.

This conditional parameter shall be present if (but not only if) the body contains two or more G3 facsimile body parts.

The data component is the facsimile images, a sequence of bit strings, each encoding a single page of GroupÊ3 facsimile data as dictated by Recommendations T.4 and T.30.

Note 1 Ñ The numberÑofÑpages component identifies the number of elements in the sequence that constitutes the data component and is thus redundant.

Note 2 Ñ If the body comprises a single such body part, its NBPs may (but need not) be conveyed by means of the envelope of the message that contains the IPM.

### 7.3.4	xe ""§G4 Class 1

A G4 Class 1 body part represents a finalÑform of document of the sort that is processable by Group 4 Class 1 facsimile terminals. It comprises a sequence of protocol elements which describe the document's layout structure.

G4Class1BodyPart ::= SEQUENCE OF ProtocolElement

### 7.3.5	xe ""§Teletex

A teletex body part represents a teletex document. It has parameters and data components.

TeletexBodyPart ::= SEQUENCE {

parameters				TeletexParameters,

data						TeletexData }

TeletexParameters ::= SET {

numberÑofÑpages				[0]		INTEGER OPTIONAL,

telexÑcompatible				[1]		BOOLEAN DEFAULT FALSE,

nonÑbasicÑparameters		[2]		TeletexNonBasicParameters OPTIONAL }

TeletexData ::= SEQUENCE OF TeletexString

The parameters component comprises the following parameters:

a)	numberÑofÑpages (O): Identifies the number of pages of teletex text present in the data component. A nonÑnegative integer.

b)	telexÑcompatible (D false): Indicates whether the document in the data component is telexÑcompatible. A Boolean.

If this parameter has the value true, every teletex string in the data component shall be restricted to the ITA2 character set. No line shall exceed 69 characters in length.

c)	nonÑbasic parameters (C): Identifies the NBPs for teletex that characterize the data component. A teletex NBPs descriptor.

This conditional parameter shall be present if (but not only if) the body contains two or more teletex body parts.

The data component is the document, a sequence of teletex strings, each of which encodes one of its pages.

Note 1 Ñ The numberÑofÑpages component identifies the number of elements in the sequence that constitutes the data components and is thus redundant.

Note 2 Ñ If the body comprises a single such body part, its NBPs may (but need not) be conveyed by means of the envelope of the message that contains the IPM.

### 7.3.6	xe ""§Videotex

A videotex body part represents videotex data. It has parameters and data components.

VideotexBodyPart ::= SEQUENCE {

parameters		VideotexParameters,

data				VideotexData }

VideotexParameters ::= SET {

syntax			[0] VideotexSyntax OPTIONAL }

VideotexData ::= VideotexString

The parameters component comprises the following parameters:

a)	syntax (O): Identifies the syntax of the data component. In the parameter's absence, the syntax shall be considered unspecified.

VideotexSyntax ::= INTEGER {

ids						(0),

dataÑsyntax1			(1),

dataÑsyntax2			(2),

dataÑsyntax3			(3) }

This parameter may assume any one of the following values, each of which denotes as follows one of the videotex syntaxes defined in Recommendations T.100 and T.101:

i)	ids: The IDS syntax.

ii)	dataÑsyntax1: Data syntax 1.

iii)	dataÑsyntax2: Data syntax 2.

iv)	dataÑsyntax3: Data syntax 3.

The data component is the videotex data, a videotex string. It shall conform to the videotex syntax denoted by the syntax parameter.

### 7.3.7	xe ""§Encrypted

An encrypted body part represents the result of encrypting a body part of a type defined by this Recommendation. It has parameters and data components.

EncryptedBodyPart ::= SEQUENCE {

parameters		EncryptedParameters,

data				EncryptedData }

EncryptedParameters ::= SET ÑÑ for further study

EncryptedData ::= BIT STRING ÑÑ for further study

The parameters of such a body part, and the encryption technique that those parameters might identify and parameterize, are for further study.

The data component is the encrypted body part, a bit string. The bits of the bit string shall encrypt a data value of (ASN.1) type BodyPart encoded in accordance with the basic encoding rules of Recommendation X.209.

### 7.3.8	xe ""§Message

A message body part represents an IPM and, optionally, its delivery envelope. It has parameters and data components.

MessageBodyPart ::= SEQUENCE {

parameters			MessageParameters,

data					MessageData }

MessageParameters ::= SET {

deliveryÑtime				[0]		MessageDeliveryTime OPTIONAL,

deliveryÑenvelope			[1]		OtherMessageDeliveryFields OPTIONAL }

MessageData ::= IPM

The parameters component comprises the following parameters:

a)	deliveryÑtime (O): The date and time the IPM was delivered. The presence of this component in the absence of the deliveryÑenvelope component is discouraged.

b)	deliveryÑenvelope (O): The IPM's other message delivery fields. The presence of this component in the absence of the deliveryÑtime component is discouraged.

The data component is the IPM.

Including one IPM in another as described in the present clause is called forwarding that IPM. The enclosing IPM is called the forwarding IPM, the enclosed IPM the forwarded IPM.

Note 1 Ñ The possible future inclusion of a message identifier in the parameters component is for further study. Its present omission provides compatibility with Recommendation X.420 (1984).

Note 2 Ñ That the IPM and purported delivery envelope of a message body part are, in any sense, genuine is unverified.

### 7.3.9	xe ""§MixedÑmode

A mixedÑmode body part represents a finalÑform document of the sort that is processable by mixedÑmode Teletex terminals and Group 4 Classes 2 and 3 facsimile terminals. It comprises a sequence of protocol elements which describe the document's layout structure.

MixedModeBodyPart ::= SEQUENCE OF ProtocolElement

### 7.3.10	xe ""§Bilaterally defined

A bilaterally defined body part represents an information object whose semantics and abstract syntax are bilaterally agreed by the IPM's originator and all of its potential recipients. It comprises an OctetString.

BilaterallyDefinedBodyPart ::= OCTET STRING

Note Ñ The use of this body parts is discouraged. It predates the externally defined body part type and is retained for backward compatibility with Recommendation X.420 (1984). The externally defined body part type provides the same capabilities and more, its use is preferred, e.g., because such use clearly distinguishes between the body parts defined by one community of users and those defined by another.

### 7.3.11	xe ""§Nationally defined

A nationally defined body part represents an information object whose semantics and abstract syntax are nationally defined by a country whose identity is bilaterally agreed by the IPM's originator and all of its potential recipients. It comprises an Any.

NationallyDefinedBodyPart ::= ANY

Note 1 Ñ This body part is intended for use in domestic communication where the country in question is implicitly that of the originator and all of the potential recipients.

Note 2 Ñ The use of this body part type is discouraged. It predates the externally defined body part type and is retained for backward compatibility with Recommendation X.420 (1984). The externally defined body part type provides the same capabilities and more, and its use is preferred, e.g., because such use clearly distinguishes between the body parts defined by one country and those defined by another.

### 7.3.12	xe ""§Externally defined

An externally defined body part represents an information object whose semantics and abstract syntax are denoted by an object identifier which the body part carries. It has parameters and data components.

ExternalDefinedBodyPart ::= SEQUENCE {

parameters				[0] ExternallyDefinedParameters OPTIONAL

data						ExternallyDefinedData }

ExternallyDefinedParameters ::= EXTERNAL

ExternallyDefinedData ::= EXTERNAL

The parameters and data components are externals (see ¤ 32 of Recommendation X.208). Their directÑreference components shall be present, their indirectÑreference and dataÑvalueÑdescriptor components absent.

On the basis of the externally defined body part type, all body part types are divided into two important classes as follows:

a)	basic: Said of any body part type except externally defined. Denoted by an integer (an ASN.1 contextÑspecific tag).

All basic body part types are defined in this Recommendation.

b)	extended: Some of the externally defined body part type restricted to any one value of the directÑreference component of the data component of such a body part. Denoted by an object identifier.

Some (but not necessarily all) extended body part types are defined in Annex B of this Recommendation.

Every extended body part type this Recommendation defines is defined by means of the following macro. Every extended body part type defined elsewhere shall be so defined as well.

EXTENDEDÑBODYÑPARTÑTYPE MACRO ::=

BEGIN

TYPE			NOTATION		::= Parameters Data

VALUE		NOTATION		::= value (VALUE OBJECT IDENTIFIER)

Paramaters						::= ÒPARAMETERSÓ type ÒIDENTIFIEDÓ ÒBYÓ value (OBJECT IDENTIFIER) | empty

Data									::= ÒDATAÓ type

END

An instance of the macro's type notation defines, by means of its PARAMETERS clause, the type of the data value that is represented by the parameters component of such an (externally defined) body part (an external), and the object identifier that appears in the directÑreference component of this parameters component. The omission of the parameters component is implied by the omission of this clause. An instance of the type notation also defines, by means of its DATA clause, the type of the data value that is represented by the data component of such a body part (an external).

An instance of the macro's value notation defines the object identifier that appears as the directÑreference component of the data component of such an (externally defined) body part. The object identifier identifies the encoding rules for the body part. Those body parts whose types this Recommendation defines shall be encoded using ASN.1 basic encoding rules.

Note 1 Ñ This body part enables the exchange of information objects of all kinds, each unambiguously and uniquely identified. This identification relies upon the directÑreference component mentioned above, which is an object identifier. object identifiers are easily obtained, e.g., by national bodies and private organizations.

Note 2 Ñ If an externally defined body part has a parameters component, the object identifier in its directÑreference component is allocated at the same time and by the same naming authority as that in the directÑreference component of the data component.

Note 3 Ñ Like body parts of other types, an externally defined body part may be subjected to conversion. However, specification of the conversion algorithms may be outside the scope of Recommendation X.408.

Note 4 Ñ The basic body part exists for purely historical reasons, predating the externally defined body part type.

# 8	xe ""§Interpersonal notifications

An interpersonal notification (IPN) is a member of a secondary class of information object conveyed between users in interpersonal messaging.

IPN ::= SET {

ÑÑ commonÑfields ÑÑ COMPONENTS OF CommonFields,

choice [0] CHOICE {

nonÑreceiptÑfields					[0] NonReceiptFields,

receiptÑfields						[1] ReceiptFields } }

An IPN may take either of the following forms:

a)	nonÑreceipt notification (NRN): An IPN that reports its originator's failure to receive, to accept, or his delay in receiving, an IPM.

NRN ::= IPN ÑÑ with nonÑreceiptÑfields chosen

b)	receipt notification (RN): An IPN that reports its originator's receipt, or his expected and arranged future receipt, of an IPM.

RN ::= IPN ÑÑ with receiptÑfields chosen

The IPM to which an IPN refers is called the subject IPM. Only a UA to which the subject IPM is actually delivered shall originate an IPN relating to it, and it shall originate at most one such IPN which shall be conveyed to the subject IPM's originator alone.

An actual recipient shall originate an IPN only in accordance with the notificationÑrequests component of the subject recipient specifier. The subject recipient specifier is that recipient specifier in the subject IPM's heading as a result of which the subject IPM is delivered to that user.

The subject recipient specifier is determined by examining the sequence of recipient specifiers that constitute the subject IPM's primary, copy and blind copy recipients heading fields. The fields are examined in the order in which they are mentioned in the preceding sentence. Within each field, the specifiers are examined in the order in which they appear there. The subject recipient specifier is the first one found whose recipient component has as its value an O/R descriptor whose formalÑname component is present and has as its value either an O/R name of the preferred recipient as a result of which the subject IPM was delivered to the user on whose behalf the examination is performed or, if the IPM reached the user because of his membership in a DL, an O/R name appearing in the message's DL expansion history (see ¤ 8.3.1.1.1.7 of Recommendation X.411).

An IPN comprises a set of information items called notification fields (or fields), each of which is of one of the following classes:

a)	common field: A notification field applicable to both NRNs and RNs.

b)	nonÑreceipt field: A notification field applicable to NRNs alone.

c)	receipt field: A notification field applicable to RNs alone.

The structure of an IPN is depicted in Figure 2/X.420.

The fields, in each of the above classes, that may appear in an IPN are defined and described below.

## 8.1	Common fields

The common fields are defined and described below.

CommonFields ::= SET {

subjectÑipm						SubjectIPMField,

ipnÑoriginator					[1]	IPNOriginatorField OPTIONAL,

ipmÑpreferredÑrecipient	[2]	IPMPreferredRecipientField OPTIONAL,

conversionÑeits					ConversionEITsField OPTIONAL }

### 8.1.1	Subject IPM

The subject IPM common field (M) identifies the subject IPM. It comprises an IPM identifier.

SubjectIPMField ::= IPMIdentifier

### 8.1.2	IPN originator

The IPN originator common field (O) identifies the IPN's originator. It comprises an O/R descriptor.

IPNOriginatorField ::= ORDescriptor

If the IPN's originator is a preferred recipient of the subject IPM, the O/R descriptor above shall be precisely that which is the value of the recipient component of the subject recipient specifier.

Fig. 2/X.420/T0705281-89 8cm

### 8.1.3	IPM preferred recipient

the IPM preferred recipient common field (C) identifies the preferred recipient of the subject IPM who gives rise to its delivery to the IPN's originator (an alternate, (DL) member, or substitute recipient). It comprises an O/R descriptor.

IPMPreferredRecipientField ::= ORDescriptor

The O/R descriptor above shall be precisely that which is the value of the recipient component of the subject recipient specifier.

This conditional field shall be present if, and only if, it would identify a user other than the IPN's originator or a DL.

### 8.1.4	Conversion EITs

The conversion EITs common field (C) identifies the EITs of the subject IPM upon delivery to the IPN's originator. It comprises an EITs descriptor.

ConversionEITsField ::= EncodedInformationTypes

This conditional field shall be present if, and only if, the IPM was subjected to conversion for delivery to the IPN's originator.

## 8.2	NonÑreceipt fields

The nonÑreceipt fields are defined and described below.

NonReceiptFields ::= SET {

nonÑreceiptÑreason			[0]	NonReceiptReasonField,

discardÑreason					[1]	DiscardReasonField OPTIONAL,

autoÑforwardÑcomment		[2]	AutoForwardCommentField OPTIONAL,

returnedÑipm					[3]	ReturnedIPMField OPTIONAL }

### 8.2.1	NonÑreceipt reason

The nonÑreceipt reason nonÑreceipt field (M) indicates why the NRN's originator has not received the subject IPM (even though it was delivered to him).

NonReceiptReasonField ::= ENUMERATED {

ipmÑdiscarded				(0),

ipmÑautoÑforwarded		(1) }

This field may assume one of the following values:

a)	ipmÑdiscarded: The IPM was discarded. This case is further illumined by the discard reason field.

b)	ipmÑautoÑforwarded: The IPM was autoÑforwarded. This case is further illumined by the autoÑforward comment field.

### 8.2.2	Discard reason

The discard reason nonÑreceipt field (C) indicates why the subject IPM was discarded (subsequent to its delivery to the NRN's originator and prior to its receipt).

DiscardReasonField ::= ENUMERATED {

ipmÑexpired							(0),

ipmÑobsoleted							(1),

userÑsubscriptionÑterminated		(2) }

This field may assume any one of the following values:

a)	ipmÑexpired: autoÑdiscard was in effect, expired IPMs were being discarded, and the time identified by the subject IPM's expiry time heading field had arrived.

b)	ipmÑobsoleted: autoÑdiscard was in effect, obsolete IPMs were being discarded, and the obsoleted IPMs heading field of another IPM, delivered to the NRN's originator, identified the subject IPM.

c)	userÑsubscriptionÑterminated: The interpersonal messaging subscription of the NRN's originator was terminated.

This conditional field shall be present only if the nonÑreceipt reason field has the value ipmÑdiscarded.

### 8.2.3	AutoÑforward comment

The autoÑforward comment nonÑreceipt field (C) is information preÑsupplied for this purpose by the NRN's originator. It comprises a printable string of from zero to a prescribed number of characters (see Annex K), chosen from the printable string character set. A length of zero is discouraged.

AutoForwardCommentField ::= AutoForwardComment

AutoForwardComment ::= PrintableString

(SIZE (0. .ubÑautoÑforwardÑcomment))

The value of this field shall be precisely the autoÑforwardÑcomment argument of the change autoÑforwarding abstract operation as a result of which the subject IPM was autoÑforwarded.

This conditional field shall be present if, and only if, the nonÑreceipt reason field has the value ipmÑautoÑforwarded and the autoÑforwardÑcomment argument above was supplied.

### 8.2.4	Returned IPM

The returned IPM nonÑreceipt field (C) is precisely the subject IPM.

ReturnedIPMField ::= IPM

This conditional field shall be present if, and only if, ipmÑreturn is among the values of the notificationÑrequests component of the subject recipient specifier and the subject IPM was not subjected to conversion for delivery to the NRN's originator.

## 8.3	Receipt fields

The receipt fields are defined and described below.

ReceiptFields ::= SET {

receiptÑtime					[0]	ReceiptTimeField,

acknowledgmentÑmode		[1]	AcknowledgmentModeField DEFAULT manual,

supplÑreceiptÑinfo				[2]	SupplReceiptInfoField DEFAULT ÒÓ }

### 8.3.1	Receipt time

The receipt time receipt field (M) identifies when the RN's originator received the subject IPM. It comprises a date and time.

ReceiptTimeField ::= Time

### 8.3.2	Acknowledgment mode

The acknowledgment mode receipt field (D manual) identifies the manner in which the RN was originated.

AcknowledgmentModeField ::= ENUMERATED {

manual			(0),

automatic		(1) }

This field may assume any one of the following values:

a)	manual: The RN was originated by means of the originate RN abstract operation.

b)	automatic: The RN was originated as a result of autoÑacknowledgment.

### 8.3.3	Suppl receipt info

The suppl receipt info receipt field (O) gives supplementary information about the receipt of the subject IPM by the RN's originator. It comprises a printable string of from zero to a prescribed number of characters (see Recommendation X.411), chosen from the printable string character set.

SupplReceiptInfoField ::= SupplementaryInformation

SECTION 3 Ñ ABSTRACT SERVICE DEFINITIONxe "ABSTRACT SERVICE DEFINITION"§

# 9	Overview

This section defines the abstract service that characterizes interpersonal messaging, and describes the environment in which service is supplied and consumed. It does both using the abstract service definition conventions of Recommendation X.407.

This section covers the following topics:

a)	primary object types,

b)	primary port types,

c)	abstract operations,

d)	abstract errors,

e)	other capabilities.

# 10	xe ""§Primary object types

The environment in which interpersonal messaging takes place can be modelled as an abstract object which is hereafter referred to as the interpersonal messaging environment (IPME).

ipme OBJECT

::= idÑotÑipme

When refined (i.e., functionally decomposed), the IPME can be seen to comprise lesser objects which interact by means of ports.

ipmeÑrefinement REFINE ipme AS

ipms

origination			[S] PAIRED WITH ipmsÑuser

reception			[S] PAIRED WITH ipmsÑuser

management		[S] PAIRED WITH ipmsÑuser

ipmsÑuserÑRECURRING

::= idÑrefÑprimary

The lesser objects are referred to as the primary objects of interpersonal messaging. They include a single, central object, the interpersonal messaging system (IPMS), and numerous peripheral objects called interpersonal messaging system users (IPMS users).

The structure of the IPME is depicted in Figure 3/X.420.

The primary object types are defined and described below. The types of ports by means of which they interact are discussed in ¤ 11.

Fig. 3/X.420/T0705290-88 6cm

## 10.1	xe ""§Interpersonal messaging system user

An interpersonal messaging system user (IPMS user) is a user that engaged in interpersonal messaging. An IPMS user originates, receives, or both originates and receives information objects of the types defined in ¤ 2.

ipmsÑuser OBJECT

PORTS {

origination		[C],

reception			[C],

management		[C] }

::= idÑotÑipmsÑuser

The IPME comprises any number of of IPMS users.

Note 1 Ñ As its name suggests, interpersonal messaging is typically an activity of people. Often, therefore, this Recommendation uses personal pronouns (e.g., ÒheÓ) to refer to IPMS users. This practice, however, is not intended to preclude other, atypical uses of interpersonal messaging in which IPMS users are not people.

Note 2 Ñ For brevity, the term ÒuserÓ is used throughout the rest of this Recommendation with the meaning of ÒIPMS userÓ.

## 10.2	xe ""§Interpersonal messaging system

The interpersonal messaging system (IPMS) is the object by means of which all users communicate with one another in interpersonal messaging.

ipms OBJECT

PORTS {

origination		[S],

reception			[S],

management		[S] }

::= idÑotÑipms

The IPME comprises exactly one IPMS.

# 11	xe ""§Primary port types

The primary objects of interpersonal messaging are joined to and interact with one another by means of ports. These ports, which the IPMS supplies, are referred to as the primary ports of interpersonal messaging. They are of the three types defined below.

Note Ñ In ¤ 16 to follow, the IPMS is decomposed into still lesser objects, among which is the MTS. This fact is anticipated in the present paragraph by the inclusion of certain MTS capabilities in the IPMS abstract service.

## 11.1	Origination

An origination port is the means by which a single user conveys to the IPMS messages containing information objects of the types defined in section two. Through such a port the user originates interpersonal messages and receipt notifications. In addition, the user may originate probes through such a port.

The IPMS supplies one origination port to each user (with the exception of indirect users served by PDAUs; see ¤ 16.5).

## 11.2	Reception

A reception port is the means by which the IPMS conveys to a single user messages containing information objects of the types defined in section two. Through such a port the user receives interpersonal messages and interpersonal notifications. In addition, the user may receive reports through such a port.

The IPMS supplies one reception port to each user.

## 11.3	Management

A management port is the means by which a single user changes information about himself on file with the IPMS. By means of such a port the user enables and disables autoÑdiscard, Ñacknowledgment, and Ñforwarding.

The IPMS supplies one management to each user (with the exception of indirect users served by PDAUs; see ¤ 16.5).

# 12	Abstract operations

Thexe ""§ IPMS abstract service is the set of capabilities that the IPMS provides to each user by means of one origination, one reception, and one management port. Those capabilities are modelled as abstract operations, which may encounter abstract errors when invoked.

The abstract operations available at origination, reception, and management ports, respectively, are defined and described below. The abstract errors they may provoke are the subject of ¤ 13.

Note 1 Ñ The IPMS abstract service involves neither abstract bind nor abstract unbind operations.

Note 2 Ñ The IPMS authenticates (i.e., establishes the identity of) the typical user before offering the IPMS abstract service to him. By this means it can verify, for example, that the user is an IPMS subscriber. Authentication, where required, is implicit (rather than explicit) in the definition of the IPMS abstract service.

Note 3 Ñ The purpose of the IPMS abstract service definition is not to prescribe the user interfaces of implementations of portions of the IPMS, but rather to clarify the meaning and intended use of the information objects of section two. A user interface need not provide commands in oneÑtoÑone correspondence to the service's abstract operations, nor indeed even divide the labor between the user and the IPMS as the service does.

Note 4 Ñ In ¤ 16 to follow, the IPMS is decomposed into objects among which is the MTS. The present paragraph reflects this fact by its inclusion of various MTSÑdefined information items in the IPMS abstract service.

## 12.1	Origination abstract operations

The abstract operations available at an origination port are invoked by the user and performed by the IPMS.

origination PORT

CONSUMER INVOKES {

OriginateProbe,

OriginateIPM,

OriginateRN }

::= idÑptÑorigination

### 12.1.1	Originate probe

The originate probe abstract operation originates a probe concerning (a class of) messages whose contents are IPMs.

OriginateProbe ::= ABSTRACTÑOPERATION

ARGUMENT SET {

envelope			[0] ProbeSubmissionEnvelope,

content			[1] IPM }

RESULT SET {

submissionÑidentifier			[0] ProbeSubmissionIdentifier,

submissionÑtime						[1] ProbeSubmissionTime }

ERRORS {

SubscriptionError,

RecipientImproperlySpecified }

This abstract operation has the following arguments:

a)	envelope (M): A probe submission envelope, whose makeup the MTS abstract service defines. The UA supplies all but the following envelope components, which the user provides:

i)	The desired perÑmessage options (i.e., perÑmessage indicators and extensions).

ii)	The O/R names of the preferred recipients and the perÑrecipient options (i.e., originator report request, explicit conversion, and extensions) desired for each.

b)	content (M): An instance of the class of IPM whose deliverability is to be probed.

This abstract operation has the following results:

1)	submissionÑidentifier (M): The probe submission identifier the MTS assigns to probe.

2)	submissionÑtime (M): The date and time the probe was directly submitted.

### 12.1.2	xe ""§Originate IPM

The originate IPM abstract operation originates a message whose content is an IPM.

OriginateIPM ::= ABSTRACTÑOPERATION

ARGUMENT SET {

envelope			[0] MessageSubmissionEnvelope,

content			[1] IPM }

RESULT SET {

submissionÑidentifier		[0] MessageSubmissionIdentifier,

submissionÑtime			[1] MessageSubmissionTime }

ERRORS {

SubscriptionError,

RecipientImproperlySpecified }

This abstract operation has the following arguments:

a)	envelope (M): A message submission envelope, whose makeup the MTS abstract service defines. The UA supplies all but the following envelope components, which the user provides:

i)	The desired perÑmessage options (i.e., priority, perÑmessage indicators, deferred delivery time, and extensions).

ii)	The O/R names of the preferred recipients and the perÑrecipient options (i.e., originator report request, explicit conversion, and extensions) desired for each.

b)	content (M): The IPM being originated. Its autoÑforwarded heading field shall be absent or have the value false.

This abstract operation has the following results:

1)	submissionÑidentifier (M): The message submission identifier the MTS assigns to the submission.

2)	submissionÑtime (M): The date and time the message was directly submitted.

### 12.1.3	xe ""§Originate RN

The originate RN abstract operation originates a message whose content is RN.

OriginateRN ::= ABSTRACTÑOPERATION

ARGUMENT SET {

envelope			[0] MessageSubmissionEnvelope,

content			[1] RN }

RESULT SET {

submissionÑidentifier		[0] MessageSubmissionIdentifier,

submissionÑtime				[1] MessageSubmissionTime }

ERRORS {

SubscriptionError,

RecipientImproperlySpecified }

An RN shall be originated only by an actual recipient of the subject IPM of whom an RN is requested by means of the notificationÑrequests component of the subject IPM's subject recipient specifier.

The user shall not have previously originated an RN in response to the subject IPM, by means of either the present abstract operation or autoÑacknowledgment.

This abstract operation has the following arguments:

a)	envelope (M): A message submission envelope, whose makeup the MTS abstract service defines. The UA supplies all but the following envelope components, which the user provides:

i)	The desired perÑmessage options (i.e., priority, perÑmessage indicators, and extensions). Implicit conversion shall be prohibited, priority that of the subject IPM.

ii)	The O/R names of the preferred recipients and the perÑrecipient options (i.e., explicit conversion and extensions) desired for each. Reports shall not be requested.

b)	content (M): The RN being originated.

This abstract operation has the following results:

1)	submissionÑidentifier (M): The message submission identifier the MTS assigns to the submission.

2)	submissionÑtime (M): The date and time the message was directly submitted.

## 12.2	Reception abstract operatons

The abstract operations available at a reception port are invoked by the IPMS and performed by the user.

reception PORT

SUPPLIER INVOKES {

ReceiveReport,

ReceiveIPM,

ReceiveRN,

ReceiveNRN }

::= idÑptÑreception

Note 1 Ñ As abstractly defined, the IPMS provides no storage for received messages because whether or not it does so for a particular user has no impact upon that user's ability to communicate with other users. Thus the provision of storage is a local matter.

Note 2 Ñ Elaborating upon the above, the receive IPM abstract operation, e.g. expels an IPM from the IPMS because its purpose is to clarify the meaning of the receipt transmittal step. In contrast, the capabilities of a user to whom storage for received messages is provided might include a ÒDisplay IPMÓ command that enables the user to view the delivered (and perhaps already received) IPM whose IPM identifier he specifies, and that allows him to do so any number of times by repeatedly invoking the command. The first, but not subsequent uses of the command to view a particular IPM represents the concrete realization of the receive IPM abstract operation in such an implementation.

### 12.2.1	Receive report

The receive report abstract operation receives a report.

ReceiveReport ::= ABSTRACTÑOPERATION

ARGUMENT SET {

envelope					[0] ReportDeliveryEnvelope,

undeliveredÑobject		[1] InformationObject OPTIONAL }

RESULT

ERRORS { }

The report received may concern any of the following previously originated by the report's recipients:

a)	A probe concerning a message whose content was an IPM that was originated with the originate probe abstract operation.

b)	A message whose content was an NRN originated as a result of autoÑdiscard or autoÑforward.

c)	A message whose content was an RN that was originated with the originate RN abstract operations by autoÑacknowledgment.

d)	A message whose content was an IMP that was originated with the originate IPM abstract operation or by autoÑforwarding.

This abstract operation has the following arguments:

1)	envelope (M): A report delivery envelope, whose makeup the MTS abstract service defines.

2)	undeliveredÑobject (C): The content of the message whose status is being reported. An IPM or IPN.

If the report was provoked by a previous originate probe abstract operation invocation, this conditional agreement shall be absent. If the report was provoked by a previous originate IPM abstract operation invocation, the argument shall be present if, and only if, content return was requestd. Otherwise (i.e., if the report was provoked by an IPN), the argument shall be absent.

This abstract operation has no results.

### 12.2.2	Receive IPM

The receive IPM abstract operation receives a message whose content is an IPM.

ReceiveIPM ::= ABSTRACTÑOPERATION

ARGUMENT SET {

envelope			[0] MessageDeliveryEnvelope,

content			[1] IPM }

RESULT

ERRORS { }

This abstract operation has the following arguments:

a)	envelope (M): The message's delivery envelope.

b)	content (M): The IPM that is the message's content.

This abstract operation has no results.

### 12.2.3	Receive RN

The receive RN abstract operation receives a message whose content is an RN. The RN is provoked by an IPM originated with the originate IPM abstract operation.

ReceiveRN ::= ABSTRACTÑOPERATION

ARGUMENT SET {

envelope			[0] MessageDeliveryEnvelope,

content			[1] RN }

RESULT

ERRORS { }

This abstract operation has the following arguments:

a)	envelope (M): The message's delivery envelope.

b)	content (M): The RN that is the message's content.

This abstract operation has no results.

### 12.2.4	Receive NRN

The receive NRN abstract operation receives a message whose content is an NRN. The NRN is provoked by an IPM originated with the originate IPM abstract operation.

ReceiveNRN ::= ABSTRACTÑOPERATION

ARGUMENT SET {

envelope			[0] MessageDeliveryEnvelope,

content			[1] NRN }

RESULT

ERRORS { }

This abstract operation has the following arguments:

a)	envelope (M): The message's delivery envelope.

b)	content (M): The NRN that is the message's content.

This abstract operation has no results.

## 12.3	Management abstract operations

The abstract operations available at a management port are invoked by the user and performed by the IPMS.

management PORT

CONSUMER INVOKES {

ChangeAutoDiscard,

ChangeAutoAcknowledgment,

ChangeAutoForwarding }

::= idÑptÑmanagement

### 12.3.1	xe ""§Change autoÑdiscard

The change autoÑdiscard abstract operation enables or disables autoÑdiscard, the automatic discard by the IPMS of expired or obsolete IPMs delivered to, but not yet received by the user.

ChangeAutoDiscard; ::= ABSTRACTÑOPERATION

ARGUMENT SET {

autoÑdiscardÑexpiredÑIPMs		[0] BOOLEAN,

autoÑdiscardÑobsoleteÑIPMs		[1] BOOLEAN }

RESULT

ERRORS { }

When it autoÑdiscards an IPM, the IPMS originates an NRN on the user's behalf if, and only if, one was requested of him by means of the notificationÑrequests component of the subject recipient specifier.

This abstract operation has the following arguments:

a)	autoÑdiscardÑexpiredÑIPMs (M): Whether or not expired IPMs are to be autoÑdiscarded. A Boolean.

b)	autoÑdiscardÑobsoleteÑIPMs (M): Whether or not obsolete IPMs are to be autoÑdiscarded. A Boolean.

This abstract operation has no results.

### 12.3.2	xe ""§Change autoÑacknowledgment

The change autoÑacknowledgment abstract operation enables or disables autoÑacknowledgment, the automatic origination of RNs by the IPMS on the user's behalf. Such origination occurs upon delivery of IPMs that request RNs of the user by means of the notificationÑrequests components of their subject recipient specifiers.

ChangeAutoAcknowledgment ::= ABSTRACTÑOPERATION

ARGUMENT SET {

autoÑacknowledgeÑIPMs						[0] BOOLEAN,

autoÑacknowledgeÑsupplÑreceiptÑinfo		[1]

SupplementaryInformation OPTIONAL }

RESULT

ERRORS {

SubscriptionError }

This abstract operation has the following arguments:

a)	autoÑacknowledgeÑIPMs (M): Whether or not IPMs are to be autoÑacknowledged. A Boolean.

b)	autoÑacknowledgeÑsupplÑreceiptÑinfo (C): The suppl receipt info receipt field of each RN provoked by autoÑacknowledgment.

This conditional argument shall be present if, and only if, the autoÑacknowledgeÑIPMs argument has the value true.

This abstract operation has no results.

### 12.3.3	xe ""§Change autoÑforwarding

The change autoÑforwarding abstract operation enables or disables autoÑforwarding, the automatic forwarding of IPMs by the IPMS to preÑspecified users or DLs. Such forwarding occurs upon delivery of the IPMs.

ChangeAutoForwarding ::= ABSTRACTÑOPERATION

ARGUMENT SET {

autoÑforwardÑIPMs				[0] BOOLEAN,

autoÑforwardÑrecipients		[1] SEQUENCE OF ORName OPTIONAL,

autoÑforwardÑheading			[2] Heading OPTIONAL,

autoÑforwardÑcomment			[3] AutoForwardComment OPTIONAL }

RESULT

ERRORS {

SubscriptionError,

RecipientImproperlySpecified }

The body of each IPM the IPMS originates as a result of autoÑforwarding comprises a single body part of type message. The content of the message represented by that body part is the forwarded IPM.

When it autoÑforwards an IPMS, the IPMS originates an NRN on the user's behalf if, and only if, one was requested of him by means of the notificationÑrequests component of the subject recipient specifier.

This abstract operation has the following arguments:

a)	autoÑforwardÑIPMs (M): Whether or not IPMs are to be autoÑforwarded. A Boolean.

b)	autoÑforwardÑrecipients (C): The users or DLs to which IPMs are to be autoÑforwarded. A sequence of O/R names.

This conditional argument shall be present if, and only if, the autoÑforwardÑIPMs argument has the value true.

c)	autoÑforwardÑheading (C): The heading that is to be used for each forwarding IPM. Its autoÑforwarded heading field shall have the value true.

This conditional argument shall be present if, and only if, the autoÑforwardÑIPMs argument has the value true.

d)	autoÑforwardÑcomment (C): The value that is to be supplied as the autoÑforward comment nonÑreceipt field of each NRN conveyed to the originator of an autoÑforwarded IPM.

The conditional argument shall be present if, and only if, the autoÑforwardÑIPMs argument has the value true.

This abstract operation has no results.

Note Ñ This abstract operation is intended to define the essence of autoÑforwarding, sophisticated autoÑforwarding capabilities, e.g., like those of an MS.

# 13	xe ""§Abstract errors

The abstract errors that may be reported in response to the invocation of the abstract operations available at origination, reception, and management ports are defined and described below or as part of the MTS abstract service definition.

Note Ñ The set of abstract errors represented below is intended to be illustrative, rather than exhaustive.

## 13.1	Subscription error

The subscription error abstract error reports that the user has not subscribed to one or more of the elements of service implicit in his invocation of the abstract operation whose performance is aborted.

SubscriptionError ::= ABSTRACTÑERROR

PARAMETER SET {

problem [0] SubscriptionProblem }

This abstract error has the following parameters:

a)	problem (M): The subscriptionÑrelated problem encountered.

SubscriptionProblem ::= ENUMERATED {

ipmsÑeosÑnotÑsubscribed(0),

mtsÑeosÑnotÑsubscribed (1) }

This parameter may assume any one of the following values:

i)	IPMSÑeosÑnotÑsubscribed: An IPMS element of service is not subscribed.

ii)	MTSÑeosÑnotÑsubscribed: An MTS element of service is not subscribed.

## 13.2	Recipient improperly specified

The recipient improperly specified abstract error reports that one or more of the O/R names supplied as arguments of the abstract operation whose performance is aborted, or as components of its arguments, are invalid.

This abstract error is defined by the MTS abstract service.

# 14	Other capabilities

In addition to the capabilities embodied in the IMPS abstract service, defined above, the IPMS shall transparently extend to each use the other MS and MTS capabilities identified below. (The enumeration of these capabilities necessarily anticipates the fact, stated in ¤ 16, that MSs and the MTS are among the IPMS' component parts.)

The following additional capabilities shall be provided:

a)	Submission: Capabilities of the MS' or MTS' submission port not embodied in the IPMS abstract service e.g., the ability to cancel delivery of a previously originated message whose content is an IPM (but not an RN), if deferred delivery was selected.

b)	Delivery: Capabilities of the MTS' delivery port not embodied in the IPMS abstract service, e.g., the ability to temporarily control the kinds of information objects the MTS conveys to the user's UA.

c)	Administration: The capabilities of the MS's or MTS's administration port.

d)	Retrieval: The capabilities of the MS' retrieval port.

In addition to the above and as a local matter, the IPMS may provide to users additional capabilities neither defined nor limited by this Recommendation. Among such capabilities are those of the directory.

Note Ñ The required capabilities of this clause are excluded from the formal definition of the IPMS abstract service for purely pragmatic reasons, in particular, because their inclusion would largely and needlessly reproduce the definitions of the MS and MTS abstract operations upon which the capabilities are based.

SECTION 4 Ñ ABSTRACT SERVICE PROVISIONxe "ABSTRACT SERVICE PROVISION"§

# 15	Overview

This section specifies how the IPMS provides the IPMS abstract service to users.

This section covers the following topics:

a)	secondary object types,

b)	secondary port types,

c)	user agent operation,

d)	message store operation,

e)	message contents,

f)	port realization,

g)	conformance.

# 16	xe ""§Secondary object types

The IPMS can be modeled as comprising lesser objects which interact with one another by means of (additional) ports.

ipmsÑrefinement REFINE ipms AS

mTS

submission		[S]	PAIRED WITH ipmsÑua, ipmsÑms

delivery			[S]	PAIRED WITH ipmsÑua, ipmsÑms

administration	[S]	PAIRED WITH ipmsÑua, ipmsÑms

ipmsÑua RECURRING

origination		[S]	VISIBLE

reception			[S]	VISIBLE

management		[S]	VISIBLE

ipmsÑms RECURRING

submission		[S]	PAIRED WITH ipmsÑua

retrieval			[S]	PAIRED WITH ipmsÑua

administration	[S]	PAIRED WITH ipmsÑua

tlma RECURRING

origination		[S]	VISIBLE

reception			[S]	VISIBLE

management		[S]	VISIBLE

tlxau RECURRING

origination		[S]	VISIBLE

reception			[S]	VISIBLE

management		[S]	VISIBLE

pdau RECURRING

reception			[S]	VISIBLE

::= idÑrefÑsecondary

These lesser objects are referred to as the secondary objects of interpersonal messaging. They include a single, central object, the MTS, and numerous peripheral objects: interpersonal messaging system user agents (IPMS UAs), interpersonal messaging system message stores (IPMS MSs), telematic agents (TLMAs), telex access units (TLXAUs), and PDAUs.

The structure of the IPMS is depicted in Figure 4/X.420. As shown by Figure 4/X.420, IPMS UAs, TLMAs, TLXAUs, and PDAUs are the instruments by means of which the IPMS provides the IPMS abstract service to users.

The secondary object types are defined and described below. The types of ports by means of which they interact are discussed in ¤ 17.

Note 1 Ñ The refinement above encompasses all possible interconnection of all possible objects. It ignores the possible absence of objects of a particular type (e.g., PDAU), and specific logical configurations of the IPMS MS. The latter are identified in Recommendation X.402.

Note 2 Ñ Recommendation T.330 effectively extends the abstract service of Interpersonal Messaging by its definition of a miscellanea port, which is not shown in the figure. See the note in ¤ 16.3.

Note 3 Ñ The MTS supplies import and export ports. However, since those ports are not formally defined (in Recommedation X.411), they are not included in the formal refinement above.

## 16.1	xe ""§Interpersonal messaging system user agent

An interpersonal messaging system user agent (IPMS UA) is a UA tailored so as to better assist a single user to engage in interpersonal messaging. It helps him originate, receive, or both originate and receive messages containing information objects of the types defined in section two.

ipmsÑus OBJECT

PORTS {

origination	[S],

reception		[S],

management	[S],

submission	[C],

delivery		[C],

retrieval		[C],

administration	[C] }

::= idÑotÑipmsÑua

Fig. 4/X.420/T0705300-88 8cm

The IPMS comprises any number of IPMS UAs.

Note Ñ For brevity, the term ÒUAÓ is used throughout the rest of this Recommendation with the meaning of ÒIPMS UAÓ.

## 16.2	Interpersonal messaging system message store

An interpersonal messaging system message store (IPMS MS) is an MS tailored so as to better assist a single UA engage in interpersonal messaging. It helps it submit, take delivery of, or both submit and take delivery of messages containing information objects of the types defined in Section 2.

ipmsÑms OBJECT

PORTS {

submission	[S],

retrieval		[S],

administration	[S],

submission	[C],

delivery		[C],

administration	[C] }

::= idÑotÑipmsÑms

The IPMS comprises any number of IPMS MSs.

Note Ñ For brevity, the term ÒMSÓ is used throughout the rest of this Recommendation with the meaning of ÒIPMS MSÓ.

## 16.3	xe ""§Telematic agent

A telematic agent (TLMA) is an AU that helps a single indirect user engage in interpersonal messaging from a telematic terminal, along with that terminal and the network that joins the two. A TLMA helps the user originate, receive, or both originate and receive messages containing information objects of the types defined in Section 2.

tlma OBJECT

PORTS {

origination	[S],

reception		[S],

management	[S],

miscellanea	[S] }

::= idÑotÑtlma

The IPMS comprises any number of TLMAs.

Note 1 ÑA TLMA consumes import and export ports. However, since those ports are not formally defined (in Recommendation X.411), they are not included in the formal definition of TLMA above.

Note 2 Ñ A TLMA's miscellanea port is defined in Recommendation T.330. It is not part of the IPMS abstract service in its most general form, which is the subject of this Recommendation. Rather it embodies capabilities available only to a TLMA user. For this reason, it is not considered further here and is not included in the formal refinement of the IPMS found in ¤ 16.

## 16.4	Telex access unit

A telex access unit (TLXAU) is an AU that helps any number of indirect users engage in interpersonal messaging from telex terminals. It helps them originate, receive, or both originate and receive messages containing information objects of the types defined in section two.

tlxau OBJECT

PORTS {

origination	[S],

reception		[S],

management	[S] }

::= idÑotÑtlxau

The IPMS comprises any number of TLXAUs.

Note Ñ A TLXAU consumes import and export defined (in Recommendation X.411) they are not included in the formal definition of TLXAU above.

## 16.5	xe ""§Physical delivery access unit

In the present context, a PDAU helps any number of indirect users engage in interpersonal messaging by means of a PDS. It helps them receive (but not originate) messages containing information objects of the types defined in section two.

pdau OBJECT

PORTS {

reception		[S] }

::= idÑotÑpdau

The IPMS comprises any number of PDAUs.

Note Ñ A PDAU consumes import and export ports. However, since those ports are not formally defined (in Recommendation X.411), they are not included in the formal definition of PDAU above.

## 16.6	Message transfer system

In the present context, the MTS conveys information objects of the types defined in Section 2 between UAs, MSs, TLMAs, and AUs.

The IPMS comprises a single MTS.

# 17	xe ""§Secondary port types

The secondary objects of interpersonal messaging are joined to and interact with one another by means of ports. These ports, which MSs and the MTS supply, are referred to as the secondary ports of interpersonal messaging. They are of the types identified below.

The capabilities embodied in one submission, one retrieval, and one administration port constitute the MS abstract service. They are defined in Recommendation X.413.

The capabilities embodied in one submission, one delivery, and one administration port constitute the MTS abstract service. They are defined in Recommendation X.411.

Note Ñ By means of the abstract bind operation which guards its ports, an MS or the MTS typically authenticates another secondary object before offering its abstract service to that object.

## 17.1	xe ""§Submission

In the present context, a submission port is the mean by which a UA (directly or indirectly) or an MS (directly) submits probes concerning and messages containing information objects of the types defined in section two.

An MS supplies one submission port to its UA.

The MTS supplies one submission port to each UA configured without an MS and to each MS.

## 17.2	xe ""§Delivery

In the present context, a delivery port is the means by which a UA or MS takes delivery of reports concerning and messages containing information objects of the types defined in Section 2.

The MTS supplies one delivery port each UA configured without an MS and to each MS.

## 17.3	xe ""§Retrieval

In the present context, a retrieval port is the means by which a UA retrieves reports concerning and messages containing information objects of the types defined in Section 2.

An MS supplies one retrieval port to its UA.

## 17.4	xe ""§Administration

In the present context, an administration port is the means by which a UA changes information about itself or its user on file with its MS, or a UA or MS changes such information on file with the MTS.

An MS supplies one administration port to its UA.

The MTS supplies one administration port to each UA configured without an MS and to each MS.

## 17.5	xe ""§Import

In the present context, an import port is the means by which the MTS imports reports concerning and messages containing information objects of the types defined in Section 2.

The MTS supplies one import port to each AU (or TLMA).

## 17.6	xe ""§Export

In the present context, an export port is the means by which the MTS exports probes concerning and messages containing information objects of the types defined in Section 2.

The MTS supplies one export port to each AU (or TLMA).

# 18	xe ""§User agent operation

A UA must employ the MTS in a particular way in order to (correctly) provide the IPMS abstract service to its user. If the user is equipped with an MS, the latter contributes to the provision of the abstract service and, therefore, is subject to the same rules.

The rules that govern the operation of a UA (and MS) are the subject of the present clause. The operation of a TLMA or AU is beyond the scope of this Recommendation.

Note 1 Ñ It is for historical reasons that the Recommendation that defines the IPMS abstract service also specifies how a UA (and MS), but not a TLMA or AU, provides it.

Note 2 Ñ The purpose of this clause is not to dictate or constrain the implementation of a real UA unnecessarily, but rather to clarify the meaning and intended effect of the IPMS abstract service.

## 18.1	State variables

The operation of a UA is described below with the aid of state variables. A state variable is an information item whose value records the results of the UA's past interactions with its user and influences future interactions. State variables are common to (i.e., shared by) the UA's origination, reception, and management ports.

The UA maintains each state variable continuously, i.e., throughout its user's IPMS subscription. Each Boolean state variable is assigned the value false when the subscription commences. The initial values of other state variables are immaterial and therefore unspecified.

The UA alters its state variables when performing or invoking abstract operations. It consults them in determining how to perform, or whether or how to invoke abstract operations. Their values (if any) transcend the binding and unbinding of ports.

Note Ñ State variables are pedagogic devices not intended to constrain the implementation of a real UA unnecessarily. In particular, a UA need not maintin runÑtime data structures corresponding to the state variables if the behaviour required of it can be assured in another way.

## 18.2	Performance of origination operations

A UA shall perform the abstract operations it makes available at its origination port as prescribed below. The UA alters none of its state variables in the performance of these particular operations.

In the performance of these operations, the UA invokes the following abstract operations of the MTS abstract service (which, for the remainder of this paragraph, are unqualified as to their source):

a)	probe submission,

b)	message submission.

Note Ñ In response to the invocation of these abstract operations, a UA reports abstract errors as appropriate. Specification of the precise circumstances under which each abstract error should be reported is beyon the scope of this Recommendation.

### 18.2.1	xe ""§Originate probe

A UA shall perform the originate probe abstract operation by invoking probe submission with the arguments indicated below, and by returning to its user the results indicated below.

The arguments of probe submission shall be as follows:

a)	Envelope: The components of this argument that constitute perÑprobe fields shall be as follows;

i)	OriginatorÑname: The O/R name of the UA's user.

ii)	ContentÑtype, contentÑlength, and originalÑencodedÑinformationÑtypes: Determined from originate probe's content argument as specified in ¤¤ 20.2 to 20.4.

iii)	ContentÑidentifier: Specified or omitted as a local matter.

The components of this argument that constitute perÑrecipient fields shall be as specified by originate probe's envelope argument.

The result of originate probe shall be as follows:

1)	SubmissionÑidentifier: Probe submission's probeÑsubmissionÑidentifier result.

2)	SubmissionÑtime: Probe submission's probeÑsubmissionÑtime result.

Note 1 Ñ The UA shall ignore all properties of originate probe's content argument other than those mentioned above.

Note 2 Ñ How the UA employs probe submission's contentÑidentifier result is a local matter.

### 18.2.2	xe ""§Originate IPM

A UA shall perform the originate IPM abstract operation by invoking message submission with the arguments indicated below, and by returning to its user the results indicated below.

The arguments of message submission shall be as follows:

a)	Envelope: The components of this argument that constitute perÑmessage fields shall be as follows; those not explicitly mentioned below shall be as specified by originate IPM's envelope argument:

i)	OriginatorÑname: The O/R name of the UA's user.

ii)	ContentÑtype and originalÑencodedÑinformationÑtypes: Determined from originate IPM's content argument as specified in ¤¤ 20.4 and 20.4, respectively.

iii)	ContentÑidentifier: Specified or omitted as a local matter.

The components of this argument that constitute perÑrecipient fields shall be as specified by originate IPM's envelope argument.

b)	Content: Determined from originate IPM's content argument (identified as an IPM) as specified in ¤Ê20.1.

If the blind copy recipients heading field of the IPM identifies one or more users and DLS, the UA shall invoke message submission multiple times, upon each occasion varying the heading field so as to comply with the information hiding requirements of ¤ 7.2.6.

The results of originate IPMS shall be as follows:

1)	SubmissionÑidentifier: Message submission's messageÑsubmissionÑidentifier result.

2)	SubmissionÑtime: Message submission's messageÑsubmissionÑtime result.

Note 1 Ñ How the UA employs message submission's contentÑidentifier result is a local matter.

Note 2 Ñ The inclusion of message submission's extensions result among originate IPM's results in proper and for further study.

### 18.2.3	xe ""§Originate RN

A UA shall perform the originate RN abstract operation by invoking message submission with the arguments indicated below, and by returning to its user the results indicated below.

The arguments of message submission shall be as follows:

a)	Envelope: The components of this argument that constitute perÑmessage fields shall be as follows; those not explicitly mentioned below shall be as specified by originate RN's envelope argument:

i)	OriginatorÑname: The O/R name of the UA's user.

ii)	ContentÑtype and originalÑencodedÑinformationÑtypes: Determined from the RN as specified in ¤¤ 20.2 and 20.4, respectively.

iii)	ContentÑidentifier: Specified or omitted as a local matter.

iv)	DeferredÑdeliveryÑtime: Omitted.

The components of this argument that constitute perÑrecipient fields shall be as specified by originate RN's envelope argument.

b)	Content: Determined from originate RN's content argument (identified as an RN) as specified in ¤ 20.1.

The result of originate RN shall be as follows:

1)	SubmissionÑidentifier: Message submission's messageÑsubmissionÑidentifier result.

2)	SubmissionÑtime: Message submission's messageÑsubmissionÑtime result.

Note 1 Ñ How the UA employs message submission's contentÑidentifier result is a local matter.

Note 2 Ñ The inclusion of message submission's extensions result among originate RN's results is proper and for further study.

## 18.3	Performance of management operations

A UA shall perform the abstract operations it makes available at its management port as specified below. The UA alters one or more of its state variables (see below) in the performance of each operation.

Note Ñ In response to the invocation of these abstract operations, a UA reports abstract errors as appropriate. Specification of the precise circumstances under which each abstract error should be repeated is beyond the scope of this Recommendation.

### 18.3.1	xe ""§Change autoÑdiscard

To assist it in providing this abstract operation, a UA maintains the following state variables:

a)	AutoÑdiscardÑexpiredÑIPMs: A Boolean that indicates whether or not autoÑdiscard is in effect for expired IPMs.

b)	AutoÑdiscardÑobsoleteÑIPMs: A Boolean that indicates whether or not autoÑdiscard is in effect for obsolete IPMs.

A UA shall perform the change autoÑdiscard abstract operation by recording the values of the autoÑdiscardÑexpiredÑIPMs and autoÑdiscardÑobsoleteÑIPMs arguments in the correspondingly named state variables.

### 18.3.2	xe ""§Change autoÑacknowledgment

To assist it in providing this abstract operation, a UA maintains the following state variables:

a)	autoÑacknowledgeÑIPMs: A Boolean that indicates whether or not autoÑacknowledgment is in effect.

b)	autoÑacknowledgeÑsupplÑreceiptÑinfo: The suppl receipt info field of each RN provoked by autoÑacknowledgment.

A UA shall perform the change autoÑacknowledgment abstract operation by recording the value of the autoÑacknowledgeÑIPMs argument in the correspondingly named state variable. If the value is true, it also shall record the value of the AutoÑacknowledgeÑsupplÑreceiptÑinfo argument in the correspondingly named state variable.

### 18.3.3	xe ""§Change autoÑforwarding

To assist it in providing this abstract operation, a UA maintains the following state variables:

a)	xe ""§autoÑforwardÑIPMs: A Boolean that indicates whether or not autoÑforwarding is in effect.

b)	xe ""§autoÑforwardÑrecipients: A sequence of O/R names that identify the users and DLs to which IPMs are being autoÑforwarded.

c)	xe ""§autoÑforwardÑheading: The heading of each forwarding IPM provoked by autoÑforwarding. Its autoÑforwarded field has the value true.

d)	xe ""§autoÑforwardÑcomment: The autoÑforward comment nonÑreceipt field of each NRN conveyed to the originator of an autoÑforwarded IPM.

A UA shall perform the change autoÑforwarding abstract operation by recording the value of the autoÑforwardÑIPMs argument in the correspondingly named state variable. If the value is true, it also shall record the values of the autoÑforwardÑrecipients, autoÑforwardÑheading, and autoÑforwardÑcomment arguments in the correspondingly named state variables.

## 18.4	Invocation ofxe ""§ reception operations

A UA shall invoke that abstract operations available at its reception port as specified below. The UA alters none of its state variables in connection with its invocation of these operations.

The UA invokes these operations in response to the MTS' invocation of the following abstract operations of the MTS abstract service (which, for the remainder of this paragraph, are unqualified as to their source):

a)	report delivery,

b)	message delivery.

Note Ñ The abstract operation of a reception port report no errors.

#### 18.4.1	xe ""§Receive report

Whenever the MTS invokes report delivery at a UA's delivery port, the UA shall invoke the receive report abstract operation with the following arguments:

a)	Envelope: Report delivery's envelope argument.

b)	UndeliveredÑobject: Determined from report delivery's returnedÑcontent argument as specified in ¤Ê20.1.

Note Ñ How the UA employs the contentÑidentifier component of report delivery's envelope argument is a local matter.

### 18.4.2	xe ""§Receive IPM

Whenever the MTS invokes message delivery at a UA's delivery port, and its content argument encodes an IPM as specified in ¤ 20.1, the UA shall invoke the receive IPM abstract operation with the following arguments, provided that the message is subject to neither autoÑforwarding nor autoÑdiscard (see ¤ 18.5):

a)	Envelope: Message delivery's envelope argument.

b)	Content: Determined from message delivery's content argument as specified in ¤ 20.1 (but no longer marked as an IPM).

### 18.4.3	xe ""§Receive RN

Whenever the MTS invokes message delivery at a UA's delivery port, and its content argument encodes in RN as specified in ¤ 20.1, the UA shall invoke the receive RN abstract operation with the following arguments:

a)	Envelope: Message delivery's envelope argument.

b)	Content: Determined from message delivery's content argument as specified in ¤ 20.1 (but no longer marked as an RN).

### 18.4.4	xe ""§Receive NRN

Whenever the MTS invokes message delivery at a UA's delivery port, and its content argument encodes an NRN as specified in ¤ 20.1, the UA shall invoke the receive NRN abstract operation with the following arguments:

a)	Envelope: Message delivery's envelope argument.

b)	Content: Determined from message delivery's content argument as specified in ¤ 20.1 (but no longer marked as an NRN).

## 18.5	Internal procedures

A UA shall perform as specified below the internal procedures of autoÑdiscard, Ñacknowledgment, and Ñforwarding in ultimate fulfilment of the abstract operations available at its management port.

The procedures involve the following abstract operations of the MTS abstract service (which, for the remainder of this paragraph, are unqualified as to their source):

a)	message submission,

b)	message delivery.

As implied by the above, in the course of the procedures, the UA has occasion to invoke message submission. What it does with the results of this abstract operation is a local matter.

The UA shall consider as a candidate for each procedure individually every message for which all of the following conditions hold:

a)	The MTS has conveyed the message to the UA by invoking message delivery at the UA's delivery port.

b)	The UA has not conveyed the message to the user by invoking receive IPM at the user's reception port.

c)	The message contains an IPM (rather than an IPN).

Note Ñ With reference to item b) above, the message might be detained in the UA, e.g., as might be typical, because of the user's unavailability.

### 18.5.1	xe ""§AutoÑdiscard

The UA shall subject to autoÑdiscard each candidate message with respect to whose content either of the following conditions holds:

a)	The autoÑdiscardÑexpiresÑIPMs state variable has the value true and the date and time denoted by the IPM's expiry time field have past.

b)	The autoÑdiscardÑobsoleteÑIPMs state variable has the value true and another candidate IPM identifies the present candidate IPM by means of its obsoleted IPMs heading field.

The UA shall autoÑdiscard each such message as follows.

#### 18.5.1.1	xe ""§Discard of IPM

The UA shall discard the IPM, so as to never convey it to the user.

#### 18.5.1.2	xe ""§Construction of NRN

The UA shall construct an NRN if, and only if, one is requested by means of the notificationÑrequests component of the IPM's subject recipient specifier.

The NRN shall have the common fields prescribed for autoÑacknowledgment (see ¤ 18.5.2.1).

The NRN shall have the following receipt fields:

a)	NonÑreceipt reason: The value ipmÑdiscarded.

b)	Discard reason: The value impÑexpired or ipmÑobsoleted, whichever applies. If both apply, either value may be specified.

c)	AutoÑforward comment: Omitted.

d)	Returned IPM: If the IPM's return is requested by means of the notificationÑrequests component of its subject recipient specifier, and the convertedÑencodedÑinformationÑtypes component of message delivery's envelope argument is absent, IPM. Omitted otherwise.

#### 18.5.1.3	xe ""§Submission of NRN

The UA shall submit the NRN (if any) above by invoking message submission. Its envelope argument shall be as prescribed for autoÑacknowledgment (see ¤ 18.5.2.2), its content argument determined from the NRN as specified in ¤ 20.1.

### 18.5.2	xe ""§AutoÑacknowledgment

The UA shall subject to autoÑacknowledgment each candidate message with respect to whose content the following condition holds:

a)	The autoÑacknowledgment state variable has the value true and the IPM requests an RN of the UA's user by means of the notificationÑrequests component of the IPM's subject recipient specifier.

The UA shall autoÑacknowledge each such message as follows.

#### 18.5.2.1	xe ""§Construction of RN

The UA shall contruct an RN.

The RN shall have the following common fields:

a)	Subject IPM: The IPM's This IPM heading field.

b)	IPN originator: Specified or omitted as a local matter (but, of course, in accordance with ¤ 8.1.2).

c)	IPM preferred recipient: The recipient component of the IPM's subject recipient specifier, unless its formalÑname component is the O/R name of the UA's user, in which case the field shall be omitted.

d)	Conversion EITs: ConvertedÑencodedÑinformationÑtypes component of message delivery's envelope argument.

The RN shall have the following receipt field:

a)	Receipt time: The current date and time.

b)	Aknowledgment mode: The value automatic.

c)	Suppl receipt info: The autoÑacknowledgeÑsupplÑreceiptÑinfo state variable.

#### 18.5.2.2	xe ""§Submission of RN

The UA shall submit the RN above by invoking message submission with the following arguments:

a)	Envelope: The components of this argument shall be as prescribed for performance of the originate RN abstract operation with the following exceptions:

i)	Priority: As specified by message delivery's envelope argument.

ii)	PerÑmessageÑindicators: A local matter, except that conversionÑprohibited shall be among the values specified.

iii)	PerÑrecipientÑfields: A single field whose recipientÑname component shall be the originatorÑname component of message delivery's envelope argument. Reports shall not be requested.

b)	Content: Determined from the RN as specified in ¤ 20.1.

### 18.5.3	xe ""§AutoÑforwarding

The UA shall subject to autoÑforwarding every candidate message, provided that the autoÑforwardÑIPMs state variable has the value true.

The UA shall autoÑforward each such message as follows.

#### 18.5.3.1	xe ""§Prevention of loops

The UA shall suppress autoÑforwarding if, and only if, the IPM to be forwarded itself contains a forwarding IPM that the UA previously created. AutoÑforwarding shall be suppressed whether the forwarding IPM appears (directly) in a message bodypart of the IPM to be forwarded, or (nested) in a message body part of the IPM that appears in such a body part.

The UA shall consider itself to have created the forwarding IPM above (whose autoÑforwarded heading fields has the value true ) if, and only if, the originatorÑname component of the IPM's parameters component matches the O/R name of the UA's user.

Note Ñ AutoÑforwarding an IPM of the kind described above would constitute an autoÑforwarding ÒloopÓ.

#### 18.5.3.2	xe ""§Construction of IPM

The UA shall construct a forwarding IPM whose heading is the autoÑforwardÑheading state variable (its autoÑforwarded field having the value true) and whose body comprises a single body part of type message.

The message body part shall have the following components:

a)	Parameters: The envelope argument of message delivery.

b)	Data: The IPM to be forwarded.

#### 18.5.3.3	xe ""§Submission of IPM

The UA shall submit the IPM it constructed above by invoking message submission with the following arguments:

a)	Envelope: The components of this argument shall be as follows:

i)	OriginatorÑname: The O/R name of the UA's user.

ii)	ContentÑtype and originalÑencodedÑinformationÑtypes: Determined from the IPM as specified in ¤¤ 20.2 and 20.4.

iii)	ContentÑidentifier: Specified or omitted as a local matter.

iv)	Priority: As specified by message delivery's envelope argument.

v)	PerÑmessageÑindicators and extensions: A local matter.

vi)	DeferredÑdeliveryÑtime: Omitted.

vii)	PerÑrecipientÑfields: Their recipientÑname components shall be the O/R names that make up the autoÑforwardÑrecipients state variable. Their other components are a local matter.

b)	Content: Determined from the IPM as specified in ¤ 20.1.

#### 18.5.3.4	xe ""§Construction of NRN

The UA shall construct an NRN if, and only if, one is requested by means of the notificationÑrequests component of the forwarded IPM's subject recipient specifier.

The NRN shall have the common fields prescribed for the performance of autoÑacknowledgment.

The NRN shall have the following receipt fields:

a)	NonÑreceipt reason: The value ipmÑautoÑforwarded.

b)	Discard reason: Omitted.

c)	AutoÑforward comment: The autoÑforwardÑcomment state variable.

d)	Returned IPM: If the IPM's return is requested by means of the notificationÑrequests component of its subject recipient specifier, and the convertedÑencodedÑinformation types component of message delivery's envelope argument is absent, the IPM. Omitted otherwise.

#### 18.5.3.5	xe ""§Submission of NRN

The UA shall submit the NRN (if any) above by invoking message submission. Message submission's envelope argument shall be as prescribed for autoÑacknowledgment, its content argument determined from the NRN as specified in ¤ 20.1.

# 19	xe ""§Message store operation

An MS must perform certain interpersonal messagingÑspecific functions to qualify as an IPMS MS and thus distinguish itself from a generic MS. These functions are the subject of the present paragraph.

## 19.1	xe ""§Creation of information objects

An IPMS shall satisfy the following requirements related to the information objects it maintains:

a)	The MS shall maintain a separate information object for each (message containing an) IPM or IPN that is delivered to it.

b)	The MS shall maintain as a separate information object not only each (message containing a) forwarding IPM (pursuant to Item a)) but also each (message containing a) forwarded IPM (recursively).

c)	The MS shall assign sequence numbers depthÑfirst to the messages in the hierarchy formed by a forwarding IPM and its forwarded IPMs.

Example Ñ If IPM A contains IPMs B and C among its body parts, and if IPM B contains IPMS D and E among its body parts, sequence numbers will be assigned to the IPMs in the following order: A, B, D, E, and C.

## 19.2	xe ""§Maintenance of attributes

An IPMS MS shall satisfy the following requirements related to MS attributes:

a)	For each IPM or IPN it holds, the MS shall support the attributes of Annex C as specified therein.

b)	For each IPM it holds, the MS shall give the following messages to the defined values of the MSÑstatus attribute:

i)	new: No attribute values have been conveyed to the UA.

ii)	listed: At least one attribute value has been conveyed to the UA, and at least one body part has not been conveyed.

iii)	processed: All body parts have been conveyed to the UA.

c)	For each IPN it holds, the MS shall give the following meanings to the defined values of the MSÑstatus attribute:

i)	new: No attribute values have been conveyed to the UA.

ii)	listed: At least one attribute value has been conveyed to the UA, and at least one attribute other than Returned IPM has not been conveyed.

iii)	processed: All attributes, with the possible exception of returned IPM, have been conveyed to the UA.

d)	The MSÑstatus attribute shall reflect the state of affairs prior to an abstract operation invocation that alters its value.

e)	The contentÑtype attribute of each (message containing an) IPM or IPN that is delivered to the MS shall have the value idÑmctÑp2Ñ1984 or idÑmctÑp2Ñ1988 (see Annex D), as appropriate, depending upon the content type of the delivered message (see ¤ 20.2).

## 19.3	Notification of nonÑreceipt

When it discards an IPM while performing the delete abstract operation of the MS abstract service, the MS shall submit a NRN if one is requested and the IPM's MSÑstatus attribute has the value listed.

## 19.4	xe ""§AutoÑforwarding

An IPMS MS shall perform the autoÑforward action of Recommendation X.413 as specified in ¤ 18.5.3. It makes use of the otherÑparameters component of the autoÑforwardÑregistration argument of the register MS abstract operation of the MS abstract service. The data type of the otherÑparameters component is defined as follows:

Forwarded Info ::= SET {

autoÑforwardingÑcomment [0] AutoForwardComment OPTIONAL,

coverÑnote [1] IA5TextBodyPart OPTIONAL,

thisÑipmÑprefix [2] PrintableString (SIZE

(1. .ubÑipmÑidentifierÑsuffix)) OPTIONAL }

In addition, the MS shall satisfy the following requirements:

a)	Submit an NRN even if it retains a copy of the forwarded IPM.

b)	Draw the NRN's autoÑforward comment field, if any, from the otherÑparameters component.

c)	Draw the coverÑnote, if any, to be included with the forwarded IPM, from the otherÑparameters component.

d)	Prefix the userÑrelativeÑidentifier component of this IPM field of the forwarding IPM's heading with, if present, thisÑipmÑprefix.

Note Ñ An (IPMS) MS performs neither autoÑdiscard nor autoÑacknowledgment, except possibly as a local matter.

## 19.5	xe ""§Manual forwarding

An IPMS shall support the manual forwarding of a message using the forwardingÑrequest extension of Recommedation X.413, as specified in ¤ 6.6. The IPMS MS user may submit an IPM, including heading and body, using the message submission operation, and identify by using the forwardingÑrequest extension a message that is already in the MS, and to be combined with the submitted message body for forwarding to the message's recipient.

The submitted message body and the forwarded message are then combined by inserting the forwarded message as a message body part into the submitted message body.

# 20	xe ""§Message contents

As has already been seen, various secondary objects (e.g., UAs) have occasion to convey the information objects of section two as the contents of messages, as well as to convey probes concerning such messages. This paragraph specifies precisely how they shall do this.

The rules governing the transmittal of such messages and probes, and the semantics and abstract and transfer syntaxes of their contents, are called the interpersonal messaging protocol (P2).

Note Ñ The name ÒP2Ó reflects the historical fact that this was the second message handling protocol to be developed.

## 20.1	Content

A secondary object that submits a message containing an IPM or IPN shall supply as the octets of the octet string that constitutes the content of the message the result of encoding the InformationObject of section two in accordance with the basic encoding rules of Recommendation X.209.

## 20.2	xe ""§Content type

A secondary object that submits a message containing an IPM or IPN shall select its content type as follows.

If the IPM or IPN satisfies all of the following constraints, the Integer 2 shall be specified:

i)	The heading (of an IPM) lacks the extensions field.

ii)	The body (of an IPM) lacks externally defined body parts.

iii)	The parameters element of any videotex body part: . . . of (an IPM) lacks the syntax member.

iv)	Every component of the IPM or IPN that is a value of a data type defined as part of the MTS abstract service meets the constraints of Recommendation X.411 (1984).

The types in question are those listed in the IMPORTS clause of the ASN.1 module defined in Annex E. The constraints in question are detailed in an annex of Recommendation X.419.

v)	The data element of any message body part (of an IPM) satisfies these same constraints (recursively).

Otherwise, the Integer 22 shall be specified.

Note 1 Ñ The message content protocol (here) denoted by the Integer 2 is identical to that specified by Recommendation X.420 (1984) (as clarified by Version 6 of the X.400Ñseries Implementor's Guide), except that the simple formattable document body part type, defined in the latter, is omitted from the former.

Note 2 Ñ The Integer 2 is favoured, above, over the Integer 22 to foster interworking between systems conforming to this Recommendation and systems conforming (only) to Recommendation X.420 (1984).

Note 3 Ñ The MTS does not convert between message content protocols. Thus it does not convert between P2 as defined by this Recommendation alone (and denoted by the Integer 22) and P2 as defined by both this Recommendation and Recommendation X.420 (1984) (and denoted by the Integer 2).

## 20.3	Content length

A secondary object that submits a probe concerning a message containing an IPM or IPN shall specify as the length of the message's content the size in octets of the encoding of the instance in question of the InformationObject of Section 2 (a choice of an IPM or an IPN) when the basic encoding rules of Recommendation X.209 are followed. If those rules permit several (e.g., both primitive and constructed) encodings of that InformationObject, the content length may reflect any one of them.

## 20.4	Encoded information types

A secondary object that submits a message containing an IPM or IPN shall specify the basic encoded information type (EITs) and nonÑbasic parameters (NBPs) of the message as follows.

In the case of an IPN, the basic EITs shall be unspecified.

In the case of an IPM, the basic EITs and NBPs shall be specified in accordance with the following rules:

a)	Multiple body parts: The basic EITs (if any) and NBPs (if any) of the message shall comprise the logical union of the basic EITs and NBPs of the IPM's individual body parts, respectively.

b)	(Forwarded) message body part: The basic EITs (if any) and NBPs (if any) of a message body part shall be those of the forwarded message.

c)	Externally defined body part: An externally defined body part whose extended type corresponds to a basic type (see Annex B) shall be treated in the manner prescribed for the basic type.

Any other extended body part shall be handled as follows. If there corresponds to the type one or more externally defined EITs, they shall be specified. Otherwise, the undefined EIT shall be indicatd. In either case, no NBPs shall be specified.

d)	Basic body part: The basic EITs (if any) and NBPs (if any) of an individual body part of type other than message and externally defined shall depend upon that body part type as specified in Table 2/X.420. A body part type for which the table specifies no basic EITs shall result in the setting of no bits in the basic EITs bit string.

e)	Encrypted body part : The effect of an encrypted body part upon the basic EITs and NBPs to be specified is for further study.

TABLE 2/X.420

Interpersonal messaging basic EITs and NBPs

| Body part type | Basic EIT | NBPs |
| --- | --- | --- |
| IA5 text | IA5 text | Ñ |
| Voice | Voice | Ñ |
| G3 facsimile | G3 facsimile | G3 facsimile |
| G4 class 1 | G4 class 1 | G4 class 1/mixedÑmode |
| Teletex | Teletex | Teletex |
| Videotex | Videotex | Ñ |
| Encrypted |  |  |
| Message | (see text) | (see text) |
| MixedÑmode | MixedÑmode | G4 class 1/mixedÑmode |
| Bilaterally defined | Undefined | Ñ |
| Nationally defined | Undefined | Ñ |
| Externally defined | (see text) | (see text) |

# 21	Port realization

How an MS or the MTS concretely realizes the secondary ports it supplies is specified in Recommenda-tionÊX.419.

How a UA, TLMA, or AU concretely realizes the primary ports it supplies is beyond the scope of this Recommendation.

Note 1 Ñ A UA's user interface is a local matter. A wide variety of interfaces involving, e.g., a wide variety of input/output devices are possible.

Note 2 Ñ A TLMA's realization of its primary ports is specified by Recommendation T.330.

Note 3 Ñ An AU provides its primary ports by means of the particular communication system to which that AU provides access.

# 22	xe ""§Conformance

The requirements a secondary object (excluding the MTS) and its implementor shall meet when the latter claims the former's conformance to this Recommendation are identified below. A number of the conformance requirements distinguish between support upon origination and support upon reception.

## 22.1	Origination versus reception

A UA, TLMA or AU shall be said to support upon origination a particular heading field, heading extension, basic body part type, or extended body part type if, and only if, it accepts, preserves, and emits, in full accord with this Recommendation, that particular heading field or extension, or body parts of that particular basic or extended type, whenever a user calls upon it to convey an IPM containing them to the MTS or the user's MS (the latter only in the case of a UA).

A UA, TLMA, or AU shall be said to support upon reception a particular heading field, heading extension, basic body part type, or extended body part type if, and only if, it accepts, preserves, and emits, in full accord with this Recommendation, that particular heading field or extension, or body parts of that particular basic or extended type, whenever the MTS or a user's MS (the latter only in the case of a UA) calls upon it to convey to the user an IPM containing them.

Note Ñ In point of fact, a PDAU supports nothing upon origination because it is not a supplier of the origination port.

## 22.2	Statement requirements

The implementor of an IPMS, UA, IPMS MS, TLMA, or AU shall state the following. For each item below he shall make separate statements concerning conformance upon origination and conformance upon reception:

a)	The heading fields and heading extensions for which he claims conformance.

b)	The basic and extended body part types for which he claims conformance.

c)	In the case of an IPMS UA or IPMS MS, the interpersonal messagingÑspecific MS attributes for which it claims conformance.

## 22.3	Static requirements

An IPMS UA, IPMS MS, TLMA or AU shall satisfy the following static requirements:

a)	An IPMS UA, IPMS MS, TLMA, or AU shall implement the heading fields and heading extensions, and the basic and extended body part types for which conformance is claimed.

b)	An IPMS UA or IPMS MS shall support the interpersonal messagingÑspecific MS attributes for which conformance is claimed, but including as a minimum those designated mandatory in Annex C.

c)	An IPMS UA, IPMS MS, TLMA, or AU shall concretely realize its abstract ports as specified in ¤ 21.

d)	An IPMS UA or IPMS MS shall be able to both submit and accept delivery of messages of both of the content types of ¤ 20.2. A TLMA or AU shall be able to both import and export such messages.

## 22.4	Dynamic requirements

An IPMS UA, IPMS MS, TLMA, or AU shall satisfy the following dynamic requirements:

a)	An IPMS UA or IPMS MS shall follow the rules of operation specified in ¤ 18 or ¤ 19, respectively.

b)	An IPMS UA, IPMS MS, TLMA, or AU shall submit and accept delivery of messages whose contents are as specified in ¤ 20.

c)	An IPMS UA, IPMS MS, TLMA, or AU shall register with the MTS its ability to accept delivery of messages of both of the content types of ¤ 20.2.

ANNEX A

(to Recommendation X.420)

xe ""§Heading extensions

This Annex is an integral part of this Recommendation.

This Annex defines all (presently defined) heading extensions.

## A.1	Incomplete copy

The incomplete copy heading extension, by its presence, indicates that one of more body parts or heading fields are absent from the body of (the present instance of) the IPM. The extension comprises a null (by default).

IncompleteÑcopy HEADINGÑEXTENSION

::= idÑhexÑincomplete copy

If this extension is absent from the extension heading field, all body parts shall be considered present.

## A.2	Languages

The languages heading extension identifies the languages used in the composition of the IPM's subject heading field and body. The extension comprises a set of zero or more printable strings, each one of the twoÑcharacter language codes identified by ISO 639.2.

languages HEADINGÑEXTENSION

VALUE SET OF Language

::= idÑhexÑlanguages

Language ::= PrintableString (SIZE (2. .2))

If this extension is absent from the extensions heading field or no languages are indicated, the languages shall be considered unspecified.

ANNEX B

(to Recommendation X.420)

xe ""§Extended body part types

This Annex is an integral part of this Recommendation.

For each basic body part type, this Recommendation defines as follows an equivalent extended body part type.

ia5ÑtextÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

PARAMETERS	IA5TextParameters IDENTIFIED BY idÑepÑia5Ñtext

DATA			IA5TextData

::= idÑetÑia5Ñtext

voiceÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

PARAMETERS	VoiceParameters IDENTIFIED BY idÑepÑvoice

DATA			VoiceData

::= idÑetÑvoice

g3ÑfacsimileÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

PARAMETERS	G3FacsimileParameters IDENTIFIED BY idÑepÑg3Ñfacsimile

DATA			G3FacsimileData

::= idÑetÑg3Ñfacsimile

g4Ñclass1ÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

DATA			G4ClassBodyPart

::= idÑetÑg4Ñclass1

teletexÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

PARAMETERS	TeletexParameters IDENTIFIED BY idÑepÑteletex

DATA			TeletexData

::= idÑetÑteletex

videotexÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

PARAMETERS	VideotexParameters IDENTIFIED BY idÑepÑvideotex

DATA			VideotexData

::= idÑetÑvideotex

encryptedÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

PARAMETERS	EncryptedParameters IDENTIFIED BY idÑepÑencrypted

DATA			EncryptedData

::= idÑetÑencrypted

messageÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

PARAMETERS	MessageParameters IDENTIFIED BY idÑepÑmessage

DATA			MessageData

::= idÑetÑmessage

mixedÑmodeÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

DATA			MixedModeBodyPart

::= idÑetÑmixedÑmode

bilaterallyÑdefinedÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

DATA			BilaterallyDefinedBodyPart

::= idÑetÑbilaterallyÑdefined

nationallyÑdefinedÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

DATA			NationallyDefinedBodyPart

::= idÑetÑnationallyÑdefined

ANNEX C

(to Recommendation X.420)

xe ""§Message store attributes

This Annex is an integral part of this Recommendation.

As described in Recommendation X.413, an MS maintains and provides access to certain attributes (e.g., the importance) of each information object it holds. An attribute comprises a type and, depending upon the type, one or more values. Attributes that may assume several values simultaneously (all pertaining to one object) are termed multiÑvalued; those that may assume just one value, singleÑvalued. Some attributes pertain to information objects of all kinds, others only to those of certain kinds (e.g., those of Section 2).

This Annex defines the MS attributes specific to interpersonal messaging.

All of the attributes defined in this Annex, except those corresponding to extended body part types (which cannot be enumerated; see ¤ C.3.6), are listed alphabetically, for reference, in the first column of Table CÑ1/X.420. This Table records their presence in a delivered message entry. None of them appears in either a delivered report entry or a returned content entry. See Recommendation X.413 for an elaboration of the Table's legend.

## C.1	Summary attributes

Some attributes summarize an interpersonal messaging information object. These attributes are defined and described below.

### C.1.1	xe ""§IPM entry type

The IPM entry type attribute identifies an information object's type.

ipmÑentryÑtype ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IPMEntryType

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑsatÑipmÑentryÑtype

IPMEntryType ::= ENUMERATED {

ipm(0),

rn(1),

nrn(2) }

This attribute may assume any one of the following values:

a)	ipm: The information object is an IPN.

b)	rn: The information object is an RN.

c)	nrn: The information object is an NRN.

An MS that supports this attribute shall maintain it for an information object that it holds if, and only if, that object is a message whose content is an IPM or IPN.

TABLE CÑ1/X.420

Summary of MS attributes

| Attribute | V | L | IPM | P NRN | RN | L | S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acknowledgement mode | S | O | Ñ | Ñ | M | Y | Y |
| Authorizing users | M | O | C | Ñ | Ñ | Y | N |
| AutoÑforward comment | S | O | Ñ | C | Ñ | Y | N |
| AutoÑforwarded | S | O | C | Ñ | Ñ | Y | Y |
| Bilaterally defined body parts | M | O | C | Ñ | Ñ | N | N |
| Blind copy recipients | M | O | C | Ñ | Ñ | Y | N |
| Body | S | M | M | Ñ | Ñ | N | N |
| Conversion EITs | M | O | Ñ | C | C | Y | N |
| Copy recipients | M | O | C | Ñ | Ñ | Y | N |
| Discard reason | S | O | Ñ | C | Ñ | Y | Y |
| Encrypted body parts | M | O | C | Ñ | Ñ | N | N |
| Encrypted data | M | O | C | Ñ | Ñ | N | N |
| Encrypted parameters | M | O | C | Ñ | Ñ | N | N |
| Expiry time | S | O | C | Ñ | Ñ | Y | N |
| Extended body part types | M | O | C | Ñ | Ñ | Y | Y |
| G3 facsimile body parts | M | O | C | Ñ | Ñ | N | N |
| G3 facsimile data | M | O | C | Ñ | Ñ | N | N |
| G3 facsimile parameters | M | O | C | Ñ | Ñ | N | N |
| G4 class 1 body parts | M | O | C | Ñ | Ñ | N | N |
| Heading | S | M | M | Ñ | Ñ | N | N |
| IA5 text body parts | M | O | C | Ñ | Ñ | N | N |
| IA5 text data | M | O | C | Ñ | Ñ | N | N |
| IA5 text parameters | M | O | C | Ñ | Ñ | N | N |
| Importance | S | O | C | Ñ | Ñ | Y | Y |
| Incomplete copy | S | O | C | Ñ | Ñ | Y | N |
| IPM entry type | S | M | M | M | M | Y | Y |
| IPM preferred recipient | S | O | Ñ | C | C | Y | N |
| IPM synopsis | S | O | M | Ñ | Ñ | N | N |
| IPM originator | S | O | Ñ | C | C | Y | N |
| Languages | M | O | C | Ñ | Ñ | Y | N |
| Message body parts | M | O | C | Ñ | Ñ | N | N |
| Message data | M | O | C | Ñ | Ñ | N | N |
| Message parameters | M | O | C | Ñ | Ñ | N | N |
| MixedÑmode body parts | M | O | C | Ñ | Ñ | N | N |
| Nationally defined body parts | M | O | C | Ñ | Ñ | N | N |
| NonÑreceipt reason | S | O | Ñ | M | Ñ | Y | Y |
| NRN requestors | M | O | C | Ñ | Ñ | Y | N |
| Obsoleted IPMs | M | O | C | Ñ | Ñ | Y | N |
| Originator | S | O | C | Ñ | Ñ | Y | N |

TABLE CÑ1/X.420 (cont.)

| Attribute | V | L | IPM | P NRN | RN | L | S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Primary recipients | M | O | C | Ñ | Ñ | Y | N |
| Receipt time | S | O | Ñ | Ñ | M | Y | N |
| Related IPMs | M | O | C | Ñ | Ñ | Y | N |
| RepliedÑto IPM | S | O | C | Ñ | Ñ | Y | N |
| Reply recipients | M | O | C | Ñ | Ñ | Y | N |
| Reply requestors | M | O | C | Ñ | Ñ | Y | N |
| Reply time | S | O | C | Ñ | Ñ | Y | N |
| Returned IPM | S | O | Ñ | C | Ñ | Y | N |
| RN requestors | M | O | C | Ñ | Ñ | Y | N |
| Sensitivity | S | O | C | Ñ | Ñ | Y | Y |
| Subject | S | O | C | Ñ | Ñ | Y | N |
| Subject IPM | S | M | Ñ | M | M | Y | N |
| Suppl. receipt info | S | O | Ñ | Ñ | C | Y | N |
| Teletex body parts | M | O | C | Ñ | Ñ | N | N |
| Teletex data | M | O | C | Ñ | Ñ | N | N |
| Teletex parameters | M | O | C | Ñ | Ñ | N | N |
| This IPM | S | M | M | Ñ | Ñ | Y | N |
| Videotex body parts | M | O | C | Ñ | Ñ | N | N |
| Videotex data | M | O | C | Ñ | Ñ | N | N |
| Videotex parameters | M | O | C | Ñ | Ñ | N | N |
| Voice body parts | M | O | C | Ñ | Ñ | N | N |
| Voice data | M | O | C | Ñ | Ñ | N | N |
| Voice parameters | M | O | C | Ñ | Ñ | N | N |

V	Single/multi valued

L	Support level by MS and access UA

P	Presence in delivered message entry

L	Available for List, alert

S	Available for summarize

### C.1.2	xe ""§IPM synopsis

The IPM synopsis attribute gives the structure, characteristics, size and processing status of an IPM at the granularity of individual body parts.

ipmÑsynopsis ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IPMSynopsis

SINGLE VALUE

::= idÑsatÑipmÑsynopsis

The synopsis of an IPM comprises a synopsis of each of its body parts. The synopses appear in the order in which the body parts appear.

IPMSynopsis ::= SEQUENCE OF BodyPartSynopsis

The synopsis of a body part takes either of two forms depending upon whether the body part is of type message. This enables the synopsis of a forwarding IPM to encompass the body parts of each forwarded IPM (recursively), as well as those of the forwarding IPM itself.

BodyPartSynopsis ::= CHOICE {

message	[0]	MessageBodyPartSynopsis

nonÑmessage	[1]	NonMessageBodyPartSynopsis }

MessageBodyPartSynopsis ::= SEQUENCE {

number	[0]	SequenceNumber,

synopsis	[1]	IPMSynopsis }

NonMessageBodyPartSynopsis ::= SEQUENCE {

type			[0]	OBJECT IDENTIFIER,

parameters	[1]	ExternallyDefinedParameters,

size			[2]	INTEGER,

processed	[3]	BOOLEAN DEFAULT FALSE }

The synopsis of a message body part has the following components:

a)	Number (M): The sequence number that the MS assigns to the entry that the message body part represents.

b)	Synopsis (M): The synopsis of the IPM that forms the content of the message that the body part represents.

The synopsis of a body part of type other than message has the following components. For purposes of this synopsis, the body part is considered to be of type externally defined, whether or not (see Annex B) it was so conveyed to the MS:

a)	Type (M): The body part's extended type, i.e., the directÑreference component of the body part's data component. An object identifier.

b)	Parameters (M): The body part's format and control parameters, i.e., the body part's parameters component. An Any.

c)	Size (M): The size in octets of the encoding of the Encoding component of the body part's Data component when the basic encoding rules of Recommendation X.209 are followed. If those rules permit several (e.g., both primitive and constructed) encodings of the component, the size may reflect any one of them. An Integer.

d)	Processed (D false): An indication of whether or not the body part has been conveyed to the UA by means of the MS' list or fetch abstract operation. A Boolean.

An MS that supports this attribute shall maintain it for an information object that it holds if, and only if, that object is a message whose content is an IPM.

Note Ñ As a consequence of its variability, the value of the size component should be considered only an estimate of the body part's size.

## C.2	xe ""§Heading attributes

Some attributes are derived from the heading of an IPM. These attributes are defined and described below.

### C.2.1	Heading

The heading attribute is the (entire) heading of an IPM.

heading ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX Heading

SINGLE VALUE

::= idÑhatÑheading

An MS that supports this attribute shall maintain it for an information object that it holds if, and only if, that object is a message whose content is an IPM.

### C.2.2	Heading analyses

Some attributes have as their values O/R descriptors selected after analysis of the heading. They identify the ÒprimaryÓ, ÒcopyÓ, and blind ÒcopyÓ recipients of an IPM of whom an RN, NRN, or reply is requested.

rnÑrequestors ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ORDescriptor

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑrnÑrequestors

nrnÑrequestors ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ORDescriptor

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑnrnÑrequestors

replyÑrequestors ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ORDescriptor

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑreplyÑrequestors

An MS that supports one of these attributes shall maintain it for an information object that it holds if, and only if, that object is a message whose content is an IPM whose heading requests, of at least one user, or DL, an RN, NRN, or reply, respectively. It shall maintain one attribute value for every recipient specifier in the IPM's primary, copy, or blind copy recipients field whose notificationÑrequests component includes the value rn (in the case of the first attribute) or nrn (in the case of the second, or whose ReplyÑrequested component signifies, by either its presence or its absence, that a reply is requested (in the case of the third). The value shall be the recipient's specifier's recipient component.

### C.2.3	xe ""§Heading fields

Some attributes bear the names of heading fields and have those fields as their values. The ordering for the expiry time and reply time attributes is increasing chronological order.

thisÑipm ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX This IPMField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑhatÑthisÑipm

originator ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX OriginatorField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑhatÑoriginator

repliedÑtoÑIPM ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX RepliedToIPMField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑhatÑrepliedÑtoÑIPM

subject ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX SubjectField

MATCHES FOR EQUALITY SUBSTRINGS

SINGLE VALUE

::= idÑhatÑsubject

expiryÑtime ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ExpiryTimeField

MATCHES FOR EQUALITY ORDERING

SINGLE VALUE

::= idÑhatÑexpiryÑtime

replyÑtime ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ReplyTimeField

MATCHES FOR EQUALITY ORDERING

SINGLE VALUE

::= idÑhatÑreplyÑtime

importance ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ImportanceField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑhatÑimportance

sensitivity ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX SensitivityField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑhatÑsensitivity

autoÑforwarded ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX AutoForwardedField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑhatÑautoÑforwarded

An MS that supports one of these attributes shall maintain it for an information object that it holds if, and only if, that object is a message whose content is an IPM whose heading contains the field whose name the attribute bears.

### C.2.4	Heading subÑfields

Some attributes bear the names of heading fields and have subÑfields of those fields as their values.

authorizingÑusers ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX AuthorizingUsersSubfield

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑauthorizingÑusers

primaryÑrecipients; ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX PrimaryRecipientsSubField

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑprimaryÑrecipients

copyÑrecipients ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX CopyRecipientsSubfield

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑcopyÑrecipients

blindÑcopyÑrecipients ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX BlindCopyRecipientsSubfield

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑblindÑcopyÑrecipients

obsoletedÑIPMs ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ObsoletedIPMsSubfield

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑobsoletedÑIPMs

relatedÑIPMs ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX RelatedIPMsSubfield

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑrelatedÑIPMs

replyÑrecipients ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ReplyRecipientsSubfield

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑreplyÑrecipients

An MS that supports one of these attributes shall maintain it for an information object that it holds if, and only if, that object is a message whose content is an IPM whose heading contains the field whose name the attribute bears. It shall maintain one attribute value for each subÑfield.

### C.2.5	Heading extensions

Some attributes bear the names of heading extensions and have as their values the values of those extensions or a part thereof.

incompleteÑcopy ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX incompleteCopyExtensionValue

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑhatÑincompleteÑcopy

languages ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX Language

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑlanguages

An MS that supports one of these attributes shall maintain it for an information object that it holds if, and only if, that object is a message whose content is an IPM whose heading contains the extension whose name the attribute bears. In the case of the language attribute, the MS shall maintain one attribute value for each language the extension identifies.

## C.3	Body attributes

Some attributes are derived from the body of an IPM. These attributes are defined and described below.

### C.3.1	Body

The body attribute is the (entire) body of an IPM.

body ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX Body

SINGLE VALUE

::= idÑbatÑbody

An MS that supports this attribute shall maintain it for an information object that it holds if, and only if, that object is a message whose content is an IPM.

### C.3.2	Basic body parts

Some attributes bear the names of basic body part types and have, with one exception, such body parts as their values.

An MS holds each forwarded IPM (i.e., each message body part) as an information object in its own right, separate from the forwarding IPM. That information object, of course, is a message whose content is an IPM. The message body parts attribute below, therefore, has as its values the sequence numbers the MS assigns to these messages.

ia5ÑtextÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IA5TextBodyPart

MULTI VALUE

::= idÑbatÑia5ÑtextÑbodyÑparts

voiceÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX VoiceBodyPart

MULTI VALUE

::= idÑbatÑvoiceÑbodyÑparts

g3ÑfacsimileÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX G3FacsimileBodyPart

MULTI VALUE

::= idÑbatÑg3ÑfacsimileÑbodyÑparts

g4Ñclass1ÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX G4Class1BodyPart

MULTI VALUE

::= idÑbatÑg4Ñclass1ÑbodyÑparts

teletexÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX TeletexBodyPart

MULTI VALUE

::= idÑbatÑteletexÑbodyÑparts

videotexÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX VideotexBodyPart

MULTI VALUE

::= idÑbatÑvideotexÑbodyÑparts

encryptedÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX EncryptedBodyPart

MULTI VALUE

::= idÑbatÑencryptedÑbodyÑparts

messageÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX SequenceNumber

MULTI VALUE

::= idÑbatÑmessageÑbodyÑparts

mixedÑmodeÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX MixedModeBodyPart

MULTI VALUE

::= idÑbatÑmixedÑmodeÑbodyÑparts

bilaterallyÑdefinedÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX BilaterallyDefinedBodyPart

MULTI VALUE

::= idÑbatÑbilaterallyÑdefinedÑbodyÑparts

nationallyÑdefinedÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX NationallyDefinedBodyPart

MULTI VALUE

::= idÑbatÑnationallyÑdefinedÑbodyÑparts

An MS that supports one of these attributes shall maintain it for an information object that it holds if, and only if, that object is a message whose content is an IPM whose body contains one or more body parts of the type whose name the attribute bears. It shall maintain one attribute value for each such body part.

### C.3.3	Basic body part parameters components

Some attributes bear the names of basic body part types and have the parameters components of such body parts as their values.

ia5ÑtextÑparameters ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IA5TextParameters

MULTI VALUE

::= idÑbatÑia5ÑtextÑparameters

voiceÑtextÑparameters ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IA5VoiceParameters

MULTI VALUE

::= idÑbatÑvoiceÑparameters

g3ÑfacsimileÑparameters ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX G3FacsimileParameters

MULTI VALUE

::= idÑbatÑg3ÑfacsimileÑparameters

teletexÑparameters ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX TeletexParameters

MULTI VALUE

::= idÑbatÑteletexÑparameters

videotexÑparameters ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX VideotexParameters

MULTI VALUE

::= idÑbatÑvideotexÑparameters

encryptedÑparameters ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX EncryptedParameters

MULTI VALUE

::= idÑbatÑencryptedÑparameters

messageÑparameters ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX MessageParameters

MULTI VALUE

::= idÑbatÑmessageÑparameters

An MS that supports one of these attributes shall maintain it for an information object that it holds if, and only if, that object is a message whose content is an IPM whose body contains one or more body parts of the type whose name the attribute bears. It shall maintain one attribute value for each such body part.

### C.3.4	xe ""§Basic body part data components

Some attributes bear the names of basic body part types and have the data components of such body parts as their values.

ia5ÑtextÑdata ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IA5TextData

MULTI VALUE

::= idÑbatÑia5ÑtextÑdata

voiceÑdata ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX VoiceData

MULTI VALUE

::= idÑbatÑvoiceÑdata

g3ÑfacsimileÑdata ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX G3FacsimileData

MULTI VALUE

::= idÑbatÑg3ÑfacsimileÑdata

teletexÑdata ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX TeletexData

MULTI VALUE

::= idÑbatÑteletexÑdata

videotexÑdata ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX VideotexData

MULTI VALUE

::= idÑbatÑvideotexÑdata

encryptedÑdata ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX EncryptedData

MULTI VALUE

::= idÑbatÑencryptedÑdata

messageÑdata ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX MessageData

MULTI VALUE

::= idÑbatÑmessageÑdata

An MS that supports one of these attributes shall maintain it for an information object that it holds if, and only if, that object is a message whose content is an IPM whose body contains one or more body parts of the type whose name the attribute bears. It shall maintain one attribute for each such body part.

### C.3.5	Extended body part types

The extended body part types attribute identifies the extended body part types represented in an IPM.

extendedÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX OBJECT IDENTIFIER

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑbatÑextendedÑbodyÑparts

An MS that supports this attribute shall maintain it for an information object that it holds if, and only if, that object is a message whose content is an IPM whose body contains one or more externally defined body parts. It shall maintain one attribute value for every such type present. The value shall denote the type as specified in ¤ 7.3.12.

Note Ñ Each value of this attribute corresponds to one of the attributes described in ¤ C.3.6 below.

### C.3.6	xe ""§Extended body parts

Some attributes, unnamed, have as their values the encoding components (see ¤ 7.3.12) of the ASN.1 externals that constitute the data components of externally defined body parts.

To each extended body part type there correspond two attributes. The first attribute is denoted by the object identifier that is the directÑreference component (again, see ¤ 7.3.12) of the external that constitutes the data component of a body part of that type. The content of this first attribute is that data component. The second attribute is denoted by the object identifier that is the directÑreference component of the external that constitutes the parameters component of a body part of that type. The content of this second attribute is that parameters component.

An MS that supports one of these body parts shall maintain both attributes for an information object that it holds if, and only if, that object is a message whose content is an IPM whose body contains one or more body parts of the type that corresponds to that attribute. It shall maintain one value of each attribute for each such body part.

Note 1 Ñ The extended body part attributes cannot be enumerated in practice because the extended body part types cannot be so enumerated.

Note 2 Ñ The extended body part types attribute (see ¤ C.3.5) determines the extended body part attributes for a particular IPM.

## C.4	xe ""§Notification attributes

Some attributes are derived from an IPN. These attributes are defined and described below.

### C.4.1	Common fields

Some attributes that bear the names of common fields and have those fields as their values.

subjectÑipm ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX SubjectIPMField

MATCHES FOR EQUALITY SUBSTRINGS

SINGLE VALUE

::= idÑnatÑsubjectÑipm

ipnÑoriginator ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IPNOriginatorField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑnatÑipnÑoriginator

ipnÑpreferredÑrecipient ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IPMPreferredRecipientField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑnatÑipmÑpreferredÑrecipient

conversionÑeits ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX MSÑEITs

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑnatÑconversionÑeits

An MS that supports one of these attributes shall maintain it for an information object that it holds if, and only if, that object is a message whose content is an IPN that contains the field whose name the attribute bears.

### C.4.2	NonÑreceipt fields

Some attributes bear the names of nonÑreceipt fields and have those fields as their values.

nonÑreceiptÑreason ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX NonReceiptReasonField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑnatÑnonÑreceiptÑreason

discardÑreason ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX DiscardReasonField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑnatÑdiscardÑreason

autoÑforwardÑcomment ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX AutoForwardCommentField

MATCHES FOR EQUALITY SUBSTRINGS

SINGLE VALUE

::= idÑnatÑautoÑforwardÑcomment

returnedÑIPM ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ReturnedIPMField

SINGLE VALUE

::= idÑnatÑreturnedÑipm

An MS that supports one of these attributes shall maintain it for an information that it holds if, and only if, that object is a message whose content is an NRN that contains the field whose name the attribute bears.

### C.4.3	Receipt fields

Some attributes bear the names of receipt fields and have those fields as their values. The ordering for the receipt time attribute is increasing chronological order.

receiptÑtime ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ReceiptTimeField

MATCHES FOR EQUALITY ORDERING

SINGLE VALUE

::= idÑnatÑreceiptÑtime

acknowledgment ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX AcknowledgmentModeField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑnatÑacknowledgmentÑmode

supplÑreceiptÑinfo ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX SupplReceiptInfoField

MATCHES FOR EQUALITY SUBSTRINGS

SINGLE VALUE

::= idÑnatÑsupplÑreceiptÑinfo

An MS that supports one of these attributes shall maintain it for an information object that it holds if, and only if, that object is a message whose content is an RN that contains the field whose name the attribute bears.

ANNEX D

(to Recommendation X.420)

xe ""§Reference definition of object identifiers

This Annex is an integral part of this Recommendation.

This Annex defines for reference purposes various object identifiers cited in the ASN.1 modules of subsequent Annexes. It uses ASN.1.

All object identifiers this Recommendation assigns are assigned in this Annex. The Annex is definitive for all but those for ASN.1 modules and the IPMS application itself. The definitive assignments for the former occur in the modules themselves; other references to them appear in IMPORT clauses. The latter is fixed.

IMPSObjectIdentifiers {jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) objectÑidentifiers(0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

ÑÑ Prologue

ÑÑ Exports everything.

IMPORTS ÑÑ nothing ÑÑ;

ID ::= OBJECT IDENTIFIER

ÑÑ Interpersonal Messaging (not definitive)

idÑipms ID ::= {jointÑisoÑccitt mhsÑmotis(6) ipms(1)} Ñ Ñnot definitive

ÑÑ Categories

idÑmod	ID ::= { idÑipms 0 } ÑÑ	modules; not definitive

idÑot	ID ::= { idÑipms 1 } ÑÑ	object types

idÑpt	ID ::= { idÑipms 2 } ÑÑ	port types

idÑref	ID ::= { idÑipms 3 } ÑÑ	refinements

idÑet	ID ::= { idÑipms 4 } ÑÑ	extended body part types

idÑhex	ID ::= { idÑipms 5 } ÑÑ	heading extensions

idÑsat	ID ::= { idÑipms 6 } ÑÑ	summary attributes

idÑhat	ID ::= { idÑipms 7 } ÑÑ	heading attributes

idÑbat	ID ::= { idÑipms 8 } ÑÑ	body attributes

idÑnat	ID ::= { idÑipms 9 } ÑÑ	notification attributes

idÑmct	ID ::= { idÑipms 10 } ÑÑ	message content types

idÑep	ID ::= { idÑipms 11 } ÑÑ	extended body part parameters

ÑÑ Modules

idÑmodÑobjectÑidentifiers						ID ::= { idÑmod 0 } ÑÑ not definitive

idÑmodÑfunctionalÑobjects						ID ::= { idÑmod 1 } ÑÑ not definitive

idÑmodÑinformationÑobjects					ID ::= { idÑmod 2 } ÑÑ not definitive

idÑmodÑabstractÑservice						ID ::= { idÑmod 3 } ÑÑ not definitive

idÑmodÑheadingÑextensions					ID ::= { idÑmod 6 } ÑÑ not definitive

idÑmodÑextendedÑbodyÑpartÑtypes			ID ::= { idÑmod 7 } ÑÑ not definitive

idÑmodÑmessageÑstoreÑattributes				ID ::= { idÑmod 8 } ÑÑ not definitive

idÑmodÑupperÑbounds							ID ::= { idÑmod 10 } ÑÑ not definitive

ÑÑ Object types

idÑotÑipme										ID ::= { idÑot 0 }

idÑotÑipmsÑuser									ID ::= { idÑot 1 }

idÑotÑipms										ID ::= { idÑot 2 }

idÑotÑipmsÑua									ID ::= { idÑot 3 }

idÑotÑipmsÑms									ID ::= { idÑot 4 }

idÑotÑtlma										ID ::= { idÑot 5 }

idÑotÑtlxau										ID ::= { idÑot 6 }

idÑotÑpdau										ID ::= { idÑot 7 }

ÑÑ Port types

idÑptÑorigination								ID ::= { idÑpt 0 }

idÑptÑreception									ID ::= { idÑpt 1 }

idÑptÑmanagement								ID ::= { idÑpt 2 }

ÑÑ Refinements

idÑrefÑprimary								ID ::= { idÑref 0 }

idÑrefÑsecondary								ID ::= { idÑref 1 }

ÑÑ Extended body part types

idÑetÑia5Ñtext									ID ::= { idÑet 0 }

idÑetÑvoice									ID ::= { idÑet 1 }

idÑetÑg3Ñfacsimile								ID ::= { idÑet 2 }

idÑetÑg4Ñclass1								ID ::= { idÑet 3 }

idÑetÑteletex									ID ::= { idÑet 4 }

idÑetÑvideotex								ID ::= { idÑet 5 }

idÑetÑencrypted								ID ::= { idÑet 6 }

idÑetÑmessage								ID ::= { idÑet 7 }

idÑetÑmixedÑmode								ID ::= { idÑet 8 }

idÑetÑbilaterallyÑdefined						ID ::= { idÑet 9 }

idÑetÑnationallyÑdefined						ID ::= { idÑet 10 }

ÑÑ Heading extensions

idÑhexÑincompleteÑcopy						ID ::= { idÑhex 0 }

idÑhexÑlanguages								ID ::= { idÑhex 1 }

ÑÑ Summary attributes

idÑsatÑipmÑentryÑtype							ID ::= { idÑsat 0 }

idÑsatÑipmÑsynopsis							ID ::= { idÑsat 1 }

ÑÑ Heading attributes

idÑhatÑheading								ID ::= { idÑhat 0 }

idÑhatÑthisÑipm								ID ::= { idÑhat 1 }

idÑhatÑoriginator								ID ::= { idÑhat 2 }

idÑhatÑrepliedÑtoÑIPM							ID ::= { idÑhat 3 }

idÑhatÑsubject								ID ::= { idÑhat 4 }

idÑhatÑexpiryÑtime							ID ::= { idÑhat 5 }

idÑhatÑreplyÑtime								ID ::= { idÑhat 6 }

idÑhatÑimportance								ID ::= { idÑhat 7 }

idÑhatÑsensitivity								ID ::= { idÑhat 8 }

idÑhatÑautoÑforwarded							ID ::= { idÑhat 9 }

idÑhatÑauthorizingÑusers						ID ::= { idÑhat 10 }

idÑhatÑprimaryÑrecipients						ID ::= { idÑhat 11 }

idÑhatÑcopyÑrecipients							ID ::= { idÑhat 12 }

idÑhatÑblindÑcopyÑrecipients					ID ::= { idÑhat 13 }

idÑhatÑobsoletedÑIPMs							ID ::= { idÑhat 14 }

idÑhatÑrelatedÑIPMs							ID ::= { idÑhat 15 }

idÑhatÑreplyÑrecipients							ID ::= { idÑhat 16 }

idÑhatÑincompleteÑcopy						ID ::= { idÑhat 17 }

idÑhatÑlanguages								ID ::= { idÑhat 18 }

idÑhatÑrnÑrequestors							ID ::= { idÑhat 19 }

idÑhatÑnrnÑrequestors							ID ::= { idÑhat 20 }

idÑhatÑreplyÑrequestors							ID ::= { idÑhat 21 }

ÑÑ Body attributes

idÑbatÑbody									ID ::= { idÑbat 0 }

idÑbatÑia5ÑtextÑbodyÑparts					ID ::= { idÑbat 1 }

idÑbatÑvoiceÑbodyÑparts						ID ::= { idÑbat 2 }

idÑbatÑg3ÑfacsimileÑbodyÑparts				ID ::= { idÑbat 3 }

idÑbatÑg4Ñclass1ÑbodyÑparts					ID ::= { idÑbat 4 }

idÑbatÑteletexÑbodyÑparts						ID ::= { idÑbat 5 }

idÑbatÑvideotexÑbodyÑparts					ID ::= { idÑbat 6 }

idÑbatÑencryptedÑbodyÑparts					ID ::= { idÑbat 7 }

idÑbatÑmessageÑbodyÑparts					ID ::= { idÑbat 8 }

idÑbatÑmixedÑmodeÑbodyÑparts				ID ::= { idÑbat 9 }

idÑbatÑbilaterallyÑdefinedÑbodyÑparts		ID ::= { idÑbat 10 }

idÑbatÑnationallyÑdefinedÑbodyÑparts		ID ::= { idÑbat 11 }

idÑbatÑextendedÑbodyÑpartÑtypes				ID ::= { idÑbat 12 }

idÑbatÑia5ÑtextÑparameters						ID ::= { idÑbat 13 }

idÑbatÑvoiceÑparameters						ID ::= { idÑbat 14 }

idÑbatÑg3ÑfacsimileÑparameters				ID ::= { idÑbat 15 }

idÑbatÑteletexÑparameters						ID ::= { idÑbat 16 }

idÑbatÑvideotexÑparameters					ID ::= { idÑbat 17 }

idÑbatÑencryptedÑparameters					ID ::= { idÑbat 18 }

idÑbatÑmessageÑparameters					ID ::= { idÑbat 19 }

idÑbatÑia5ÑtextÑdata							ID ::= { idÑbat 20 }

idÑbatÑvoiceÑdata								ID ::= { idÑbat 21 }

idÑbatÑg3ÑfacsimileÑdata						ID ::= { idÑbat 22 }

idÑbatÑteletexÑdata							ID ::= { idÑbat 23 }

idÑbatÑvideotexÑdata							ID ::= { idÑbat 24 }

idÑbatÑencryptedÑdata							ID ::= { idÑbat 25 }

idÑbatÑmessageÑdata							ID ::= { idÑbat 26 }

ÑÑ Notification attributes

idÑnatÑsubjectÑipm							ID ::= { idÑnat 0 }

idÑnatÑipnÑoriginator							ID ::= { idÑnat 1 }

idÑnatÑipmÑpreferredÑrecipient				ID ::= { idÑnat 2 }

idÑnatÑconversionÑeits							ID ::= { idÑnat 3 }

idÑnatÑnonÑreceiptÑreason						ID ::= { idÑnat 4 }

idÑnatÑdiscardÑreason							ID ::= { idÑnat 5 }

idÑnatÑautoÑforwardÑcomment				ID ::= { idÑnat 6 }

idÑnatÑreturnedÑipm							ID ::= { idÑnat 7 }

idÑnatÑreceiptÑtime							ID ::= { idÑnat 8 }

idÑnatÑacknowledgmentÑmode				ID ::= { idÑnat 9 }

idÑnatÑsupplÑreceiptÑinfo						ID ::= { idÑnat 10 }

ÑÑ Message content types (for use by MS only)

idÑmctÑp2Ñ1984 ID ::= { idÑmct 0 } ÑÑ P2 1984

idÑmctÑp2Ñ1988 ID ::= { idÑmct 1 } ÑÑ P2 1988

ÑÑ Extended body part parameters

idÑepÑia5Ñtext								ID ::= { idÑep 0 }

idÑepÑvoice									ID ::= { idÑep 1 }

idÑepÑg3Ñfacsimile							ID ::= { idÑep 2 }

idÑepÑteletex									ID ::= { idÑep 4 }

idÑepÑvideotex								ID ::= { idÑep 5 }

idÑepÑencrypted								ID ::= { idÑep 6 }

idÑepÑmessage								ID ::= { idÑep 7 }

END ÑÑ of IPMSObjectIdentifiers

ANNEX E

(to Recommendation X.420)

xe ""§Reference definition of abstract information objects

This Annex is an integral part of this Recommendation.

This Annex, a supplement to Section 2, defines for reference purposes the abstract information objects of interpersonal messaging.

IPMSInformationObjects {jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) informationÑobjects(2) }

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

ÑÑ Prologue

ÑÑ Exports everything.

IMPORTS

ÑÑ IPMS upper bounds

ubÑautoÑforwardÑcomment, ubÑfreeÑformÑname, ubÑipmÑidentifierÑsuffix,

ubÑlocalÑimpÑidentifier, ubÑsubjectÑfield, ubÑtelephoneÑnumber

ÑÑÑÑ

FROM IPMSUpperBounds { jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) upperÑbounds(10) }

ÑÑ DTAM

ProtocolElement

ÑÑÑÑ

FROM dTAM

ÑÑÑ MTS abstract service

EncodedInformationTypes, G3FacsimileNonBasicParameters,

MessageDeliveryTime, ORAddress, ORName,

OtherMessageDeliveryFields, SupplementaryInformation,

TeletexNonBasicParameters,

ÑÑÑÑ

FROM MTSAbstractService { jointÑisoÑccitt

mhsÑmotis(6) mts(3) modules(0) mtsÑabstractÑservice(1) };

Time ::= UTCTime

ÑÑ Information object

InformationObject ::= CHOICE {

ipm			[0] IPM,

ipn			[1] IPN }

ÑÑ IPM

IPM ::= SEQUENCE {

heading		Heading,

body		Body }

ÑÑ Heading

Heading ::= SET {

thisÑIPM							ThisIPMField,

originator					[0]		OriginatorField OPTIONAL,

authorizingÑusers				[1]		AuthorizingUsersField OPTIONAL,

primaryÑrecipients			[2]		PrimaryRecipientsField DEFAULT { },

copyÑrecipients				[3]		CopyRecipientsField DEFAULT { },

blindÑcopyÑrecipients		[4]		BlindCopyRecipientsField OPTIONAL,

repliedÑtoÑIPM				[5]		RepliedToIPMField OPTIONAL,

obsoletedÑIPMs				[6]		ObsoletedIPMsField DEFAULT { },

relatedÑIPMs					[7]		RelatedIPMsField DEFAULT { },

subject						[8]		EXPLICIT SubjectField OPTIONAL,

expiryÑtime					[9]		ExpiryTimeField OPTIONAL,

replyÑtime					[10]	ReplyTimeField OPTIONAL,

replyÑrecipients				[11]	ReplyRecipientsField OPTIONAL,

importance					[12]	ImportanceField DEFAULT normal,

sensitivity					[13]	SensitivityField OPTIONAL,

autoÑforwarded				[14]	AutoForwardedField DEFAULT FALSE,

extensions					[15]	ExtensionsField DEFAULT { } }

ÑÑ Heading component types

IPMIdentifier ::= [APPLICATION 11] SET {

user						ORAddress OPTIONAL,

userÑrelativeÑidentifier		LocalIPMIdentifier }

LocalIPMIdentifier ::= PrintableString

(SIZE (0. .ubÑlocalÑipmÑidentifier))

RecipientSpecifier ::= SET {

recipient				[0]	ORDescriptor,

notificationÑrequests	[1]	NotificationRequests DEFAULT { },

replyÑrequested		[2]	BOOLEAN DEFAULT FALSE }

NotificationRequests ::= BIT STRING {

rn(0),

nrn(1),

ipmÑreturn(2) }

ORDescriptor ::= SET {

formalÑname				ORName OPTIONAL,

freeÑformÑname		[0]	FreeFormName OPTIONAL,

telephoneÑnumber	[1]	TelephoneNumber OPTIONAL }

FreeFormName ::= TeletexString (SIZE (0. .ubÑfreeÑformÑname))

TelephoneNumber ::= PrintableString (SIZE (0. .ubÑtelephoneÑnumber))

ÑÑ This IPM heading field

This IPMField ::= IPMIdentifier

ÑÑ Originator heading field

OriginatorField ::= ORDescriptor

ÑÑ Authorizing users heading field

AuthorizingdUsersField ::= SEQUENCE OF AuthorizingUsersSubfield

AuthorizingUsersSubfield ::= ORDescriptor

ÑÑ Primary recipients heading field

PrimaryRecipientsField ::= SEQUENCE OF PrimaryRecipientsSubfield

PrimaryRecipientsSubfield ::= RecipientSpecifier

ÑÑ Copy recipients heading field

CopyRecipientsField ::= SEQUENCE OF CopyRecipientsSubfield

CopyRecipientsSubfield ::= RecipientSpecifier

ÑÑ Blind copy recipients heading field

BlindCopyRecipientsField ::= SEQUENCE OF BlindCopyRecipientsSubfield

BlindCopyRecipientsSubfield ::= RecipientSpecifier

ÑÑ RepliedÑto IPM heading field

RepliedToIPMField ::= IPMIdentifier

ÑÑ Obsoleted IPMs heading field

ObsoletedIPMsField ::= SEQUENCE OF ObsoletedIPMsSubfield

ObsoletedIPMsSubfield ::= IPMIdentifier

ÑÑ Related IPMs heading field

RelatedIPMsField ::= SEQUENCE OF RelatedIPMsSubfield

RelatedIPMsSubfield ::= IPMIdentifier

ÑÑ Subfield heading field

SubjectField ::= TeletexString (SIZE (0. .ubÑsubjectÑfield))

ÑÑ Expiry time heading field

ExpiryÑTimeField ::= Time

ÑÑ Reply time heading field

ReplyTimeField ::= Time

ÑÑ Reply recipient heading field

ReplyRecipientsField ::= SEQUENCE OF ReplyRecipientsSubfield

ReplyRecipientsSubfield ::= ORDescriptor

ÑÑ Importance heading field

ImportanceField ::= ENUMERATED {

low			(0),

normal		(1),

high			(2) }

ÑÑ Sensitivity heading field

SensitivityField ::= ENUMERATED {

personal					(1),

private					(2),

companyÑconfidential		(3) }

ÑÑ AutoÑforwarded heading field

AutoForwardedField ::= BOOLEAN

ÑÑ Extensions heading field

ExtensionsField ::= SET OF HeadingExtension

HeadingExtension ::= SEQUENCE {

type			OBJECT IDENTIFIER,

value		ANY DEFINED BY type DEFAULT NULL NULL }

HEADINGÑEXTENSION MACRO ::=

BEGIN

TYPE NOTATION		::=	ÒVALUEÓ type | empty

VALUE NOTATION	::=	value (VALUE OBJECT IDENTIFIER)

END

ÑÑ Body

Body ::= SEQUENCE OF BodyPart

BodyPart ::= CHOICE {

ia5Ñtext						[0]		IA5TextBodyPart,

voice						[2]		VoiceBodyPart,

g3Ñfacsimile					[3]		G3FacsimileBodyPart,

g4Ñclass1					[4]		G4Class1BodyPart,

teletex						[5]		TeletexBodyPart,

videotex						[6]		VideotexBodyPart,

encrypted					[8]		EncryptedBodyPart,

message						[9]		MessageBodyPart,

mixedÑmode					[11]	MixedModeBodyPart,

bilaterallyÑdefined			[14]	BilaterallyDefinedBodyPart,

nationallyÑdefined			[7]		NationallyDefinedBodyPart,

externallyÑdefined			[15]	ExternallyDefinedBodyPart }

ÑÑ IA5 text body part

IA5TextBodyPart ::= SEQUENCE {

parameters	IA5TextParameters,

data			IA5TextData }

IA5TextParameters ::= SET {

repertoire	[0] Repertoire DEFAULT ia5 }

IA5TextData ::= IA5String

Repertoire ::= ENUMERATED {

ita2(2),

ia5 (5) }

ÑÑ Voice body part

VoiceBodyPart ::= SEQUENCE {

parameters	VoiceParameters,

data			VoiceData }

VoiceParameters ::= SET ÑÑ for further study

VoiceData ::= BIT STRING ÑÑ for further study

ÑÑ G3 Facsimile body part

G3FacsimileBodyPart ::= SEQUENCE {

parameters	G3FacsimileParameters,

data			G3FacsimileData }

G3FacsimileParameters ::= SET {

numberÑofÑpages	[0]	INTEGER OPTIONAL,

nonÑbasicÑparameters	[1]	G3FacsimileNonBasicParameters OPTIONAL }

G3FacsimileData ::= SEQUENCE OF BIT STRING

ÑÑ G4 class 1 and mixedÑmode body parts

G4Class1BodyPart ::= SEQUENCE OF ProtocolElement

MixedModeBodyPart ::= SEQUENCE OF ProtocolElement

ÑÑ Telex body part

TeletexBodyPart ::= SEQUENCE {

parameters	TeletexParameters,

data			TeletexData }

TeletexParameters ::= SET {

numberÑofÑpages	[0]	INTEGER OPTIONAL,

telexÑcompatible	[1]	BOOLEAN DEFAULT FALSE,

nonÑbasicÑparameters	[2]	TeletexNonBasicParameters OPTIONAL }

TeletexData ::= SEQUENCE OF TeletexString

ÑÑ Videotex body part

VideotexBodyPart ::= SEQUENCE {

parameters	VideotexParameters,

data			VideotexData }

VideotexParameters ::= SET {

syntax		[0] VideotexSyntax OPTIONAL

VideotexSyntax ::= INTEGER

ids					(0),

dataÑsyntax1			(1),

dataÑsyntax2			(2),

dataÑsyntax3			(3) }

VideotexData ::= VideotexString

ÑÑ Encrypted body part

EncryptedBodyPart ::= SEQUENCE {

parameters	EncryptedParameters,

data			EncryptedData }

EncryptedParameters ::= SET ÑÑ for further study

EncryptedData ::= BIT STRING ÑÑ for further study

ÑÑ Message body part

MessageBodyPart ::= SEQUENCE {

parameters	MessageParameters,

data			MessageData }

MessageParameters ::= SET {

deliveryÑtime		[0]	MessageDeliveryTime OPTIONAL

deliveryÑenvelope	[1]	OtherMessageDeliveryFields OPTIONAL }

MessageData ::= IPM

ÑÑ Bilaterally defined body part

BilaterallyDefinedBodyPart ::= OCTET STRING

ÑÑ Nationally defined body part

NationallyDefinedBodyPart ::= ANY

ÑÑ Externally defined body part

ExternallyDefinedBodyPart ::= SEQUENCE {

parameters [0]		ExternallyDefinedParameters OPTIONAL,

data				ExternallyDefinedData }

ExternallyDefinedParameters ::= EXTERNAL

ExternallyDefinedData ::= EXTERNAL

EXTENDEDÑBODYÑPARTÑTYPE MACRO ::=

BEGIN

TYPE NOTATION		::=	Parameters Data

VALUE NOTATION	::=	value (VALUE OBJECT IDENTIFIER)

Parameters				::=	ÒPARAMETERSÓ type ÒIDENTIFIEDÓ ÒBYÓ value (OBJECT IDENTIFIER) | empty

Data						::=	ÒDATAÓ type

END

ÑÑ IPN

IPN ::= SET {

ÑÑ commonÑfields ÑÑ COMPONENTS OF CommonFields,

choice [0] CHOICE {

nonÑreceiptÑfields				[0] NonReceiptFields,

receiptÑfields					[1] ReceiptFields } }

RN ::= IPN ÑÑ with receiptÑfields chosen

NRN ::= IPN ÑÑ with nonÑreceiptÑfields chosen

CommonFields ::= SET {

subjectÑipn								SubjectIPNFields,

ipnÑoriginator						[1]		IPNOriginatorField OPTIONAL,

ipnÑpreferredÑrecipient			[2]		IPNPreferredRecipientField OPTIONAL,

conversionÑeits							ConversionEITsField OPTIONAL }

NonReceiptFields ::= SET {

nonÑreceiptÑreason				[0]		NonReceiptReasonField,

discardÑreason					[1]		DiscardReasonField OPTIONAL,

autoÑforwardÑcomment			[2]		AutoForwardCommentField OPTIONAL,

returnedÑipm						[3]		ReturnedIPNField OPTIONAL }

ReceiptFields ::= SET {

receiptÑtime						[0]		RecipientTimeField,

acknowledgmentÑmode			[1]		AcknowledgementModeField DEFAULT manual,

supplÑreceiptÑinfo				[2]		SupplReceiptInfoField DEFAULT Ò Ó }

ÑÑ Common fields

SubjectIPMField ::= IPMIdentifier

IPMOriginatorField ::= ORDescriptor

IPMPreferredRecipientField ::= ORDescriptor

ConversionEITsField ::= EncodedInformationTypes

ÑÑ NonÑreceipt fields

NonReceiptReasonField ::= ENUMERATED {

ipmÑdiscarded					(0),

ipmÑautoÑforwarded				(1) }

DiscardReasonField ::= ENUMERATED {

ipmÑexpired							(0),

ipmÑobsoleted						(1),

userÑsubscriptionÑterminated		(2) }

AutoForwardCommentField ::= AutoForwardComment

AutoForwardComment ::= PrintableString

(SIZE (0. .ubÑautoÑforwardÑcomment))

ReturnedIPMField ::= IPM

ÑÑ Receipt fields

ReceiptTimeField ::= Time

AcknowledgmentModeField ::= ENUMERATED {

manual				(0),

automatic			(1) }

SupplReceiptInfoField/::= SupplementaryInformation

ÑÑ Message store realization

ForwardedInfo ::= SET {

autoÑforwardingÑcomment [0]

AutoForwardComment OPTIONAL,

coverÑnote [1]

IA5TextBodyPart OPTIONAL,

thisÑipmÑprefix [2]

PrintableString (SIZE (1. .ubÑipmÑidentifierÑsuffix))

OPTIONAL }

END ÑÑ of IPMSInformationObjects

ANNEX F

(to Recommendation X.420)

xe ""§Reference definition of functional objects

This Annex is an integral part of this Recommendation.

This Annex, a supplement to ¤¤ 10, 11 and 16, defined for reference purposes the functional objects of interpersonal messaging. It uses the OBJECT and REFINE macros of Recommendation X.407.

IPMFunctionalObjects { jointÑisoÑccitt

mhsÑmotis(6) modules(0) functionalÑobjects(1) }

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

ÑÑ Prologue

ÑÑ Exports everything.

IMPORTS

ÑÑ IPMS abstract service

management, origination, reception

ÑÑÑÑ

FROM IPMSAbstractService { jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) abstractÑservice(3) }

ÑÑ IPMS object identifiers

idÑotÑipme, idÑotÑipms, idÑotÑipmsÑms, idÑotÑipmsÑua,

idÑotÑipmsÑuser, idÑotÑpdau, idÑotÑtima, idÑotÑtlxau,

idÑrefÑprimary, idÑrefÑsecondary

ÑÑÑÑ

FROM IPMSObjectIdentifiers { jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) objectÑidentifiers(0) }

TLMA abstract service

miscellanea

ÑÑÑÑ

FROM TLMAAbsService { ccitt

recommendation(0) t(20) 330 tlmaabsservice(0) }

ÑÑ MS abstract service

retrieval

ÑÑÑÑ

FROM MSAbstractService { jointÑisoÑccitt

mhsÑmotis(6) ms(4) modules(0) abstractÑservice(1) }

ÑÑ MTS abstract service

administration, delivery, mTS, submission

ÑÑÑÑ

FROM MTSAbstractService { jointÑisoÑccitt

mhsÑmotis(6) mts(3) modules(0) mtsÑabstractÑservice(1) }

ÑÑ Abstract service definition conventions

OBJECT, REFINE

ÑÑÑÑ

FROM AbstractServiceNotation { jointÑisoÑccitt

mhsÑmotis(6) asdc(2) modules(0) notation(1) }

ÑÑ ÒRootÓ object type

ipme OBJECT

::= idÑotÑipme

ÑÑ Primary refinement

ipmeÑrefinement REFINE ipme AS

ipms

origination			[S]		PAIRED WITH ipmsÑuser

reception			[S]		PAIRED WITH ipmsÑuser

management		[S]		PAIRED WITH ipmsÑuser

ipmsÑuser RECURRING

::= idÑrefÑprimary

ÑÑ Primary object types

ipmsÑuser OBJECT

PORTS {

origination			[C],

reception			[C],

management		[C] }

::= idÑotÑipmsÑuser

ipms OBJECT

PORTS }

origination			[S],

reception			[S],

management		[S] }

::= idÑotÑipms

ÑÑ Secondary refinement

ipmsÑrefinement REFINE ipms AS

mTS

submission	[S]	PAIRED WITH ipmsÑua, ipmsÑms

delivery	[S]	PAIRED WITH ipmsÑua, ipmsÑms

administration	[S]	PAIRED WITH ipmsÑua, ipmsÑms

ipmsÑua RECURRING

origination	[S]	VISIBLE

reception	[S]	VISIBLE

management	[S]	VISIBLE

ipmsÑms RECURRING

submission	[S]	PAIRED WITH ipmsÑua

retrieval	[S]	PAIRED WITH ipmsÑua

administration	[S]	PAIRED WITH ipmsÑua

tlma RECURRING

origination	[S]	VISIBLE

reception	[S]	VISIBLE

management	[S]	VISIBLE

tlxau RECURRING

origination	[S]	VISIBLE

reception	[S]	VISIBLE

management	[S]	VISIBLE

pdau RECURRING

reception	[S]	VISIBLE

::= idÑrefÑsecondary

ÑÑ Secondary objects

ipmsÑua OBJECT

PORTS {

origination	[S],

reception	[S],

management	[S],

submission	[C],

delivery	[C],

retrieval	[C],

administration	[C] }

::= idÑotÑipmsÑua

ipmsÑms OBJECT

PORTS {

submission	[S],

retrieval	[S],

administration	[S],

submission	[C],

delivery	[C],

administration	[C] }

::= idÑotÑipmsÑms

tlma OBJECT

PORTS {

origination	[S],

reception	[S],

management	[S],

miscellanea	[S] }

::= idÑotÑtlma

tlxau OBJECT

PORTS {

origination	[S],

reception	[S],

management	[S] }

::= idÑotÑtlxau

pdau OBJECT

PORTS {

reception	[S]

::= idÑotÑpdau

END ÑÑ of IPMSFunctionalObjects

ANNEX G

(to Recommendation X.420)

xe ""§Reference definition of abstract service

This Annex is an integral part of this Recommendation.

This Annex, a supplement to ¤¤ 12 and 13, defines for reference purposes the IPMS abstract service. It uses the PORT and ABSTRACTÑOPERATION and ÑERROR macros of Recommendation X.407.

IPMSAbstractService { jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) abstractÑservice(3) }

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

ÑÑ Prologue

ÑÑ Exports everything.

IMPORTS

ÑÑ IPMS information objects

AutoForwardComment, Heading, IPM, NRN, RN

ÑÑÑÑ

FROM IPMSInformationObjects { jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) informationÑobjects(2) }

ÑÑ IPMS object identifiers

idÑptÑmanagement, idÑptÑorigination, idÑptÑreception

ÑÑÑÑ

FROM IPMSObjectsIdentifiers { jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) objectsÑidentifiers(0) }

ÑÑ MTS abstract service

MessageDeliveryEnvelope, MessageSubmissionEnvelope,

MessageSubmissionIdentifier, MessageSubmissionTime,

ProbeSubmissionEnvelope, ProbeSubmissionIdentifier,

ProbeSubmissionTime, RecipientImproperlySpecified,

ReportDeliveryEnvelope,

ÑÑÑÑ

FROM MTSAbstractService { jointÑisoÑccitt

mhsÑmotis(6) mts(3) modules(0) mtsÑabstractÑservice(1) }

ÑÑ Abstract service definition conventions

ABSTRACTÑERROR, ABSTRACTÑOPERATION, PORT

ÑÑÑÑ

FROM AbstractServiceNotation { jointÑisoÑccitt

mhsÑmotis(6) asdc(2) modules(0) notation(1) };

Time ::= UTCTime

ÑÑ Ports

origination PORT

CONSUMER INVOKES {

OriginateProbe,

OriginateIPM,

OriginateRN }

::= idÑptÑorigination

reception PORT

SUPPLIER INVOKES {

ReceiveReport,

ReceiveIPM,

ReceiveRN,

ReceiveNRN }

::= idÑptÑreception

management PORT

CONSUMER INVOKES {

ChangeAutoDiscard,

ChangeAutoAcknowledgment,

ChangeAutoForwarding }

::= idÑptÑmanagement

ÑÑ Origination abstract operations

OriginateProbe ::= ABSTRACTÑOPERATION

ARGUMENT SET {

envelope	[0] ProbeSubmissionEnvelope,

content		[1] IPM }

RESULT SET {

submissionÑidentifier	[0] ProbeSubmissionIdentifier,

submissionÑtime		[1] ProbeSubmissionTime }

ERRORS {

SubscriptionError,

RecipientImproperlySpecified }

OriginateIPM ::= ABSTRACTÑOPERATION

ARGUMENT SET {

envelope	[0] MessageSubmissionEnvelope,

content		[1] IPM }

RESULT SET {

submissionÑidentifier	[0] MessageSubmissionIdentifier,

submissionÑtime		[1] MessageSubmissionTime }

ERROR {

SubscriptionError,

RecipientImproperlySpecified }

OriginateRN ::= ABSTRACTÑOPERATION

ARGUMENT SET {

envelope	[0] MessageSubmissionEnvelope,

content		[1] RN }

RESULT SET {

submissionÑidentifier	[0] MessageSubmissionIdentifier,

submissionÑtime		[1] MessageSubmissionTime }

ERROR {

SubscriptionError,

RecipientImproperlySpecified }

ÑÑ Reception abstract operations

ReceiptReport ::= ABSTRACTÑOPERATION

ARGUMENT SET {

envelope		[0] ReportDeliveryEnvelope,

undeliveredÑobject	[1] InformationObject OPTIONAL }

RESULT

ERRORS { }

ReceiveIPM ::= ABSTRACTÑOPERATION

ARGUMENT SET {

envelope	[0] MessageDeliveryEnvelope,

content		[1] IPM }

RESULT

ERRORS { }

ReceiveRN ::= ABSTRACTÑOPERATION

ARGUMENT SET {

envelope	[0] MessageDeliveryEnvelope,

content		[1] RN }

RESULT

ERRORS { }

ReceiveNRN ::= ABSTRACTÑOPERATION

ARGUMENT SET {

envelope	[0] MessageDeliveryEnvelope,

content		[1] NRN }

RESULT

ERRORS { }

ÑÑ Management abstract operations

ChangeAutoDiscard ::= ABSTRACTÑOPERATION

ARGUMENT SET {

autoÑdiscardÑexpiredÑIPMs		[0] BOOLEAN,

autoÑdiscardÑobsoleteÑIPMs		[1] BOOLEAN }

RESULT

ERRORS { }

ChangeAutoAcknowledgement ::= ABSTRACTÑOPERATION

ARGUMENT SET {

autoÑacknowledgeÑIPMs [0] BOOLEAN,

autoÑacknowledgeÑsupplÑreceiptÑinfo [1]

SupplementaryInformation }

RESULT

ERRORS {

SubscriptionError }

ChangeAutoForwarding ::= ABSTRACTÑOPERATION

ARGUMENT SET {

autoÑforwardÑIPMs		[0] BOOLEAN,

autoÑforwardÑrecipients		[1] SEQUENCE OF ORName OPTIONAL,

autoÑforwardÑheading		[2] Heading OPTIONAL,

autoÑforwardÑcomment		[3] AutoForwardComment OPTIONAL }

RESULT

ERRORS {

SubscriptionError,

RecipientImproperlySpecified }

ÑÑ Abstract errors

SubscriptionError ::= ABSTRACTÑERROR

PARAMETER SET {

problem		[0] SubscriptionProblem }

SubscriptionProblem ::= ENUMERATED {

ipmsÑeosÑnotÑsubscribed	(0),

mtsÑeosÑnotÑsubscribed		(1) }

END ÑÑ of IPMSAbstractService

ANNEX H

(to Recommendation X.420)

xe ""§Reference definition of heading extensions

This Annex is an integral part of this Recommendation.

This Annex, a supplement to Annex A, defines for reference purposes the heading extensions defined for interpersonal messaging. It uses the HEADINGÑEXTENSION macro of ¤ 12.2.17.

IPMSHeadingExtensions { jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) headingÑextensions(6) }

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

ÑÑ Prologue

ÑÑ Exports everything.

IMPORTS

ÑÑ IPMS information objects

HEADINGÑEXTENSION

ÑÑÑÑ

FROM IPMSInformationObjects { jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) informationÑobjects(2) };

ÑÑ IPMS object identifiers

idÑhexÑincompleteÑcopy, idÑhexÑlanguages

ÑÑÑÑ

FROM IPMSObjectsIdentifiers { jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) objectsÑidentifiers(0) };

ÑÑ Incomplete copy

Incomplete copy HEADINGÑEXTENSION

::= idÑhexÑincompleteÑcopy

IncompleteCopy::= NULL

ÑÑ Languages

languages HEADINGÑEXTENSION

VALUE SET OF Language

::= idÑhexÑlanguages

Language ::= PrintableString (SIZE (2. .2))

END ÑÑ of IPMSHeadingExtensions

ANNEX I

(to Recommendation X.420)

xe ""§Reference definition of extended body part types

This Annex is an integral part of this Recommendation.

This Annex, a supplement to Annex B, defines for reference purposes certain extended body part types.

IPMSExtendedBodyPartTypes {jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) extendedÑbodyÑpartÑtypes(7)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

ÑÑ Prologue

ÑÑ Exports everything.

IMPORTS

ÑÑ IPMS information objects

BilaterallyDefinedBodyPart, EncryptedData,

EncryptedParameters, EXTENDEDÑBODYÑPARTÑTYPE, G3FacsimileData,

G3FacsimileParameters, G4Class1BodyPart, IA5TextData,

IA5TextParameters, MessageData, MessageParameters,

MixedModeBodyPart, NationallyDefinedBodyPart,TeletexData,

TeletexParameters, VideotexData, VideotexParameters,

VoiceData, VoiceParameters

ÑÑÑÑ

FROM IPMSInformationObjects {jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) informationÑobjects(2)}

ÑÑ IPMS object identifiers

idÑepÑencrypted,

idÑepÑg3Ñfacsimile,

idÑepÑia5Ñtext,

idÑepÑmessage,

idÑepÑteletex,

idÑepÑvideotex,

idÑepÑvoice,

idÑetÑbilaterallyÑdefined, idÑetÑencrypted, idÑetÑg3Ñfacsimile,

idÑetÑg4Ñclass1, idÑetÑia5Ñtext, idÑetÑmessage,

idÑetÑmixedÑmode, idÑetÑnationallyÑdefined, idÑetÑteletex,

idÑetÑvideotex, idÑetÑvoice

ÑÑÑÑ

FROM IPMSObjectsIdentifiers {jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) objectsÑidentifiers(0)}

ÑÑ Extended IA5 text body part

ia5ÑtextÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

PARAMETERS	IA5TextParameters IDENTIFIED BY idÑepÑia5Ñtext

DATA	IA5TextData

::= idÑetÑia5Ñtext

ÑÑ Extended voice body part

voiceÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

PARAMETERS	VoiceParameters IDENTIFIED BY idÑepÑvoice

DATA	VoiceData

::= idÑetÑvoice

ÑÑ Extended G3 facsimile body part

g3ÑfacsimileÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

PARAMETERS	G3FacsimileParameters IDENTIFIED BY idÑepÑg3Ñfacsimile

DATA	G3FacsimileData

::= idÑetÑg3Ñfacsimile

ÑÑ Extended G4 class 1 body part

g4Ñclass1ÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

DATA	G4Class1BodyPart

::= idÑetÑg4Ñclass1

ÑÑ Extended teletex body part

teletexÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

PARAMETERS	TeletexParameters IDENTIFIED BY idÑepÑteletex

DATA	TeletexData

::= idÑetÑteletex

ÑÑ Extended videotex body part

videotexÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

PARAMETERS	VideotexParameters IDENTIFIED BY idÑepÑvideotex

DATA	VideotexData

::= idÑetÑvideotex

ÑÑ Extended encrypted body part

encryptedÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

PARAMETERS	EncryptedParameters IDENTIFIED BY idÑepÑencrypted

DATA	EncryptedData

::= idÑetÑencrypted

ÑÑ Extended message body part

messageÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

PARAMETERS	MessageParameters IDENTIFIED BY idÑepÑmessage

DATA	MessageData

::= idÑetÑmessage

ÑÑ Extended mixedÑmode body part

mixedÑmodeÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

DATA MixedModeBodyPart

::= idÑetÑmixedÑmode

ÑÑ Extended bilaterally defined body part

bilaterallyÑdefinedÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

DATA BilaterallyDefinedBodyPart

::Ñ idÑetÑbilaterallyÑdefined

ÑÑ Extended nationally defined body part

nationallyÑdefinedÑbodyÑpart EXTENDEDÑBODYÑPARTÑTYPE

DATA NationallyDefinedBodyPart

::Ñ idÑetÑnationallyÑdefined

END ÑÑ of IPMSExtendedBodyPartTypes

ANNEX J

(to Recommendation X.420)

xe ""§Reference definition of message store attributes

This Annex is an integral part of this Recommendation.

This Annex, a supplement to Annex C, defines for reference purposes the MS attributes specific to interpersonal messaging. It uses the ATTRIBUTE macro of Recommendation X.500.

IPMSMessageStoreAttributes { jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) messageÑstoreÑattributes(8) }

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

ÑÑ Prologue

ÑÑ Exports everything.

IMPORTS

ÑÑ IPMS heading extensions

IncompleteCopy, Language

ÑÑÑÑ

FROM IPMSHeadingExtensions { jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) headingÑextensions(6) }

ÑÑ IPMS information objects

AcknowledgmentModeField, AuthorizingUsersSubfield,

AutoForwardCommentField, AutoForwardedField,

BilaterallyDefinedBodyPart, BlindCopyRecipientsSubfield, Body,

ConversionEITsField, CopyRecipientsSubfield,

DiscardReasonField, EncryptedBodyPart, EncryptedData,

EncryptedParameters, ExpiryTimeField,

ExternallyDefinedParameters, G3FacsimileBodyPart,

G3FacsimileData, G3FacsimileParameters, G4Class1BodyPart,

Heading, IA5TextBodyPart, IA5TextData, IA5TextParameters,

ImportanceField, IPMPreferredRecipientField,

IPNOriginatorField, MessageBodyPart, MessageData,

MessageParameters, MixedModeBodyPart,

NationallyDefinedBodyPart, NonReceiptReasonField,

ObsoletedIPMsSubfield, ORDescriptor, OriginatorField,

PrimaryRecipientsSubfield, ReceiptTimeField,

RelatedIPMsSubfield, RepliedToIPMField,

ReplyRecipientsSubfield, ReplyTimeField, ReturnedIPMField,

SensitivityField, SubjectField, SubjectIPMField,

SupplReceiptInfoField, TeletexBodyPart, TeletexData,

TeletexParameters, ThisIPMField, VideotexBodyPart,

VideotexData, VideotexParameters, VoiceBodyPart, VoiceData,

VoiceParameters

ÑÑÑÑ

FROM IPMSInformationObjects { jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) informationÑobjects(2) }

ÑÑ IPMS object identifiers

idÑbatÑbilaterallyÑdefinedÑbodyÑparts, idÑbatÑbody,

idÑbatÑencryptedÑbodyÑparts, idÑbatÑencryptedÑdata,

idÑbatÑencryptedÑparameters, idÑbatÑextendedÑbodyÑpartÑtypes,

idÑbatÑg3ÑfacsimileÑbodyÑparts, idÑbatÑg3ÑfacsimileÑdata,

idÑbatÑg3ÑfacsimileÑparameters, idÑbatÑg4Ñclass1ÑbodyÑparts,

idÑbatÑia5ÑtextÑbodyÑparts, idÑbatÑia5ÑtextÑdata,

idÑbatÑia5ÑtextÑparameters, idÑbatÑmessageÑbodyÑparts,

idÑbatÑmessageÑdata, idÑbatÑmessageÑparameters,

idÑbatÑmixedÑmodeÑbodyÑparts,

idÑbatÑnationallyÑdefinedÑbodyÑparts,

idÑbatÑteletexÑbodyÑparts, idÑbatÑteletexÑdata,

idÑbatÑteletexÑparameters, idÑbatÑvideotexÑbodyÑparts,

idÑbatÑvideotexÑdata, idÑbatÑvideotexÑparameters,

idÑbatÑvoiceÑbodyÑparts, idÑbatÑvoiceÑdata,

idÑbatÑvoiceÑparameters, idÑhatÑauthorizingÑusers,

idÑhatÑautoÑforwarded, idÑhatÑblindÑcopyÑrecipients,

idÑhatÑcopyÑrecipients, idÑhatÑexpiryÑtime, idÑhatÑheading,

idÑhatÑimportance, idÑhatÑincompleteÑcopy, idÑhatÑlanguages,

idÑhatÑnrnÑrequestors, idÑhatÑobsoletedÑIPMs,

idÑhatÑoriginator, idÑhatÑprimaryÑrecipients,

idÑhatÑrelatedÑIPMs, idÑhatÑrepliedÑtoÑIPM,

idÑhatÑreplyÑrecipients, idÑhatÑreplyÑrequestors,

idÑhatÑreplyÑtime idÑhatÑrnÑrequestors, idÑhatÑsensitivity,

idÑhatÑsubject, idÑhatÑthisÑipm, idÑnatÑacknowledgmentÑmode,

idÑnatÑautoÑforwardÑcomment, idÑnatÑconversionÑeits,

idÑnatÑdiscardÑreason, idÑnatÑipmÑpreferredÑrecipient,

idÑnatÑipnÑoriginator, idÑnatÑnonÑreceiptÑreason,

idÑnatÑreceiptÑtime, idÑnatÑreturnedÑipm, idÑnatÑsubjectÑipm,

idÑnatÑsupplÑreceiptÑinfo, idÑsatÑipmÑentryÑtype,

idÑsatÑipmÑsynopsis

ÑÑÑÑ

FROM IPMSObjectIdentifiers { jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) objectÑidentifiers(0) }

ÑÑ MS abstract service

MSÑEITs, SequenceNumber

ÑÑÑÑ

FROM MSAbstractService { jointÑisoÑccitt

mhsÑmotis(6) ms(4) modules(0) abstractÑservice(1) }

ÑÑ MTS abstract service

EncodedInformationTypes

ÑÑÑÑ

FROM MTSAbstractService { jointÑisoÑccitt

mhsÑmotis(6) mts(3) modules(0) mtsÑabstractÑservice(1) };

ÑÑ Directory information framework

ATTRIBUTE

ÑÑÑÑ

FROM InformationFramework { jointÑisoÑccitt

ds(5) modules(1) informationFramework(1) };

Time ::= UTCTime

ÑÑ SUMMARY ATTRIBUTES

ÑÑ IPM entry type

ipmÑentryÑtype ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IPMEntryType

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑsatÑipmÑentryÑtype

IPMEntryType ::= ENUMERATED {

ipm			(0),

rn			(1),

nrn			(2) }

ÑÑ IPM synopsis

ipmÑsynopsis ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IPMSynopsis

SINGLE VALUE

::= idÑsatÑipmÑsynopsis

IPMSynopsis ::= SEQUENCE OF BodyPartSynopsis

BodyPartSynopsis ::= CHOICE {

messsage			[0] MessageBodyPartSynopsis,

nonÑmesssage		[1] NonMessageBodyPartSynopsis }

MessageBodyPartSynopsis ::= SEQUENCE {

number			[0] SequenceNumber,

synopsis			[1] IPMSynopsis }

NonMessageBodyPartSynopsis ::= SEQUENCE {

type				[0] OBJECT IDENTIFIER,

parameters		[1] ExternallyDefinedParameters,

size				[2] INTEGER,

processed		[3] BOOLEAN DEFAULT FALSE }

ÑÑ HEADING ATTRIBUTES

ÑÑ Heading

heading ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX Heading

SINGLE VALUE

::= idÑhatÑheading

ÑÑ Heading analyses

rnÑrequestors ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ORDescriptor

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑrnÑrequestors

nrnÑrequestors ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ORDescriptor

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑnrnÑrequestors

replyÑrequestors ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ORDescriptor

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑreplyÑrequestors

ÑÑ Heading fields

thisÑipm ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ThisIPMField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑhatÑthisÑipm

originator ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX OriginatorField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑhatÑoriginator

repliedÑtoÑIPM ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX RepliedToIPMField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑhatÑrepliedÑtoÑIPM

subject ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX SubjectField

MATCHES FOR EQUALITY SUBSTRINGS

SINGLE VALUE

::= idÑhatÑsubject

expiryÑtime ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ExpiryTimeField

MATCHES FOR EQUALITY ORDERING

SINGLE VALUE

::= idÑhatÑexpiryÑtime

replyÑtime ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ReplyTimeField

MATCHES FOR EQUALITY ORDERING

SINGLE VALUE

::= idÑhatÑreplyÑtime

importance ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ImportanceField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑhatÑimportance

sensitivity ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX SensitivityField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑhatÑsensitivity

autoÑforwarded ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX AutoForwardedField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑhatÑautoÑforward

ÑÑ Heading subÑfields

authorizingÑusers ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX AuthorizingÑUsersSubfield

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑauthorizingÑusers

primaryÑrecipients ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX PrimaryRecipientsSubfield

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑprimaryÑrecipients

copyÑrecipients ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX CopyRecipientsSubfield

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑcopyÑrecipients

blindÑcopyÑrecipients ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX BlindCopyRecipientsSubfield

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑblindÑcopyÑrecipients

obsoletedÑIPMs ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ObsoletedIPMsSubfield

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑobsoletedÑIPMs

relatedÑIPMs ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX RelatedIPMsSubfield

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑrelatedÑIPMs

replyÑrecipients ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ReplyRecipientsSubfield

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑreplyÑrecipients

ÑÑ Heading extensions

incompleteÑcopy ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IncompleteCopy

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑhatÑincompleteÑcopy

languages ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX Language

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑhatÑlanguages

ÑÑ BODY ATTRIBUTES

ÑÑ Body

body ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX Body

SINGLE VALUE

::= idÑbatÑbody

ÑÑ Basic body parts

ia5ÑtextÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IA5TextBodyParts

MULTI VALUE

::= idÑbatÑia5ÑtextÑbodyÑparts

voiceÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX VoiceBodyPart

MULTI VALUE

::= idÑbatÑvoiceÑbodyÑparts

g3ÑfacsimileÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX G3FacsimileBodyPart

MULTI VALUE

::= idÑbatÑg3ÑfacsimileÑbodyÑparts

g4Ñclass1ÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX G4Class1BodyPart

MULTI VALUE

::= idÑbatÑg4Ñclass1ÑbodyÑparts

teletexÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX TeletexBodyPart

MULTI VALUE

::= idÑbatÑteletexÑbodyÑparts

videotexÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX VideotexBodyPart

MULTI VALUE

::= idÑbatÑvideotexÑbodyÑparts

encryptedÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX EncryptedBodyPart

MULTI VALUE

::= idÑbatÑencryptedÑbodyÑparts

messageÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX SequenceNumber

MULTI VALUE

::= idÑbatÑmessageÑbodyÑparts

mixedÑmodeÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX MixedModeBodyPart

MULTI VALUE

::= idÑbatÑmixedÑmodeÑbodyÑparts

bilaterallyÑdefinedÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX BilaterallyDefinedBodyPart

MULTI VALUE

::= idÑbatÑbilaterallyÑdefinedÑbodyÑparts

nationallyÑdefinedÑbodyÑparts ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX NationallyDefinedBodyPart

MULTI VALUE

::= idÑbatÑnationallyÑdefinedÑbodyÑparts

ÑÑ Basic body part parameters component

ia5ÑtextÑparameters ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IA5TextParameters

MULTI VALUE

::= idÑbatÑia5ÑtextÑparameters

voiceÑparameters ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX VoiceParameters

MULTI VALUE

::= idÑbatÑvoiceÑparameters

g3ÑfacsimileÑparameters ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX G3FacsimileParameters

MULTI VALUE

::= idÑbatÑg3ÑfacsimileÑparameters

teletexÑparameters ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX TeletexParameters

MULTI VALUE

::= idÑbatÑteletexÑparameters

videotexÑparameters ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX VideotexParameters

MULTI VALUE

::= idÑbatÑvideotexÑparameters

encryptedÑparameters ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX EncryptedParameters

MULTI VALUE

::= idÑbatÑencryptedÑparameters

messageÑparameters ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX MessageParameters

MULTI VALUE

::= idÑbatÑmessageÑparameters

ÑÑ Basic body part data components

ia5ÑtextÑdata ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IA5TextData

MULTI VALUE

::= idÑbatÑia5ÑtextÑdata

voiceÑdata ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX VoiceÑData

MULTI VALUE

::= idÑbatÑvoiceÑdata

g3ÑfacsimileÑdata ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX G3FacsimileData

MULTI VALUE

::= idÑbatÑg3ÑfacsimileÑdata

teletexÑdata ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX TeletexData

MULTI VALUE

::= idÑbatÑteletexÑdata

videotexÑdata ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX VideotexData

MULTI VALUE

::= idÑbatÑvideotexÑdata

encryptedÑdata ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX EncryptedData

MULTI VALUE

::= idÑbatÑencryptedÑdata

messageÑdata ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX MessageData

MULTI VALUE

::= idÑbatÑmessageÑdata

ÑÑ Extended body part types

extendedÑbodyÑpartÑtypes ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX OBJECT IDENTIFIER

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑbatÑextendedÑbodyÑpartÑtypes

ÑÑ Extended body parts

ÑÑ (These attributes cannot be enumerated. See ¤ C.3.6)

ÑÑ NOTIFICATION ATTRIBUTES

ÑÑ Common fields

subjectÑipm ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX SubjectIPMField

MATCHES FOR EQUALITY SUBSTRINGS

SINGLE VALUE

::= idÑnatÑsubjectÑipm

ipnÑoriginator ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IPNOriginatorField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑnatÑipnÑoriginator

ipmÑpreferredÑrecipient ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX IPMPreferredRecipientField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑnatÑipmÑpreferredÑrecipient

conversionÑeits ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX MSÑEITs

MATCHES FOR EQUALITY

MULTI VALUE

::= idÑnatÑconversionÑeits

ÑÑ NonÑreceipt fields

nonÑreceiptÑreason ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX NonReceiptReasonField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑnatÑnonÑreceiptÑreason

discardÑreason ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX DiscardReasonField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑnatÑdiscardÑreason

autoÑforwardÑcomment ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX AutoForwardCommentField

MATCHES FOR EQUALITY SUBSTRINGS

SINGLE VALUE

::= idÑnatÑautoÑforwardÑcomment

returnedÑipm ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ReturnedIPMField

SINGLE VALUE

::= idÑnatÑreturnedÑIPM

ÑÑ Receipt fields

receiptÑtime ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX ReceiptTimeField

MATCHES FOR EQUALITY ORDERING

SINGLE VALUE

::= idÑnatÑreceiptÑtime

acknowledgmentÑmode ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX AcknowledgmentModeField

MATCHES FOR EQUALITY

SINGLE VALUE

::= idÑnatÑacknowledgmentÑmode

supplÑreceiptÑinfo ATTRIBUTE

WITH ATTRIBUTEÑSYNTAX SupplReceiptInfoField

MATCHES FOR EQUALITY SUBSTRINGS

SINGLE VALUE

::= idÑnatÑsupplÑreceiptÑinfo

END ÑÑ of IPMSMessagesStoreAttributes

ANNEX K

(to Recommendation X.420)

xe ""§Reference definition of upper bounds

This Annex is an integral part of this Recommendation but is not a part of the corresponding ISO International Standard.

This Annex defines for reference purposes the upper bounds of various variableÑlength information items whose abstract syntaxes are defined in the ASN.1 modules of prior annexes.

IPMSUpperBounds { jointÑisoÑccitt

mhsÑmotis(6) ipms(1) modules(0) upperÑbounds(10) }

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

ÑÑ Prologue

ÑÑ Exports everything.

IMPORTS ÑÑ nothing ÑÑ;

ÑÑ Upper bounds

ubÑautoÑforwardÑcomment			INTEGER ::= 256

ubÑfreeÑformÑname				INTEGER ::= 64

ubÑipmÑidentifierÑsuffix			INTEGER ::= 2

ubÑlocalÑipmÑidentifier				INTEGER ::= 64

ubÑsubjectÑfield					INTEGER ::= 128

ubÑtelephoneÑnumber				INTEGER ::= 32

END ÑÑ of IPMSUpperBounds

ANNEX L

(to Recommendation X.420)

xe ""§Support of the interpersonal messaging service

This Annex is an integral part of this Recommendation.

The interpersonal messaging service which the IPMS provides to users is defined, in nonÑtechnical terms, in Recommendation X.400. The service comprises a number of elements of service (IPM EOS), each representing one aspect of the service, and each defined in one or two paragraphs of prose. The present Annex indicates in detail how the present, more technical specification realizes each IPM EOS. Equivalently, it identifies the aspects of the specification a UA, e.g., must implement for it to be said to support a particular IPM EOS.

Associated with each IPM EOS are one or more information items that may appear as IPM components. The information item associated with the sensitivity indication IPM EOS, e.g., is the sensitivity heading field. A UA, TLMA, or AU shall be said to support a particular IPM EOS upon origination or reception if, and only if, it supports upon origination or reception (see ¤ 22.1) the information items associated with that IPM EOS.

Note 1 Ñ The task of realizing an IPM EOS may fall, in principle, upon any of the secondary objects that result from the refinement of the IPMS. In the present context, however, it is assumed that the MTS and every MS, by virtue of their applicationÑindependence, support every IPMÑEOS, and that they do so without having made special provision for any of them.

Note 2 Ñ As described in ¤ 14, a UA makes available to its user many of the capabilities that its MS offers. These capabilities realize the elements of the message retrieval service which is defined in Recommendation X.400. The correspondence between the elements of that service and the associated technical capabilities is given in Recommendation X.413.

Note 3 Ñ As described in ¤ 14, a UA makes available to its user many of the capabilities that the MTS offers. These capabilities realize the elements of the message transfer service which is defined in Recommendation X.400. The correspondence between the elements of that service and the associated technical capabilities is given in Recommendation X.411.

## L.1	Support of recipient specifier components

Some IPM EOS are realized by means of recipient specifier components. The IPM EOS in this category are listed in the first column of Table LÑ1/X.420. The second and third columns identify the recipient specifier component, and the particular value of that component, that are the information items associated with each listed IPM EOS.

TABLE LÑ1/X.420

Support of recipient specifier components

| Element of service | Recipient specifier component | Value |
| --- | --- | --- |
| NonÑreceipt notification request | NotificationÑrequests | nrn |
| Receipt notification request indication | NotificationÑrequests | rn |
| Reply request indication (see also Table LÑ2/X.420) | ReplyÑrequested | true |

Note 1 Ñ Recipient specifiers appear as subÑfields of the Primary Recipients, Copy Recipients, and Blind Copy Recipients heading fields.

Note 2 Ñ Every IPM EOS except Reply Request Indication falls into exactly one category. The Reply Request Indication IPM EOS falls into two categories, as indicated in the Table.

## L.2	Support of heading fields

Some IPM EOS are realized by means of heading fields. The IPM EOS in this category are listed in the first column of Table LÑ2/X.420. The second column identifies the heading fields that are the information items associated with each listed IPM EOS. In the case of the extension field, the second column also identifies, in parentheses, the relevant heading extension.

## L.3	Support of body aspects

Some IPM EOS are realized by means of aspects of the body. The IPM EOS in this category are listed in the first column of Table LÑ3/X.420. The second column identifies the body aspect that is the information item associated with each listed IPM EOS.

TABLE LÑ2/X.420

Support of heading fields

| Element of service | Heading field |
| --- | --- |
| Authorizing users indication | Authorizing users |
| AutoÑforwarded indication | AutoÑforwarded |
| Blind copy recipient indication | Blind copy recipients |
| CrossÑreferencing indication | Related IPMs |
| Expiry date indication | Expiry time |
| Importance indication | Importance |
| IPÑmessage identification | This IPM |
| Incomplete copy indication | Extensions (incomplete copy) |
| Language indication | Extensions (languages) |
| Obsoleting indication | Obsoleted IPMs |
| Originator indication | Originator |
| Primary and | Primary recipients |
| copy recipients indication | Copy recipients |
| Reply request indication (see | Reply time |
| also Table LÑ3/X.420) | Reply recipients |
| Replying IPÑmessage indication | RepliedÑto IPM |
| Sensitivity indication | Sensitivity |
| Subject indication | Subject |

Note Ñ Every IPM EOS except Reply Request Indication falls into exactly one category. The Reply Request Indication IMP EOS falls into two categories, as indicated in the Table.

TABLE LÑ3/X.420

Support of body aspects

| Element of service | Body aspect |
| --- | --- |
| Body part encryption indication | Encrypted body part |
| Forwarded IPÑmessage indication | Message body part |
| MultiÑpart body | Body with two or more parts |
| Typed body | Body (itself) |

Note Ñ Support of the Typed Body IPM EOS is intrinsic to any implementation of any secondary object.

ANNEX M

xe ""§Differences between CCITT Recommendation and ISO Standard

This Annex is not a part of this Recommendation.

This Annex lists all but the purely stylistic differences between this Recommendation and the corresponding ISO International Standard.

The following are the differences that exist:

a)	The ISO International Standard corresponding to this Recommendation defines a general text body part, while this Recommendation does not.

b)	The upper bounds of Annex K are an integral part of this Recommendation but are not a part of the corresponding ISO International Standard.

c)	The CCITT text on subject recipient specifier in ¤ 8 states that it may contain either Òan O/R name of the preferred recipientÓ or Òan O/R name appearing in the message's DL expansion historyÓ. The ISO Standard has excluded this second possibility.

ANNEX N

xe ""§Summary of changes to 1984 specification

This Annex is not a part of this Recommendation.

Editorially, this Recommendation differs substantially from Recommendation X.420 (1984). Technically, however, the differences are few. The present Annex lists the technical changes. It is intended as an aid to an implementor of Recommendation X.420 (1984), enabling him to see at a glance how his implementation might be affected by the 1988 specification.

The following, and only the following, substantive changes relevant to interworking between 1984 and 1988 UAs, MSs, TLMAs, and AUs are embodied in the present specification. All but the first are changes to the format of the information objects now defined in the ASN.1 module, IPMSInformationObjects:

a)	The content type assigned to P2 has changed. Formerly identified by the integer 2, P2 now is identified by either the integer 2 or 22, depending upon the functionality employed in a particular instance of communication by means of the MTS (see ¤ 20.2.).

b)	The omission of the user members of IPMIdentifier is now denigrated.

c)	The extensions member has been added to heading. Its grade is optional.

d)	The telex and simple formattable document body part types have been abandoned. (The former had been identified but not defined.)

e)	The syntax member has been added to VideotexParameters. Its grade is optional.

f)	The presence of the deliveryÑtime member of the MessageParameters in the absence of its deliveryÑenvelope member, or viceÑversa, is now denigrated.

g)	The bilaterallyÑdefined and externallyÑdefined alternatives have been added to BodyPart.

h)	The following protocol elements, defined in Recommendation X.411 and incorporated in protocol elements of this Recommendation by reference, have changed:

i)	ORName

ii)	ORAddress

iii)	MessageDeliveryEnvelope

iv)	EncodedInformationTypes

v)	SupplementaryInformation.

i)	Specifying a value of zero length of any of the following data types is now denigrated:

i)	LocalIPMIdentifier

ii)	FreeFormName

iii)	TelephoneNumber

iv)	SubjectField

v)	AutoForwardComment.

j)	Upper bounds have been imposed upon certain variableÑlength protocol elements.

Note Ñ The upper bounds imposed are those found in ¤ 4.3 of Version 6 of the X.400Ñseries Implementor's Guide.

1	) 	Recommendation X.420 and ISO 10021Ñ7 [Information Processing Systems Ñ Text Communication Ñ MOTIS Ñ Interpersonal Messaging System] were developed in close collaboration and are technically aligned, except for the differences.